| IMAGE SENSOR |  |
| --- | --- |
| Type | APS-C CMOS |
| Effective Pixels | Approx. 32.5 megapixels |
| Total Pixels | Approx. 34.4 megapixels |
| Aspect Ratio | 3:2 |
| Sensor Cleaning | integrated cleaning system |
| Colour Filter Type | Primary Colour |
| IMAGE PROCESSOR | DIGIC 8 sau echivalent |
| Type |  |
| LENS |  |
| Lens Mount | EF/EF-S |
| Focal Length | Equivalent to 1.6x the focal length of the lens |
| FOCUSING |  |
| Live View Shooting | Dual Pixel CMOS AF |
| Viewfinder Shooting | TTL secondary image-registration, phase-difference detection with the dedicated AF sensor |
| AF System/ Points | 45 cross-type AF points |
| AF working range | EV -3 - 18 (at 23°C & ISO100) |
| AF Modes | Spot AF 1-Point AF Zone AF Large Zone AF Face Priority AF AI Focus One Shot AI Servo Movie Servo AF Continuous AF Eye Detection AF |
| AF area (live shooting) | Horizontal: approx. 88%, Vertical: approx. 100% |
| AF Lock | Locked when shutter button is pressed half way |
| AF Assist Beam | Intermittent firing of built-in flash or emitted by optional dedicated Speedlite |
| AF Microadjustment | Adjust all lenses by same amount Adjust by lens |
| Manual Focus | Selected on lens |
| EXPOSURE CONTROL | Viewfinder shooting: Approx 220,000 pixels RGB+IR metering sensor 216-zone (18x12) TTL full-aperture metering Live View shooting and Move recording: 384-zone metering with image sensor |
| Metering modes |  |
| Metering Range | EV 1-20 (optical viewfinder) EV -2 – 20 (Live View) EV 0 – 20 (Movie recording) |
| AE Lock | Auto: In One-shot AF mode with evaluative metering exposure is locked when focus is achieved. Manual: By AE lock button in P, TV, AV and M mode. |
| Exposure Compensation | +/-5 EV in 1/3 or 1/2 stop increments (viewfinder) +/-3 EV in 1/3 or 1/2 stop increments (Live View/ Movie) |
| AEB | +/-3 EV 1/3 or 1/2 stop increments |
| ISO Sensitivity (8) | Auto (100-12800min / 100-25600 max) in 1-stop increments. Manual (100-25600) in 1/3-stop increments, expanded to 51200 equivalent. Minimum limit may change depending on the lens |
| SHUTTER |  |
| Type | Vertical-travel mechanical focal-plane shutter, electronically controlled at all speeds |
| Speed | Creative Zone: 1/16000 – 30 sec, bulb Basic Zone: 1/8000 – 1 sec Available range varies by shooting mode |
| WHITE BALANCE |  |
| Type | Auto white balance with the imaging sensor |
| Settings | AWB (Ambience priority, White priority), Daylight, Shade, Cloudy, Tungsten light, White Fluorescent light, Flash, Custom, Colour Temperature Setting. White balance compensation: 1. Blue/Amber +/-9. Magenta/ Green +/-9 |
| Custom White Balance | Yes, 1 setting can be registered |
| WB Bracketing | +/-3 levels in single level increments |
| VIEWFINDER |  |
| Type | Pentaprism |
| Coverage (Vertical/Horizontal) | Approx. 100% |
| Magnification | Approx. 0.95x (4) |
| Eyepoint | Approx. 22mm (from eyepiece lens centre) |
| Dioptre Correction | -3 to +1 m-1 (dioptre) |
| Focusing Screen | Fixed |
| Mirror | Quick-return half mirror |
| Viewfinder Information | AF information: AF points, focus confirmation, AF area selection modeExposure information: Shutter speed, aperture value, ISO speed (always displayed), AE lock, exposure level/compensation, spot metering circle, exposure warning, AEB.Flash information: Flash ready, high-speed sync, FE lock, flash exposure compensation, red-eye reduction light.Image information: Card information, maximum burst (2 digit display), Highlight tone priority (D+).Composition information: Grid, Electronic level, Aspect RatioOther information: Battery check, Alert symbol, Flicker Detection |
| Depth of field preview | Yes, with Depth of Field preview button. |
| Eyepiece shutter | None. Eyepiece cover attached to strap |
| LCD MONITOR |  |
| Type | Touch screen vari angle 7.7cm (3.0") 3:2 Clear View II TFT, approx. 1040K dots |
| Coverage | Approx. 100% |
| Viewing Angle (horizontally/vertically) | Approx. 170° |
| Coating | Anti smudge |
| Brightness Adjustment | Adjustable to one of seven levels |
| Touch Screen Operations | Touch Shutter Touchscreen sensitivity Beep |
| FLASH |  |
| Type | Manual pop-up retractable flash in the pentaprism housing |
| Guide number | Approx 12/39.4 (ISO100 m/feet) |
| Flash coverage | Approx. 17mm focal length/angle of view |
| Flash exposure compen ation | +/- 3 stops in 1/3 or ½ stop increments. It is available using either built-in flash on the side of the camera or an external flash unit. |
| Built-in Flash recycle time | Approx. 3 seconds |
| Modes | Auto, Manual flash, Integrated Speedlite Transmitter |
| Colour temperature | Equivalent to daylight |
| Red eye reduction | Using the red eye reduction lamp |
| FE lock flash metering | Supported |
| Flash sync speed | Max 1/250 sec |
| Slow synchro | 1/250 – 30 sec (auto) 1/250 – 1/60 sec (auto 1/250 sec (fixed) |
| External Flash Compatibility | EL/EX series Speedlites supported |
| E-TTL II flash metering | Evaluative (Face Priority) Evaluative Average |
| SHOOTING |  |
| Modes | Scene Intelligent Auto (Stills and Movie), SCN (Portrait, Group Photo, Landscape, Close-up, Sports, Night Portrait, Handheld Night Scene, HDR Backlight Control, Food, Kids, Candlelight, Panning), HDR movies, Creative filters, Program AE, Shutter priority AE, Aperture priority AE, Manual (Stills and Movie), Bulb, Custom |
| Picture Styles | Auto, Standard, Portrait, Landscape, Fine Detail, Neutral, Faithful, Monochrome, User Defined (x3) |
| Colour Space | Selectable between sRGB and Adobe RGB |
| Image Processing | Highlight Tone Priority (Disable/Enable/Enhanced) Auto Lighting Optimizer (4 settings) Long exposure noise reduction High ISO speed noise reduction (4 settings) Multi Shot Noise Reduction Auto Correction of Lens Peripheral illumination and Chromatic aberration correction Diffraction correction Digital Lens Optimizer Creative filters (Grainy B/W, Soft focus, Fish-eye effect, Water painting effect, Toy camera effect, Miniature effect, HDR x 4) RAW image processing (Brightness adjustment, White balance, Picture Style, ALO, High ISO speed NR, image quality, Colour space, Lens aberration correction.) |
| Drive modes | Single, Continuous L, Continuous H, Continuous Shooting Panning, Silent Single Shooting, Self timer (2s+remote, 10s +remote, Continuous) |
| Continuous Shooting | One shot: Max approx. 10 fps (viewfinder) / 11 fps (live view) Servo: Max approx. 10 fps (viewfinder) / 11 fps (live view) |
| LIVE VIEW MODE |  |
| Type | Electronic viewfinder with image sensor |
| Coverage | Approx. 100% |
| Frame Rate | 30 fps |
| Focusing | Manual Focus (Magnify the image 5x or 10x at any point on screen)Autofocus: Dual Pixel CMOS AF (Face detection and Tracking AF, FlexiZone-Multi, FlexiZone-Single), Tracking sensitivity (-3 to +3) and Movie Servo AF Speed (- 7 to +2) |
| Metering | Real-time evaluative metering with image sensor. Evaluative metering, partial metering, spot metering, center-weighted average metering. |
| Display Options | Basic, Detailed, On-screen buttons, Histogram, Electronic level |
| FILE TYPE |  |
| Still Image Type | JPEG: (Exif 2.31 compliant) RAW: RAW (14bit, Canon original RAW CR3), |
| RAW+JPEG simultaneous recording | RAW/C-RAW and JPEG simultaneous image recording quality is supported. |

| Principal specifications |  |
| --- | --- |
| Lens type | Prime lens |
| Max Format size | 35mm FF |
| Focal length | 100 mm |
| Image stabilization | Yes |
| CIPA Image stabilization rating | 5 stop(s) |
| Aperture |  |
| Maximum aperture | F2.8 |
| Minimum aperture | F32 |
| Aperture ring | No |
| Number of diaphragm blades | 9 |
| Optics |  |
| Elements | 17 |
| Groups | 13 |
| Special elements / coatings | Super Spectra Coating |
| Focus |  |
| Minimum focus | 0.26 m (10.24″) |
| Maximum magnification | 1.4× |
| Autofocus | Yes |
| Motor type | Nano ultrasonic |
| Full time manual | Yes |
| Focus method | Internal |
| Distance scale | No |
| DoF scale | No |
| Focus distance limiter | Yes |
| Physical |  |
| Weight | 730 g (1.61 lb) |
| Diameter | 82 mm (3.23″) |
| Length | 148 mm (5.83″) |
| Materials | Magnesium alloy |
| Sealing | Yes |
| Colour | Black |
| Filter thread | 67 mm |
| Hood supplied | Yes |
| Aditional |  |
| Lithium-Ion Battery (7.2V, 2130mAh) |  |
| Soft Sided Bag Vanguard DIVIDER BAG 37 sau echivalent compatibil cu aparatul foto și obiectivul |  |