Anexa 1

Caracteristici tehnice camere video fixe.

| Date generale | Camera bullet IP de exterior |
| --- | --- |
| Codecuri | H.265, H.264, MJPEG multiple streaming |
| Dimensiuni Linza | lentila fixa 3.6mm, |
| Tip linza | Limza fixa |
| Rezolutie | Min 4MP |
| Tehnologie | IP |
| Distanta IR | Min 20 m |
| Formatul senzorului de imagine | 1/3" CMOS |
| Alimentare | 12VDC/PoE,consum 7W |
| Audio | audio 1 in, |
| Alarme | alarma 1 in/1 out |
| Protectie | IK10,IP66, detectie miscare si sabotare |
| Temperatura de functionare | emperatura de functionare -30°C~+55°C |
| Alte functionalitati | BLC, SSDR, SSNR, WDR 120dB, suport card SD, mod coridor, LDC,Defocus Detection,ICR mecanic 0.5lux, F1.4, 0lux cu IR pornit |
| Garantie | Min 3 ani |

Caracteristici tehnice camere video mobile.

| Camera Module |  |
| --- | --- |
| Image Sensor | 1/2.8’’ Progressive Scan CMOS |
| Min. Illumination | F1.6, AGC On: Color: 0.05 lux, B/W : 0.01 lux, 0 lux with IR |
| Min. Image Resolutio | 4,0 Mpx(1920 × 1080) |
| Focal Length | 4.7-94.0mm, 20x |
| Digital Zoom | 16X |
| Zoom Speed | Approx.3s(Optical Wide~Tele) |
| Angle of View | 58.3-3.2 degree (Wide~Tele) |
| Min. Working Distance | 10~1,000mm(Wide~Tele) |
| Aperture Range | F1.6~F3.5 |
| Focus Mode | Auto / Semiautomatic / Manual |
| DWDR | Support |
| Shutter Time | 1-1/10,000s |
| AGC | Auto / Manual |
| White Balance | Auto / Manual /ATW/Indoor/Outdoor/Daylight lamp/Sodium lamp |
| Day & Night | IR Cut Filter |
| Privacy Mask | 8 privacy masks programmable |
| Enhancement | 3D DNR, Defog, HLC/BLC, EIS |
| Pan and Tilt |  |
| Range | Pan:360° endless; Tilt: -15°~90°(Auto Flip) |
| Speed | Pan Manual Speed: 0.1°~80°/s, Pan Preset Speed: 80°/s Tilt Manual Speed: 0.1°~80°/s, Tilt Preset Speed: 80°/s |
| Number of Preset | 300 |
| Patrol | 8 patrols, up to 32 presets per patrol |
| Pattern | 4 pattern scans, record time over 10 minutes for each scan |
| Park Action | Preset / Patrol / Pattern / Pan scan / Tilt scan / Random scan / Frame scan / Panorama scan |
| Scheduled Task | Auto scan / Frame scan / Random scan / Patrol / Pattern / Preset / Panorama scan / Tilt scan / Dome reboot / Dome adjust |
| Features |  |
| Detection | Intrusion detection, Line crossing detection, Motion detection |
| ROI encoding | Support 4 areas with adjustable levels |
| Infrared |  |
| IR Distance | Up to 100m |
| IR Intensity | Automatically adjusted, depending on the zoom ratio |
| Network |  |
| Ethernet | 10Base-T / 100Base-TX, RJ45 connector |
| Stream | Support three Streams |
| Main Stream | 50Hz:25fps (1920×1080,1280×960,1280×720), 50fps (1280×960,1280×720) ; 60Hz: 30fps(1920×1080,1280×960,1280×720), 60fps (1280×960,1280×720) |
| Sub Stream | 50Hz:25fps(704×576,640×480,352×288);60Hz:30fps(704×480,640×480,352×240) |
| Three Stream 50 | 50Hz:25fps (1920×1080,1280×960,1280×720,704×576,640×480,352×288); 60Hz: 30fps(1920×1080,1280×960,1280×720,704×480,640×480,352×240) |
| Image Compression | Image Compression H.264/MJPEG |
| Audio Input | 1 audio input (Mic/Line in), 2-2.4V[p-p]; output impedance: 1KΩ, ±10% |
| Audio Output | 1 output, Line level, impedance: 600Ω |
| Audio Compression | G.711/G.722/G.726/MP2L2 |
| Protocols | IPv4/IPv6, HTTP, HTTPS, 802.1X, QoS, FTP, SMTP, UPnP, SNMP, DNS, DDNS, NTP, RTSP, RTP, TCP, UDP, IGMP, ICMP, DHCP, PPPoE |
| Simultaneous Live View | Up to 20 users |
| Mini SD Memory Card | Support up to 128GB Micro SD/SDHC/SDXC card. Support Edge recording |
| User/Host Level | Up to 32 users,3 Levels: Administrator, Operator and User |
| Security Measures | User authentication (ID and PW); Host authentication (MAC address); IP address filtering |
| Application programming | Open-ended API, support ONVIF, PSIA and CG , Support cloud P2P |
| Web Browser | IE 7+, Chrome 18 +, Firefox 5.0 +, Safari 5.02 + |
| Power | 12 VDC &POE(802.3at, class4), Max.20W |
| Working Temperature - | -30°C ~ 65°C (-22°F ~ 149°F) |
| Humidity | 90% or less |
| Protection Level | IP66, TVS 4,000V lightning protection, surge protection and voltage transient protection |
| Certification | FCC, CE, UL, RoHS, IEC/EN 61000, IEC/EN 55022, IEC/EN 55024, IEC /EN60950-1 |
| Dimensions | Φ164.5(mm)×295(mm) |
| Weight (approx.) | 2kg (4.41lbs) |
| Mount Option | Long-arm wall mount: DS-1602ZJ; Corner mount: DS-1602ZJ-corner; Pole Mount: DS1602ZJ-pole; Power box mount:DS-1602ZJ-box; Swan-neck mount: DS-1619ZJ |
| Garanție | Min 3 ani |

Caracteristici tehnice NVR (Registrator Video)

| Date generale | Network video recorder |
| --- | --- |
| Numar de camere suportate | Min 16 cu licenta inclusa |
| Sistem de operare | Windows 10 IoT Enterprise |
| Memorie RAM | Min 8GB |
| Storage | 2x2TB |
| CPU | Min i3 |
| Alimentare | 220 AC |
| I/O | 2 x GbE, RJ45, 4 x USB 3.0, 1 x DVI/1, 1 x HDMI, 1 x line-out/mic-in/line-in, 1 x eSATA, 1 x Alarm I/O |
| Mediu de operare | Temperatura de operare 0~40°C, storage temp: -20°C ~70°C |
| Certificate | CE, FCC, UL |
| exportul datelor | AVI, MKV, JPEG,comentariu de export. |
| Video push | + |
| Manager alarme | + |
| Cautare smart | + |
| Compabilitate | ONVIF cameras, Microsoft Active Directory, interconectarea cu sistemul IGPF |
| Codecuri | MJPEG, MPEG4, MPEG4 ASP, MxPEG, H.264 și H.265 pentru înregistrarea camerelor IP și a codificatoarelor video IP. |
| PTZ | control manual, activarea presetărilor PTZ, prioritatea PTZ, gestionarea presetărilor PTZ și patrolling, blocarea / deblocarea presetărilor PTZ și lansarea sesiunii PTZ. |
| Functie harta | Suportă servicii de hartă, cum ar fi Bing, Google și OpenStreetMaps, precum și hărți GIS geografice și desene CAD. |
| Clienti | Web, Windows si Mobile(iOS si Android) |
| Securitate | Suporta caracteristicile de securitate digitală (criptarea AES pe 64 de biți DES 128, 192 și 256-biți și protecția prin parolă, semnătura digitală a videoclipului exportat, conexiunea securizată a camerei HTTPS pe dispozitivele care suportă HTTPS) |
| Garantie | 3 ani |