Caiet de sarcini

| Technical specifications per server. |  |  |
| --- | --- | --- |
| 1.1 | Type | Enterprise Server, Quantity = 2pcs. |
| 1.2 | Form factor | Rack mount, max 2U (rail-kit included). |
| 1.3 | CPU included | Quantity: 2 x CPU per server. Cores: 32cores per CPU. Threads: 64 threads per CPU. Base Clock Speed: Min. 2.60 GHz. Cache: Min. 50 MB. Launch Date: Not earlier than Q1’2023. |
| 1.4 | Memory | Min. 32 DDR5 slots. |
| 1.5 | Memory included | Min. 12 x 64GB DDR5. |
| 1.6 | Min. drive bays included | Min. 24 bays 2.5” for data, Support Hot-Swap drives. Min. 2 bays for OS boot, RAID 1, Support Hot-Swap drives. |
| 1.7 | Drives included | Min. 3 x 1.6TB SSD, Caching Tier, Endurance Class D , Performance Class F, SAS 24Gb/s, Hot-Swap. Min. 21 x 3.2TB SSD, Capacity Tier, Endurance Class D min.15000TBW, Performance Class F, SAS 24Gb/s, Hot-Swap. Drives SSD for vSAN should support min. vSAN releases 7.0U3-8.0U3. Min. 2 x 240GB SSD for OS boot, Hot-Swap. |
| 1.8 | Storage Adapters included | Should support Pass-through mode for all bays. Should be in vSAN compatibility guide. Should be compatible with min. vSAN releases 7.0U3-8.0U3. 1 x RAID controller for OS boot, 2 x SSD, RAID-1. |
| 1.9 | NICs, ports, modules included (Server side) | Min. 2 x NIC’s, 2 x 10/25GbE ports, identical NIC’s models. NIC’s should support network virtualization offload. NIC’s should support RDMA (RoCE v2). NIC’s should be in vSAN compatibility guide. NIC’s should support min. vSAN releases 7.0U3-8.0U3. Min. 4 x SFP-10G-SR SFP+ MM modules. Min. 1 x Dedicated 1Gb/s Ethernet Port for remote management. |
| 1.10 | Modules included (Switch side) | Min. 4 x SFP28-25Gb-SR (Cisco Compatible). |
| 1.11 | Interfaces included | Min. 2 x USB ports. Min. 1 x VGA port. |
| 1.12 | Options included | Trusted Platform Module 2.0. |
| 1.13 | Supported operating environments | Microsoft Windows Server®. Microsoft Hyper-V®. Red Hat® Enterprise Linux. VMware® (VMware® vSphere, VMware® vSAN 7.0U3-8.0U3). |
| 1.14 | Power supplies included | 2 Hot-Swap PSU with support for 1+1 redundancy. Power cables c13-c14(0.6 m). |
| 1.15 | Fan Modules included | Hot-Swap with N+1 redundancy. |
| 1.16 | Management included | A web-based solution for KVM must be included with full functionality for manage and monitoring, including at least: - View information about the state of the managed server; - Inventory and monitoring of network adapters and data storage subsystems without software agents in the OS; - View inventory information; - View information from sensors; - Monitoring and control of electricity consumption; - Turn on/off the server; - Remote update of BIOS, firmware of network and RAID controllers; - Virtual console, virtual media devices; - OS installation from virtual media devices and through network shared folders. Centralized resource management solution for hardware management must be included, with full functionality that provides agent-free management, automates discovery, inventory, tracking, updates, monitoring, and provisioning". |
| 1.17 | Warranty | Hardware and software support from vendor, with available customer portal. Next business day hardware replacement included for min. 5 years. |
| 1.18 | Terms and conditions | All requirements are minimum and mandatory. One requirement must not limit another requirement. All components must be current and not advertised as EOS / EOL. The expansion of memory (ram) and storage capacity must not include any hardware or software limitations. Faulty storage drives are not subject of return in order to be compliant with the security requirements. |