ANUNȚ DE PARTICIPARE

privind achiziționarea Echipament informatic
(se indică obiectul achiziției)
prin procedura de achiziție  Cererea Ofertelor de preț,

licitație electronică, Numărul de runde -5____________
                                                                          (tipul procedurii de achiziție)

- Denumirea autorității contractante: Î.S. ”Institutul de Dezvoltare a Societății Informaționale”

- IDNO: 1008600032684

- Adresa: Mun. Chișinău, str. Academiei, 5a

- Numărul de telefon/fax: (022) 28 98 47, (022) 28 98 49

- Adresa de e-mail și de internet a autorității contractante: info@idsi.md
www.idsi.md

- Adresa de e-mail sau de internet de la care se va putea obține accesul la documentația de atribuire: documentația de atribuire este anexată în cadrul procedurii în SIA RSAP

- Tipul autorității contractante și obiectul principal de activitate (dacă este cazul, mențiunea că autoritatea contractantă este o autoritate centrală de achiziție sau că achiziția implică o altă formă de achiziție comună): Autoritate contractantă

- Cumpărătorul invită operatorii economici interesați, care îi pot satisface necesitățile, să participe la procedura de achiziție privind livrarea/prestarea/executarea următoarelor bunuri /servicii/lucrări:

| Nr. d/o | Cod CPV | Denumirea bunurilor/serviciilor/lucrărilor solicitate | Unitatea de măsură | Cantitatea | Specificarea tehnică deplină solicitată, Standarde de referință | Valoarea estimată (se va indica pentru fiecare lot în parte) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 32500000-8 | Lotul 1 – HDD-uri |  |  |  | 3100 |
| 1 | 32500000-8 | HDD-uri | Buc. | 2 | 1TB 5.4K RPM SATA 2.5" |  |
|  | 32500000-8 | Lotul 2 – Server |  |  |  | 192 400 |
| 1 | 32500000-8 | Server | Buc. | 1 | Server parameters: Number of CPU Sockets: 2 Form factor (fully configured): 1U Form factor chassis: Rack Power supply type supported: 2 x 800W Flex Slot Platinum Hot Plug Power Power supply installed: 2 Installed CPU qty: 2 Expansion slots: 2xPCIe 3.0 Memory: Installed Size: 4x32 Gb (two modules for every CPU) RDIMM module Total Memory slots: 24 DIMM slots Memory type: DDR4 SmartMemory, Advanced ECC Memory Form Factor: RDIMM / LRDIMM Hard Drive Supported: 8xSFF drives Dimensions: 4.29 x 43.46 x 70.7 cm Weight: 13.04 kg minimum, 16.27 kg maximum Server remote management module specifications: - Embedded ARM processor core operating at 800 MHz - PCI express based health and remote management ASIC - Firmware upgradeable VIA flash ROM - Video support: 1920 x 1200 (32 bp) - 10/100/1000 Mb/s network interface - HTML5 remote console (No dependency on JAVA or .NET) - Native browser support for Chrome, Firefox, Edge, Internet Explorer, Safari - Keyboard, video, mouse redirection - Virtual media support (VIA URLs) - Support for Online Certificate Status Protocol (OCSP) for CAC/SmartCard/Certificate based authentication - Support for RSA-PSS certificate signatures - The management processor must obtain its power from the auxiliary power plane of the server, so it must be always ON when the server is plugged into a power source. If the server provides Redundant Power Supplies (RPS) then the management processor will use redundant power and will continue operation in the event of a power supply failure. - Monitor and alerting on key internal server components: CPUs, memory, temperatures, fans, Smart Array controllers, hard drives (Including cache modules) and power supplies without installing any driver. - Ability to put harden management processor module for specific security requirements. - Possibility to prevent any possibility of tampering of security signatures throughout the supply-chain. - The system must give users preview of the system health summary and sub-system details. - Diagnostics tools/scanners wrapped into one; always on, continuous monitoring; Rich configuration history; health and service alerts; Easy export and upload to service and support. - Provisioning and configuring the server without any separate media (No Smart Cards or CDs). - Command line support: SSHv2, RESTful API and Serial port. - RESTful API options supported: get full inventory, control power and reset, configure BIOS and Smart Array settings, status of server health, fetch event logs. - Security: runtime firmware validation, CNSA support, security modes, encrypted virtual media. - Active directory support. - Management protocols supported: SNMP, IPMI V2.0. - Possibility to connect to the remote management module trough a USB to ethernet cable. Some of the features may require additional license. USB3.0 support: 1 front port, 2 x rear ports and 2 x internal ports Power On/Standby button and system power LED UID button/LED MicroSD Card slot Warranty: 3/3/3 - Server Warranty includes three years of parts, three years of labor, three years of onsite support coverage. Not refurbished (Certificat/document care ar confirma că echipamentul nu este refurbished) Networking: 2x10Gbps Base-T adapters 802.1Q VLAN support Jumbo Frames: 9000KB Network adapter teaming CPU Parameters: CPU Cache: 35 MB CPU Clock Speed: 3 GHz CPU socket: FCLGA3647 Max Turbo Speed: 4 GHz Number of Cores: 24-core Number of Threads: 48 threads Thermal Design Power: 205 W Lithography: 14nm Max Memory Size: 1TB Memory Type: DDR4-2933 Instruction Set Extensions: Intel® SSE4.2, Intel® AVX, Intel® AVX2, Intel® AVX-512 Hyper-Threading Technology: Yes Execute Disable Bit: Yes Security: - UEFI Secure Boot and Secure Start support - Immutable Silicon Root of Trust - FIPS 140-2 validation - Common Criteria certification - Configurable for PCI DSS compliance - Advanced Encryption Standard (AES) and Triple Data Encryption Standard (3DES) on browser - Smart card (PIV/CAC) and Kerberos based 2-factor Authentication - Secure Recovery – recover critical firmware to known good state on detection of compromised FW - Ability to rollback firmware - Secure erase of NAND - TPM (Trusted Platform Module) 1.2 option - TPM (Trusted Platform Module) 2.0 option - Bezel Locking Kit - Chassis Intrusion detection option Industry Standard Compliance: - ACPI 6.1 Compliant - PCIe 3.0 Compliant - WOL Support - Microsoft® Logo certifications - PXE Support - USB 3.0 Compliant - USB 2.0 Compliant (only on optional Universal Media Bay) - SMBIOS 3.1 - UEFI 2.6 (Unified Extensible Firmware Interface Forum) - Secure Digital 4.0 - TPM 1.2 and 2.0 support - Advanced Encryption Standard (AES) - Triple Data Encryption Standard (3DES) - SNMP v3 - TLS 1.2 - ASHRAE A3/A4 UEFI Capabilities: - Secure Boot and Secure Start enable for enhanced security - Operating system specific functionality - Support for > 2.2 TB (using GPT) boot drives - USB 3.0 Stack - Embedded UEFI Shell - PXE boot support for IPv6 networks - Workload Profiles for simple performance optimization UEFI Boot mode only: - TPM 2.0 Support - NVMe Boot Support - Platform Trust Technology (PTT) can be enabled - iSCSI Software Initiator Support. - HTTP/HTTPs Boot support as a PXE alternative - Boot support for option cards that only support a UEFI option ROM Supported Operating Systems: - Windows Server 2019: Essentials, Standard, Datacenter - Windows Server 2016: Essentials, Standard, Datacenter - Windows Server 2012 R2: Essentials, Standard, Datacenter - Microsoft Hyper-V Server: 2012 R2, 2016 & 2019 - VMware vSphere: 6.0 U3, 6.5 U2 through U.3 & 6.7 U1 through U3, 7.0 - ClearOS: 7.6; ClearVM: 2.0 - Red Hat Enterprise Linux (RHEL): 7.6 w/ Kbase **, 8.0 - SUSE Linux Enterprise Server (SLES): 12 SP3, 15 (includes Xen) - Ubuntu: 18.04 LTS (4.15.0) - Oracle Linux: Oracle Linux 7.6 UEK Release 5 Update 2; Oracle VM 3.4.6 (UEK Release 4 Update 7) - Citrix: Hypervisor: 8,0, 8.1, 8.2; XenServer 7.1, 7.4, 7.5, 7.6 HDD: 2x300 Gb SAS 12G Enterprise 10K SFF(2.5 in) hot swap. All additional components that will be ordered need to be installed. Sau echivalent: HPE ProLiant DL360 Gen10 ( SKU # P24743-B21) |  |
| 2 |  | HBA pentru serverul din pozitia 1, in caz ca serverul nu are deja HBA cu astfel de parametri. | Buc. | 1 | HBA Parameters: 16 Gb Dual Port Fibre Channel Host Bus Adapter Backward compatible with 8 and 4Gb FC environment. SFPs and optical cables included |  |
| 3 |  | Raid Controller pentru serverul din pozitia 1, in caz ca el nu are deja controller cu astfel de parametri. | Buc | 1 | Storage interface (SAS/SATA): - 8 SAS lanes across 2x4 internal Mini-SAS ports - 12 Gb/s SAS, 6 Gb/s SATA technology - Mix-and-match SAS and SATA drives to the same controller - Support for SAS tape drives 2GB FBWC PCI Express 3.0 x8 link RAID 0, 1, 5, 6, 10, 50, 60, 1 Advanced Data Mirroring (ADM), 10 ADM Mixed mode (RAID logic drives and HBA physical drives simultaneously) Legacy and UEFI boot operation UEFI System Utilities (Storage configuration) Up to 238 physical drives Up to 64 logical drives Smart Array SR SmartCache (Optional license) Smart Array SR Secure Encryption (Optional license) SSD Smart Path Rapid Parity Initialization (RPI) Rapid rebuild Drive sanitize Performance optimization degraded reads and read coalescing Performance: 12 Gb/s SAS (1200 MB/s theoretical bandwidth per SAS lane) 6 Gb/s SATA (600 MB/s theoretical bandwidth per SAS lane) PCI Express 3.0 x8 link width 2GB 72-bit wide DDR4-2100 FBWC provides up to 16.8 GB/s maximum cache bandwidth Read ahead caching Write-back caching Array configuration features: Expand Array Heal Array Move Drives Re-Mirror Array Shrink Array Spare Management Split Mirrored Array Logical drive configuration features: Extend Logical Drive Instant Secure Erase Migrate RAID/Stripe Size Modifying Cache Write Policy Move Logical Drive Re-enable Failed Logical Drive Fault prevention features: Predictive drive failure detects possible drive failure before it occurs, allowing replacement ofthe component before failure occurs Background surface scan checks drives during inactive periods and automatically remaps badsectors, ensuring data integrity Smart SSD Wear Gauge reports the amount of lifetime remaining on users SSDs Fault recovery: - Recovery ROM: This feature protects the user from a firmware image corruption by storing aredundant copy of the firmware image. If the active firmware image becomes corrupt, the controller will use the redundant firmware image and continue operating. - On-Line Spares: There is no limit to the number of spare drives that can be installed prior todrive failure. If a failure occurs, recovery begins with an On-Line Spare and data is reconstructed automatically - DRAM ECC detects and corrects data bit errors. SSD Smart Path feature included in the Smart Array software stack improves Solid State Disk (SSD) read performance by up to 4x. Smart Path chooses the optimum path to the SSD and accelerates reads for all RAID levels and RAID 0 writes. Operating systems support: - Microsoft Windows Server - Microsoft Windows Hyper-V Server - VMware vSphere ESXi - Red Hat Enterprise Linux (RHEL) - SUSE Linux Enterprise Server (SLES) - Oracle Solaris - Oracle Linux - Clear OS - Cent OS - Citrix XenServer Storage management software available Sau echivalent HPE Smart Array P408i-a Controller. |  |
|  | 32500000-8 | Lotul 3 – Accesorii echipament de rețea |  |  |  | 17 900 |
| 1 | 32500000-8 | SFP Fibra Optica | Buc. | 2 | Application: 10GBASE-LR SFP+ Max Distance: 10km Fiber Type: Single Mode Compatibility: Cisco Catalyst 9200-L Series (C9200L-48P-4X) |  |
| 2 |  | SFP UTP | Buc. | 2 | 10GBASE-T module Connectivity options at the following data rates: 100M/1G/10Gbps. Form factor: SFP+ Interface: RJ-45 interface Cable supported: UTP CAT5e/CAT6A/CAT7 UTP Cable length: 100M/1G – up to 100M; 10G – up to 30M Compatibility: Cisco Catalyst 9200-L Series (C9200L-48P-4X) |  |
| 3 | 32500000-8 | Mini computer | Buc. | 1 | - Broadcom BCM2711, Quad core Cortex-A72 (ARM v8) 64-bit SoC @ 1.5GHz - 4GB LPDDR4-3200 SDRAM - 2.4 GHz and 5.0 GHz IEEE 802.11ac wireless, Bluetooth 5.0, BLE - Gigabit Ethernet - 2 USB 3.0 ports; 2 USB 2.0 ports. - 40 pin GPIO header - 2 × micro-HDMI ports (up to 4kp60 supported) - 2-lane MIPI DSI display port - 2-lane MIPI CSI camera port - 4-pole stereo audio and composite video port - H.265 (4kp60 decode), H264 (1080p60 decode, 1080p30 encode) - OpenGL ES 3.1, Vulkan 1.0 - Micro-SD card slot for loading operating system and data storage - 5V DC via USB-C connector (minimum 3A*) - 5V DC via GPIO header (minimum 3A*) - Power over Ethernet (PoE) enabled (requires separate PoE HAT) - Operating temperature: 0 – 50 degrees C ambient - MicroSD Class 10 64GB Card - Case - 3.5” HDMI touch screen display - Mainboard dimensions: 85mmx56mm Sau echivalent: Raspberry Pi 4 Model B/4GB |  |
| 4 | 32500000-8 | Filtru pt UPS-uri | Buc. | 6 | Size: 19" rack mountable Rack Units: 1.5U Output Connectors: 10 x IEC 60320 C13 Input Connector: IEC 60320 C20 Input Current: 16A Cable Length: 1.8M |  |
| 5 | 32500000-8 | Filtru pt UPS-uri | Buc. | 6 | Size: 19" rack mountable Rack Units: 1.5U Output Connectors: 10 x IEC 60320 C13 Input Connector: CEE 7/4 Input Current: 16A Cable Length: 1.8M |  |
| 6 | 32500000-8 | Adapter USB to COM | Buc | 3 | Supported by Linux and Windows operating systems. |  |
|  | 32500000-8 | Lotul 4 – Echipament Video |  |  |  | 83 500 |
| 1 | 32500000-8 | Camera video | Buc | 1 | - Senzor - Exmor R CMOS de tip 1.0 inci - Elemente de imagine eficiente - Approx. 14.2 M pixels (16:9)/Approx. 10.6 M pixels (4:3) - Filtre optica încorporata - OFF: Clear, 1: 1/4ND, 2: 1/16ND, 3: 1/64ND - Iluminare minima - [50i] 1.7 lux (LOW LUX mode) - Viteza obturatorului - [50i]: 1/6 -1/10,000 - Funcție de mișcare lentă și rapidă - [50i]: 1080p: Frame rate selectable 1.2.3.6.12.25,50 fps - Echilibru alb – Preset - Gamma Curve – Selectable - Formate inregistrare video -XAVC S 4K (3840 x 2160)@25p, 100Mbps/60Mbps XAVC S HD (1920 x 1080)@50p, 25p, 50 Mbps AVCHD (1920 x 1080)@50p, PS mode (28 Mbps) AVCHD (1920 x 1080) @50i/25p, FX mode (24 Mbps) FH mode (17 Mbps) AVCHD (1440 x 1080)@50i, HQ mode (9 Mbps) LP mode (5 Mbps) AVCHD (1280 x 720)@50p, FX mode (24 Mbps) FH mode (17 Mbps) HQ mode ( 9 Mbps) - Formate inregistrare audio - XAVC S 4K : Liniar PCM 2 canale, 16 biti, 48kHz XAVC S HD : Liniar PCM 2 canale, 16 bit, 48kHz AVCHD : Liniar PCM 2 canale, 16 biti, 48 kHz / Dolby Digital 2 canale, 16 biti, 48 kHz DV : Liniar PCM 2 canale, 16 biti, 48 kHz - Intrari audio – 2x XLR cu 3 pini (mama), Line/Mic/Mic/+48 V selectabil - Iesire video – Conector BNC (×1), Compozit 1.0Vp-p, 75 Ω - Iesire HDMI – Tip A - Iesiri audio – RCA pin, Stereo mini jack - USB – 1x port Multi/Micro USB - Difuzoare incorporate - Vizor – Approx. 0,6 cm (0,24 tip) Aprox. 1,56 M puncte. - Stabilizare imagine - ON / OFF selectabil, - Lentile de schimbare Iris - F2.8 - F11 auto/manual selectable - Diametru filtru – Approx M62 mm - Alimentare – DC - Baterie detasabila |  |
| 2 | 32500000-8 | Microfon pentru camera video | Buc | 1 | - Numar transmitatoare: 1 - Numar receptoare: 1 - Tip microfon: Lavaliera omni - Conector transmitator: TRS - Conector casti: Da - Tip receptor: Portabil - Banda comunicare: UHF - Alimentare: R6 - Display & Indicators - Form Factor: Camera-Mount - Antenna: 2 x 1/4 Wave External, Fixed (Top-Mount) - Mounting Options: Shoe-Mount (with Included Hardware) - Tip transmitator: Body pack - Compatibil cu camera din Lot 4 punct 1 |  |
| 3 | 32500000-8 | Stativ pentru camera | Buc | 1 | - Spreader - La mijloc - Incarcatura sustinuta – Approx. 5 kg - Greutate – Approx. 3.6 kg - Material trepied – Aluminiu - Inaltime – Approx. 154 cm - Dimensiune strans – Approx. 72cm - Husă transport - Compatibil cu camera din Lot 4 punct 1 |  |
| 4 | 32500000-8 | Rucsac pt camera | Buc | 1 | - Compatibil cu camera din Lot 4 punct 1 + loc pentru accesorii (baterii, incarcator, microfon, echipament de streaming, modem, cabluri) - Impermeabil (Rezistent la apa si praf) |  |
| 5 | 32500000-8 | Card de memorie camera | Buc | 2 | - Tip Card Memorie – SDHC - Capacitate - 128GB - Clasa de Viteza – 10 - Tehnologie - UHS-I - Rata de transfer (MB/s) – 120 - Dimensiuni - 2,17 x 23,91 x 31,82 mm |  |
|  | 32500000-8 | Lotul 5 - Echipament streaming |  |  |  | 36 700 |
| 1 | 32500000-8 | Echipament streaming | Buc | 1 | Wireless; Wi-Fi: Wi-Fi 4 (802.11n) Interfaces; Video Connectors: 1 x HDMI Input: Audio Connectors: 1 x 1/8" / 3.5 mm TRS Stereo: Other I/O: 2 x USB Mini-B USB 2.0 1 x RJ45 10/100 Ethernet: Embedded Audio: HDMI: IP Encoding/Decoding; IP Video: H.264: 1280 x 720 to 1920 x 1080 at 24p, 25p, 30p, 50i, 50p, 59.94i, 60p |  |
| 2 | 32500000-8 | Mixer pentru video steaming | Buc | 1 | Intrari minim 4 x HDMI tip A 2 x mufe audio de 3.5 mm stereo analogic 1 x RJ45 Ethernet Iesiri minim 1 x HDMI 1 x USB tip C Formate video pe intrare HDMI Video: 1080p: 60, 59.94, 50, 30, 29.97, 25, 24, 23.98 1080i: 60, 59.94, 50 720p: 60, 59.94, 50 Rezolutii HDMI pentru computere: 1080p: 60, 59.94, 50, 30, ... 1080i: 60, 59.94, 50 720p: 60, 59.94, 50 Formate video pe iesire 1080p: 60, 59.94, 50, 30 ... Sampling video Audio embedat Intrare si iesire pe 2 canale Mixer audio Rezolutie monitor 1366 x 768 sau peste |  |
|  | 32500000-8 | Lotul 6 - UPS-uri si extensii de baterii |  |  |  | 40 100 |
| 1 | 32500000-8 | APC Replacement battery cartridge #140 | Buc. | 2 | Pentru APC Smart-UPS RT 10,000VA |  |
| 2 | 32500000-8 | Acumulator, 12V, 7Ah | Buc. | 3 | Compatibil cu Back-UPS CS 650 |  |
| 3 | 32500000-8 | Acumulator 12V, 12 Ah, SV12120 | Buc. | 4 | Compatibil cu UPS APC 1000 |  |
| 4 | 32500000-8 | APC Replacement Battery Cartridge #43 | Buc. | 1 | Pentru UPS APC 3000VA RMI2U |  |
| 5 | 32500000-8 | Acumulator 12V, 9A | Buc. | 3 | Compatibil cu ART 1KVA |  |
| 6 | 32500000-8 | Acumulator pentru APC 2200 | Buc. | 3 | Compatibil cu APC 2200 |  |
|  | 32500000-8 | Lotul 7 – Laptop |  |  |  | 16 700 |
| 1 | 32500000-8 | Laptop | Buc | 1 | 16" / Core i7 / 16GB / 512GB SSD, wifi 2.4 Ghz and 5 Ghz bands, bluetooth, LAN 1Gbps, HDMI out Sistem de operare windows 10 Pro english |  |
| Valoarea estimativă totală |  |  |  |  |  | 390400,00 |

- În cazul în care contractul este împărțit pe loturi un operator economic poate depune oferta (se va selecta):

- Pentru un singur lot;

- Pentru mai multe loturi;

- Pentru toate loturile;

- Alte limitări privind numărul de loturi care pot fi atribuite aceluiași ofertant___________________________________________________________________.

- Admiterea sau interzicerea ofertelor alternative: ___nu se admite_____________

(indicați se admite sau nu se admite)

- Termenii și condițiile de livrare/prestare/executare solicitați: În decurs de 45 zile din data semnarii contractului şi înregistrării acestuia la Agenţia Achiziţii Publice, livrate la adresa MD-2028, str, Academiei, 5A, mun. Chişinău, Republica Moldova

- Termenul de valabilitate a contractului: 31 decembrie 2021

- Contract de achiziție rezervat atelierelor protejate sau că acesta poate fi executat numai în cadrul unor programe de angajare protejată (după caz): ________nu________________

(indicați da sau nu)

- Prestarea serviciului este rezervată unei anumite profesii în temeiul unor acte cu putere de lege sau al unor acte administrative (după caz): __________nu_______________________________________________________________

(se menționează respectivele acte cu putere de lege și acte administrative)

- Scurta descriere a criteriilor privind eligibilitatea operatorilor economici care pot determina eliminarea acestora și a criteriilor de selecție; nivelul minim (nivelurile minime) al (ale) cerințelor eventual impuse; se menționează informațiile solicitate (DUAE, documentație):

| Nr. d/o | Descrierea criteriului/cerinței | Mod de demonstrare a îndeplinirii criteriului/cerinței: | Nivelul minim/ Obligativitatea |
| --- | --- | --- | --- |
| 1 | Formularul DUAE | Original, completat și confirmat prin aplicarea semnturii și ștampilei participantului *(se va prezenta la depunerea ofertei de către toți ofertanți) | Obligatoriu |
| 2 | Oferta de preț | Formularul F4.2 din documentația standard (capitolul IV), confirmat prin aplicarea semnăturii și ștampilei participantului. *(se va prezenta la depunerea ofertei de către toți ofertanți) | Obligatoriu |
| 3 | Oferta tehnică | Formularul F4.1 din documentația standard (capitolul IV), confirmat prin aplicarea semnăturii și ștampilei participantului. *(se va prezenta la depunerea ofertei de către toți ofertanți) | Obligatoriu |
| 4 | Extras din Registru de Stat a persoanei juridice | Copie-semnat si ștampilat de către operatorul economic | Obligatoriu |
| 5 | Certificat de conformitate sau alt certificat ce confirmă calitatea bunului oferit | Semnat electronic de către operatorul economic - (original - semnat și ștampilat de către operatorul economic –se va prezenta la semnarea contractului) | Obligatoriu |
| 6 | Certificat/document care ar confirma că echipamentul nu este refurbished | Semnat electronic de către operatorul economic - (original - semnat și ștampilat de către operatorul economic –se va prezenta la livrarea echipamentului) | Obligatoriu |
| 7 | Certificat/document care ar confirma numărul de înregistrare ca producator de echipamente electrice și electronice (EEE) în conformitatea cu HG 212/2018 privind gestionarea Echipamentelor Electrice si Electronice si deseuri ce rezulta din ele (Regulament DEEE) | Semnat electronic de către operatorul economic - (original - semnat și ștampilat de către operatorul economic –se va prezenta la livrarea echipamentului) | Facultativ |

- Motivul recurgerii la procedura accelerată (în cazul licitației deschise, restrînse și al procedurii negociate), după caz nu se aplică

- Tehnici și instrumente specifice de atribuire (dacă este cazul specificați dacă se va utiliza acordul-cadru, sistemul dinamic de achiziție sau licitația electronică): nu se aplică

- Condiții speciale de care depinde îndeplinirea contractului (indicați după caz): nu se aplică

- Criteriul de evaluare aplicat pentru adjudecarea contractului: prețul cel mai scăzut în corespundere cu cerințele solicitate

- Factorii de evaluare a ofertei celei mai avantajoase din punct de vedere economic, precum și ponderile lor:

| Nr. d/o | Denumirea factorului de evaluare | Ponderea% |
| --- | --- | --- |
|  | Nu se aplică |  |

- Termenul limită de depunere/deschidere a ofertelor:

- până la: [ora exactă] Conform informației din SIA RSAP ”MTender”

- pe: [data] Conform informației din SIA RSAP ”MTender”

- Adresa la care trebuie transmise ofertele sau cererile de participare:

Ofertele sau cererile de participare vor fi depuse electronic prin intermediul SIA RSAP

- Termenul de valabilitate a ofertelor: 30 zile

- Locul deschiderii ofertelor: ________ SIA RSAP ”MTender”____________________

(SIA RSAP sau adresa deschiderii)

Ofertele întîrziate vor fi respinse.

- Persoanele autorizate să asiste la deschiderea ofertelor: 
Ofertanții sau reprezentanții acestora au dreptul să participe la deschiderea ofertelor, cu excepția cazului cînd ofertele au fost depuse prin SIA “RSAP”.

- Limba sau limbile în care trebuie redactate ofertele sau cererile de participare:

limba de stat_________________________________________________________

- Respectivul contract se referă la un proiect și/sau program finanțat din fonduri ale Uniunii Europene: Nu

- Denumirea și adresa organismului competent de soluționare a contestațiilor:

Agenția Națională pentru Soluționarea Contestațiilor

Adresa: mun. Chișinău, bd. Ștefan cel Mare și Sfânt nr.124 (et.4), MD 2001;

Tel/Fax/email: 022-820 652, 022 820-651, contestatii@ansc.md

- Data (datele) și referința (referințele) publicărilor anterioare în Jurnalul Oficial al Uniunii Europene privind contractul (contractele) la care se referă anunțul respective (dacă este cazul): Nu

- În cazul achizițiilor periodice, calendarul estimat pentru publicarea anunțurilor viitoare: Nu

- Data publicării anunțului de intenție sau, după caz, precizarea că nu a fost publicat un astfel de anunţ: Nu

- Data transmiterii spre publicare a anunțului de participare: Conform informației din SIA RSAP ”MTender”

- În cadrul procedurii de achiziție publică se va utiliza/accepta:

| Denumirea instrumentului electronic | Se va utiliza/accepta sau nu |
| --- | --- |
| depunerea electronică a ofertelor sau a cererilor de participare | Da |
| sistemul de comenzi electronice | Da |
| facturarea electronică | Da |
| plățile electronice | Nu |

- Contractul intră sub incidența Acordului privind achizițiile guvernamentale al Organizației Mondiale a Comerțului (numai în cazul anunțurilor transmise spre publicare în Jurnalul Oficial al Uniunii Europene): Nu

- Alte informații relevante: ______________________________________________________

Conducatorul grupului de lucru pentru achizitii:

dr. Cernăuțeanu Victor                                        .                 ______________

(Nume, Prenume)                                                        (Semnătura)