| Anexa nr. 22 la Documentația standard nr. ocds-b3wdp1-MD-1739369651562 Specificații tehnice Numărul procedurii de achiziție: ocds-b3wdp1-MD-1739369651562 din 12.02.2025 Obiectul achiziției: Lumini, becuri și piese de rezervă pentru Balizajul luminos RWY 08-26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Acest tabel va fi completat de către ofertant în coloanele 2, 3, 4, 6, 7, iar de către autoritatea contractantă – în coloanele 1, 5,]. |
| Denumirea bunurilor/ serviciilor | Denumirea modelului bunului/ serviciului |  |  | Ţara de origine | Produ-cătorul | Specificarea tehnică deplină solicitată de către autoritatea contractantă | Specificarea tehnică deplină propusă de către ofertant | Standarde de referinţă |  |
| 1 | 2 |  |  | 3 | 4 | 5 | 6 | 7 |  |
| Becuri |  |  |  |  |  |  |  |  |  |
| Bec cu halogen |  |  |  |  |  | 48W 6,6A,DICHTROIC female |  |  |  |
| Bec cu halogen |  |  |  |  |  | 105W 6,6A DICHTROIC female |  |  |  |
| Bec cu halogen |  |  |  |  |  | HLX-C 200W-15 6,6A PK 30d |  |  |  |
| Bec cu halogen |  |  |  |  |  | HLX 100W-15 6,6A PK 30d |  |  |  |
| Bec cu halogen |  |  |  |  |  | EWR 150W 6,6A GY9,5 |  |  |  |
| Lumini |  |  |  |  |  |  |  |  |  |
| Inset Lights (W/R) |  |  |  |  |  | Runway Centerline ¦ Straight ¦ Photometrics: ICAO ¦ CAT III ¦ 8 Inch inset ¦ Side A: White, no toe-in ¦ Side B: Red, no toe-in ¦ Sapphire prism ¦ Cable: 250 mm, one plug ¦ Plug: standard FAA 2-pole plug ¦ With fail-open (relay) ¦ No heater ¦ No special options ¦ 011.633-717 |  |  |  |
| Inset Lights (G/G) |  |  |  |  |  | Taxiway Centerline ¦ Straight ¦ Photometrics: ICAO ¦ CAT III ¦ 8 Inch inset ¦ Side A: Green, no toe-in ¦ Side B: Green, no toe-in ¦ Sapphire prism ¦ Cable: 250 mm, one plug ¦ Plug: standard FAA 2-pole plug ¦ With fail-open (relay) ¦ No heater ¦ No special options ¦ 011.635-537 |  |  |  |
| Inset Lights (G/0) |  |  |  |  |  | Taxiway Centerline ¦ Straight ¦ Photometrics: ICAO ¦ CAT III ¦ 8 Inch inset ¦ Side A: Green, no toe-in ¦ Side B: Blank ¦ Sapphire prism ¦ Cable: 250 mm, one plug ¦ Plug: standard FAA 2-pole plug ¦ With fail-open (relay) ¦ No heater ¦ No special options ¦ 011.637-077 |  |  |  |
| LED HIRE Elevated |  |  |  |  |  | Runway Edge ¦ Straight ¦ Photometrics: ICAO ¦ CAT III ¦ Elevated ¦ Side A: White, 4,0° toe-in ¦ Side B: Yellow, 4,0° toe-in ¦ Glass for elevated lights ¦ Cable: 110 mm, one plug ¦ Plug: standard FAA 2-pole plug ¦ No fail-open ¦ No heater ¦ No special options ¦ HIRE-Family ¦ LED ¦ Coupling Type A ¦ 012.765-002 |  |  |  |
| LED HIRE Elevated |  |  |  |  |  | Runway Edge ¦ Straight ¦ Photometrics: ICAO ¦ CAT III ¦ Elevated ¦ Side A: Yellow, 4,0° toe-in ¦ Side B: White, 4,0° toe-in ¦ Glass for elevated lights ¦ Cable: 110 mm, one plug ¦ Plug: standard FAA 2-pole plug ¦ No fail-open ¦ No heater ¦ No special options ¦ HIRE-Family ¦ LED ¦ Coupling Type A ¦ 012.765-003 |  |  |  |
| 792 LED Green Elevated |  |  |  |  |  | Threshold ¦ Straight ¦ Photometrics: ICAO ¦ CAT III ¦ Elevated ¦ Side A: Green, no toe-in ¦ Side B: Blank ¦ Glass for elevated lights ¦ Cable: 400mm, one plug ¦ Plug: standard FAA 2-pole plug ¦ With fail-open (relay) ¦ No heater ¦ No special options ¦ 792 ¦ LED ¦ Coupling Type A ¦ NOTE: Old model, elevated light 792 has been superseded by elevated light 797 ¦ 792.LED.3.FO.A |  |  |  |
| Piese de schimb |  |  |  |  |  |  |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | Electronic Housing Group ¦ 20 W / 20 W ¦ 1 cable 250 mm ¦ standard FAA plug ¦ 8 inch ¦ With fail-open (relay) |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | Electronic Housing Group ¦ 40 W ¦ No cable ¦ Heatsink HIRE |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | Electronic Housing Group ¦ 40 W / 40 W ¦ 1 cable 250 mm ¦ standard FAA Plug ¦ 12 inch ¦ With fail-open (relay) |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | LED module ¦ Taxiway Centerline ¦ 8 inch ¦ Green ¦ CAT III ¦ No heating ¦ No special options ¦ With fail-open ¦ 6.6 A |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | LED module ¦ Taxiway Centerline / Intermediate Holding Position ¦ 8 inch ¦ Yellow ¦ CAT III ¦ No heating ¦ No special options ¦ With fail-open ¦ 6.6 A |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | LED module ¦ Taxiway Centerline Curved ¦ 8 inch ¦ Yellow ¦ CAT III ¦ No heating ¦ No special options ¦ With fail-open ¦ 6.6 A |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | LED module ¦ Runway Centerline / Touch Down Zone ¦ 8 inch ¦ Red ¦ CAT III ¦ No heating ¦ No special options ¦ With fail-open ¦ 6.6 A |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | LED module ¦ HIRE ¦ side A ¦ Yellow ¦ CAT III ¦ No fail-open ¦ No special options |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | LED module ¦ HIRE ¦ side B ¦ Yellow ¦ CAT III ¦ No fail-open ¦ No special options |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | LED module ¦ Runway Edge ¦ 12 inch ¦ White ¦ CAT III ¦ No heating ¦ No special options ¦ With fail-open ¦ 6.6 A |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | LED module ¦ Runway Edge ¦ 12 inch ¦ Yellow ¦ CAT III ¦ No heating ¦ No special options ¦ With fail-open ¦ 6.6 A |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | PAPI 801 cod 952210/60-120 Breakable coupling |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | PAPI 801 cod 801202 Ball bearing |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | O-ring gasket 8 inch ¦ for elevated lights 792-Family |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | Cod 792886206 Spare light unit for 921 LED RGL follow ¦ without fail-open ¦ with driver |  |  |  |
| Piese de schimb, analogic sau echivalent cu specificația descrisa |  |  |  |  |  | Cod 792887206 Spare light unit for 921 LED RGL on/off ¦ without fail-open ¦ with driver |  |  |  |
| TOTAL |  |  |  |  |  |  |  |  |  |
| Semnat:_______________ Numele, Prenumele:_____________________________ În calitate de: ________________ Ofertantul: _______________________ Adresa: __________________________ |  |  |  |  |  |  |  |  |  |