|  |  | Technical specifications (form F4.1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | [ This table will be completed by the tenderer in column 4 and by the contracting authority - in columns 1, 2, 3, 5]. |
| Auction number ______ from ___________ |  |  |  |  |  |  |
| Auction name: Acquisition of construction services of the exhibition stand (103.5m²) for the China Food and Drinks Fair in Chengdu, China, to be held March 21-23, 2019. |  |  |  |  |  |  |
| Cod CPV |  | Name of services | Full technical specification required by the contracting authority | Full technical specification proposed by the bidder | Reference standards |  |
| 1 |  | 2 | 3 | 4 | 5 |  |
| 39100000-3 |  | Exhibition stand | Exhibition stand construction: Insular exhibition stand, size 9x11,5m, 103,5 sq.m. 1. Design according to graphical representation attached to the bidding documents. - Colours range according to brand-book “Wine of Moldova”; - Dull colours on the entire stand surface ; 2. Floor: red moquette, closer to red colour used by the brand “Wine of Moldova”; 3. Storage room: Size W 4,0 x L 3,0 x H 6,0m Material used for the manufacture of the storage room: 18mm melamine fibreboard ; Door with lock and key: 2 pcs; The right and left sides of the storage room (4m sides) contain the logotype “Wine of Moldova” with interior lighting and “WINE OF MOLDOVA A LEGEND ALIVE” inscription, and the 3m sides contain the logotype “WINE OF MOLDOVA A LEGEND ALIVE” in Chinese language, with interior lighting and the text written in volumetric black letters. The front side, 3 meters, will contain a 2,5m x 2m LED screen. A refrigerator with at least 4 metallic racks and a minimal volume of 370 litres will be installed in the storage room. A coffee machine with disposable glasses, sugar and stirrer sticks 200 bottles of bottled water (of 0,5 litres) The storage room will have 2 metallic racks for wine bottles, size W 3000 x L 500 x H2400, at least 4 racks, and load capacity of 250 kg each. The lower part of the storage room is luminescent (10 cm); 4. Furniture: a) Exhibitors’ work table: Material – white melamine fibreboard - dimensions : 800 x 550 x 1100 cm - 24 pcs - dimensions : 1600 x 550 x 1100 cm - 1 pcs Each work table will contain on the front side the logotype of the companies applied on white oracal. The front corner of the work table shall contain a glass show case for displaying the wine bottles The interior of the work table shall contain a rack (shelf) of 18mm melamine fibreboard, mounted 25cm down of the upper part of the table. The lower part of the work table is luminescent (10cm) – 28 white bar chairs b) Conversation table: Round glass conversation tables (H70-80cm) – 6 pcs. Chairs (ordinary, not bar chairs) – 24 pcs. 5. The lightning of the stand will white (warm) so that the whole surface of the stand is evenly lightened. The stand must be handed over to the contracting authority on March 20, 2019, in the 12th pavilion 12B006T in Western China International Expo City, New Tianfu District, Chengdu, China. Dismantling the stand will take place on March 24, 2019. |  |  |  |
|  |  | TOTAL |  |  |  |  |

Name of company: ___________________________

Name, surname of authorized person : _______________________

Signature: ____________________________

Stamp