Anexa nr. 1 (fizico-chimie)

|  | Denumirea bunurilor/serviciilor/lucrărilor solicitate | Unitatea de măsură | Canti- tatea | Specificarea tehnică deplină solicitată, Standarde de referință |
| --- | --- | --- | --- | --- |
|  | Alcool etilic 96% | L | 100 | 95.1 % V/V - 96.9 % V/V; bp: about 78 °C; Acidity or alkalinity conform Ph.Eur. Relative density: 0.805 - 0.812; Residue on evaporation: maximum 25 ppm; Ambalaj: 1 L. |
|  | Buffer solution, pH 6,86 | L | 1 | pH 6,86 (25°C). Ambalaj: 0,5 L. |
|  | Buffer solution pH 4.01 | L | 1 | pH 4,01 (25°C). Ambalaj: 0,5 L. |
|  | Buffer solution pH 1.68 | L | 1 | pH 1,68 (25°C). Ambalaj: 1 L. |
|  | (+)-Isomenthone. | mg | 100 | Conform Ph.Eur. A colourless liquid, very slightly soluble in water, soluble in ethanol (96%). density: about 0.904; refractive index: about 1.453; optical activity: about + 93.2. Suitable for Gas chromatografy; Content: minimum 80.0%, calculated by the normalisation procedure. Ambalaj: 100 mg. |
|  | Sabinene | mg | 100 | Conform Ph.Eur.: A colourless, oily liquid, suitable for Gas chromatografy. Content: minimum 95.0%, calculated by the normalisation procedure. Ambalaj: 100mg. |
|  | Refractive index standart kit 1 | buc | 1 | isooctan/apă; testat la 20°C; cu încertitudinea măsurării. |
|  | Refractive index standart kit 2 | buc | 1 | toluen/apă; testat la 20°C; cu încertitudinea măsurării. |
|  | Refractive index standart kit 3 | buc | 2 | 1-Methyl-naphthalene/apă; testat la 20°C; cu încertitudinea măsurării. |
|  | N,N-diethyl-m-toluamide | g | 100 | Reagent Ph. Eur., ≥98.0%; CAS: 134-62-3; densitate: 0.996 g/ml (20 C); indice de refracție: n20/D 1.52; ambalaj: 100 g. |
|  | Phenacetin | g | 2 | CRM for melting point cu incertitudinea. Ambalaj: 2g |
|  | Peroxid de hydrogen 30% | L | 100 | Content: 29.0% - 31.0% m/m; Acidity conform Ph.Eur.; Organic stabilisers: maximum 500 ppm; Non-volatile residue: maximum 2 g/L. Ambalaj: 2.5L. |
|  | Di-Potassium hydrogen phosphate | kg | 1 | 98.0 - 101.0 % (dried substance); Monopotassium phosphate: maximum 2.5%; Chlorides: maximum 200 ppm; Sulfates: maximum 0.1 %; Iron: maximum 10 ppm; Sodium: maximum 0.1 %; Loss on drying: maximum 2.0 per cent. Ambalaj: 1kg. |
|  | Sodium dihydrogen phosphate | kg | 1 | 99.0-101.0%. Ambalaj: 1 kg. |
|  | Trometamol | g | 100 | 99.0-100.5% (dried subst); pH 10.0 to 11.5; Related substances conform Ph.Eur; Chlorides max 100pm; Iron max. 10ppm; Loss on drying max 0.5%; Sulfated ash max 0.1%; Ambalaj: 100g. |
|  | Titrant 5 for KF one component titration | L | 1 | one-component reagent for volumetric Karl Fischer titration; titr-5 mg H₂O/ml. Ambalaj: 1L. |
|  | HEXAN | L | 2 | ≥98,0% (GC), gradul: pentru HPLC; impurități: ≤0,0005% substanțe nevolatile; ≤0,001% acid liber (ca CH3COOH); ≤0,01% apă (prin Karl Fischer);reziduu după evaporare: ≤ 1 mg/l; aciditate:≤ 0.0002 meq/g; alcalinitate: ≤ 0.0002 meq/g; indice de refracție n20/D: 1,375;densitate: 0,66 g/mL (la 20 °C); transmitanță la 210 nm: ≥50%; la 220nm: ≥85%; la 245nm: ≥98%. Temperatura de fierbere: 69 °C; Temperatura de topire: − 94,3 °C;UV absorbție (referință: apă):la 200 nm: Amax: ≤0.70, la 225 nm: Amax: ≤0.10, la 250 nm: Amax: ≤0.01. Ambalaj: 1 L |
|  | - Polyamide 6 | g | 100 | tip ПА6-210/310 sau ПА6-120/321. Ambalaj: 100g. |
|  | Potassium nitrate | g | 25 | 99.0-101.0%; acidity and alcalinity: conform Ph.Eur; reducible substances: conform Ph.Eur; sulfates: max 150ppm; ammonium: max 100ppm; calcium: max 100ppm; iron: max 20ppm; sodium: max 0.1%; loss on drying: max 0.5%; Ambalaj: 100g. |
|  | Perchloric acid | L | 0,5 | reag. Ph. Eur., 70.0-72.0%. Ambalaj: 0.5 L |
|  | Absolute ethanol | L | 2,5 | ≥99.5 % V/V, conform Ph. Eur.;Acidity <0.0005meq/g, Alkalinity <0.0002meq/g,Absorbance at 240nm < 0.40 Absorbance at 250 to 260nm <0.30, Absorbance at 270 to 340nm <0.10. Ambalaj: 2.5L |
|  | Potassium permanganate | g | 100 | reag. Ph. Eur., ≥99%. Ambalaj: 100g. |
|  | Camphene | g | 50 | >90%. Ambalaj: 50g |
|  | Potassium periodate | g | 100 | reag. Ph.Eur. Ambalaj: 100g |
|  | Acetone for HPLC | L | 1 | Purity (GC) > 99.90 % _Water (by Karl Fischer) < 0.5 % , impurities ≤0.0005% non-volatile matter, ≤0.002% free acid (as CH3COOH), transmittance 330 nm, ≥15%, 335 nm, ≥50%, 340 nm, ≥80%, 350 nm, ≥98%. Ambalaj: 1L |
|  | Tartaric acid | g | 50 | 99.5-101.0% (dried subst);Specific optical rotation: + 12.0 to + 12.8 (dried substance).; Oxalic acid: maximum 360 ppm, calculated as anhydrous oxalic acid.; Chlorides: maximum 100 ppm.; Sulfates: maximum 150 ppm.; Calcium: maximum 200 ppm.; Loss on drying: maximum 0.2 per cent,; Sulfated ash: maximum 0.1 per cent. Ambalaj: 50g |
|  | Iodine | g | 50 | Content: 99.5 per cent to 100.5 per cent of I.; Bromides and chlorides: maximum 250 ppm.; Non-volatile substances: maximum 0.1 per cent.. Ambalaj: 50g |
|  | 1-methylhexylamine (2-aminoheptane) | ml | 100 | PURITY (GC AREA %) ≥ 98.5 %, APPEARANCE (COLOR) Colorless, APPEARANCE (FORM) Liquid. Ambalaj: 100ml |
|  | 2,4,6-Trinitrophenol (picric acid) | g | 50 | conform PhEur. Ambalaj:50g |
|  | D imetilsulfoxide for GC | L | 2,5 | conform Ph.Eur, for GC-HeadSpace. Ambalaj: 2.5L |
|  | Methoxyphenylacetic acid | g | 5 | PURITY (TLC AREA %) ≥ 98.5 %, APPEARANCE (COLOR) White to Off White, APPEARANCE (FORM) Powder or Crystal or Chunk(s), TITRATION (T) NAOH 0.1M 98.5 - 101.5 %. Ambalaj: 5g |
|  | Chlorobenzene anhydrous | mL | 100 | PURITY ≥ 99.8%, Appearance (Form) Liquid , Infrared spectrum Conforms to Structure, Purity (GC) > 99.75 % ,Water (by Karl Fischer) < 0.005 % , Residue on Evaporation < 0.0005 %. Ambalaj: 100mL |
|  | Boric acid | g | 100 | >99.5% reag. Ph.Eur., Pb < 0.001%, chlorides<0.0003%, sulphate<0.0005%, phosphate<0.0005%.Ambalaj: 100g |
|  | Benzanilide | g | 100 | >97.5% reag. Ph.Eur. Ambalaj: 100g |
|  | 2-Octyl-1-dodecanol | mL | 25 | Appearance (Form) Liquid, GC (area %) > 96.5 % .Ambalaj: 25mL |
|  | Cesium chloride | g | 50 | conform PhEur. Ambalaj:50g |
|  | Hydrindantin dihydrate | g | 10 | >98%. Ambalaj: 10g |
|  | Ethylene oxide solution | ml | 2 | 50mg/ml in methanol. Ambalaj: 1ml. |
|  | Ethylene oxide solution | ml | 2 | 2mg/ml in dichloromethane. Ambalaj: 1ml. |
|  | Nessler cylinder | buc | 10 | 20ml - Class A - Borosilicate Glass - for Colorimetric Analysis. |
|  | Nessler cylinder | buc | 20 | 50ml - Class A - Borosilicate Glass - for Colorimetric Analysis. |
|  | Adsorber tube with tubing nipple | buc | 5 | Height 155mm; Inner diameter 32mm; SGJ size B-14/15. |
|  | Fused silica tubing | buc | 1 | I.D. 75 µm ± 3 µm / O.D. 280 µm ± 10 µm, 5 meters(LCP# FS-75) |
|  | Fused silica tubing | buc | 1 | I.D. 50 µm ± 3 µm / O.D. 280 µm ± 10 µm, 5 meters(LCP# FS-50) |
|  | microseringa 10 mkL | buc | 2,00 | seringa 10mkL, ac ingustat, fixat, plunjor cu capac PTFE, 23/42/HP, cod: 5181-3354 |
|  | plunjor cu capac PTFE pentru seringa 10mkL | buc | 4,00 | plunjor cu capac PTFE, seringa 10mkL, cod: 5181-3365 |
|  | Termometru p/u frigider din sticla | buc | 20 | p/u frigider, t= -30°C - +30°C diviziunea 0,5 sau 0,1 cu certificat etalonare -20°C - + 5°C |
|  | Spatula-“st. steel”-“micro spoon” | buc | 20 | Material: oțel inoxidabil; cu microlingură; lungimea: 180 mm; lățimea lamei: 10 mm; lungimea lamei: 35 mm; diametrul tijei: 3,5 mm; autoclavabilă (047.06.180) |
|  | Mănuşi nitril marimea L | buc | 500 | Mănuşi de nitril, nepudrate, ambidextre, nesterile, albastre, rezistente la acizi şi baze. Nu conţin cauciuc natural din latex. Manșetă: lungă, rotungită (pentru protecție mai eficientă) |
|  | Сuveta pentru polarimetru P-2000 (Jasco) | buc | 2 | Cyrindrical glass cell 10 mm x 100 mm, Model CG1-100 (1103-1101A) |
|  | Agitator magnetic cylindric lungimea: 20 mm | buc | 5 | Material: PTFE; formă: cilindrică, cu inel central (pentru balone cu fund neregulat); diametrul 6 mm; lungimea: 20 mm |
|  | Pre-column Coupler 1/16 pentru coloana Intersil ODS-3V | buc | 10 | GL Sciences Pre-column Coupler SUS (6010-49210) |
|  | Electrod pH | buc | 1 | ЭСЛ-43-07СР. compatibil cu pH metru И-160М |
|  | Electrod pH de referinta | buc | 1 | ЭВЛ-1М3.1. compatibil cu pH metru И-160М |
|  | Capace pentru vialuri de 2ml (septa and cap) | buc | 2000 | pentru HPLC cu autosampler |
|  | Indicatori chimici pentru sterilizarea cu aburi 121 grade, 15 min. | buc | 1000 | Indicatori chimici pentru sterilizarea cu aburi. Care arată schimbarea culorii la atingerea nivelului de presiune şi temperatură. Conform ISO 11140-1:2014. Clasa a 4-a. Termen minim de valabilitate 2 ani. Cu certificat de conformitate. |

Conducătorul grupului de lucru:           ____________________                          Dragoș GUȚU