Anexa nr.1

| Nr. d/o | Funcționalități | Descrierea specificației solicitate |
| --- | --- | --- |
| 1 | Tip | Server |
|  | Cantitate | 4 buc |
|  | Chassis | Rack mount, 2U, 19” rack mount |
|  | Processor | min. two (2x) processors; |
|  |  | min. 18 core; |
|  |  | min. 36 threads; |
|  |  | min. 2.6 Ghz; |
|  |  | min. cache 25 MB; |
|  |  | min. memory 2933 Mhz. |
|  |  | last generation at the moment |
|  | Memory | min. 512 GB (8 x 64 Gb); |
|  |  | min. DDR4 2933 MHz; |
|  |  | min. 24 slots or more |
|  | RAID controller | Hardware Controller; RAID 0/1/5/10/50/60; min. Cache 4Gb. |
|  | Drive bays | min. 16x 2.5" SAS/SATA (front) with drive bays; |
|  | Hard drives | min. two (2x) 2,5''; min. 400 GB; Enterprise Performance SAS SSD; 12Gbps; hot-plug. |
|  | Cooling system | Redundant hot plug |
|  | Power supply | Redundant hot plug power supplies; min. 900W min. 80 PLUS Platinum; Two (2x) Power cables, PDU connection |
|  | Expansion slots | min. 1x Full PCIe x16; min. 6x Full PCIe x8. |
|  | Network interfaces | Two (2x) - Gigabit Ethernet NIC with four (4x) ports 100/1000 |
|  | Additional cards | Two (2x) - FC HBA cards with min. one FC port, min. 8Gbit |
|  | Ports: | min. 1x front video port; min. 1x back end VGA (15pin/female); min. 1x Management port; min. 8 x RJ-45; min. 2 x USB 2.0; min. 2 x USB 3.0. |
|  | Server management (SM) | Dedicated Ethernet NIC for management. |
|  |  | Local and remote control of system resources (manage, diagnose and monitor). |
|  |  | Remote power control, remote firmware updating, remote presence, capture of OS failure screen (BSOD) via web interface, virtual media attachment to a server of USB drive, CD/DVD drive |
|  |  | Serial over LAN, DHCP, IPMI V2.0, SNMP, email alerts |
|  |  | LDAP user authentication |
|  |  | Browser based administration with full control over the remote host server's display, keyboard, and mouse, including host OS graphical interface. |
|  |  | Virtual media option that provides virtual CD drive and remote image mounting. |
|  |  | SM & server diagnostics logging. |
|  |  | SSL or SSH encryption. |
|  | Rack mounting rails | Rails with full extraction, Cable management arm |
|  | Operating Systems Supported | MS Windows Server 2016/2019, Microsoft Hyper-V Server 2016/2019, SUSE Linux Enterprise Server x86 (64-bit) |
|  | SW-Support | The latest software, drivers, and firmware fully optimized and tested for the server (to be supported on MS Windows Server 2016 and up) and options, to be able to download from websites without costs. |
|  | Installation, cabling and configuration | Including all necessary parts and specialists |
|  | Warranty | 5 years parts/labor/onsite, repair next business-day |