| Nr. d/o | Denumirea | Unitate de măsură | Cantitatea | Specificația |
| --- | --- | --- | --- | --- |
| 1 | 1,4-dioxane | mL | 100 | Conform Ph.Eur. A clear, colourless liquid, miscible with water and with most organic solvents; density: about 1.03; Water: max. 0.5%; Freezing point: minimum 11.0 °C. Ambalaj: 100 mL |
| 2 | 1,2-Dichloroethane | mL | 100 | Conform Ph.Eur. A clear, colourless liquid, soluble in about 120 parts of water and in 2 parts of ethanol (96 per cent).Density: about 1.25. Distillation range: Not less than 95 per cent distils between 82 °C and 84 °C. Ambalaj: 100 mL |
| 3 | 1-Propanol | L | 1 | for HPLC; ≥99,9%; Water:≤0,05%; Clear colourless liquid, miscible with water and with ethanol (96%); density: 0.802-0.806; bp: about 97.2 °C. Distillation range: Not less than 95% distils between 96°C and 99°C; Ambalaj: 1L. |
| 4 | Ammonium dihydrogen phosphate | g | 500 | Conform Ph.Eur. A white or almost white, crystalline powder or colourless crystals, freely soluble in water. pH : about 4.2 for a 23 g/L solution. Ambalaj: 500g. |
| 5 | Ammonium oxalate monohydrate | g | 250 | Conform Ph.Eur. Ambalaj: 250g |
| 6 | Benzene | L | 0.5 | Conform Ph.Eur, Assay 99.0%, Ambalaj: 0,5 L |
| 7 | Citric acid monohydrate | kg | 0.5 | Conform Ph.Eur, assay 99.5-100.5% (calc. on anhydrous substance); oxalic acid: max.360ppm; sulfati: max.150ppm; Water:7,5-9,0%; Sulfated ash: maximum 0.1%. Ambalaj: 0,5 kg |
| 8 | Cobalt(II) chloride hexahydrate | g | 50 | conform Ph.Eur. Ambalaj: 50 g |
| 9 | di-Ammonium hydrogen phosphate | g | 100 | conform Ph.Eur.; pH: about 8 for a 200 g/L solution. Ambalaj: 100 g |
| 10 | Diethyl ether | L | 1 | Conform Ph.Eur a clear, colourless, volatile liquid, soluble in water, miscible with ethanol (96%), with methylene chloride and with fatty oils, assay ≥99.7% (GC), Relative density: 0.714 to 0.716, Water : maximum 2 g/L. Ambalaj: 1 L |
| 11 | DIETHYLENE GLYCOL | mL | 25 | Conform Ph.Eur a clear, colourless liquid, hygroscopic, miscible with water, with acetone and with ethanol 96%; assay: min.99.5% m/m; density: 1.118; refractive index: 1.447. Ambalaj: 25 mL |
| 12 | 2-(1-methylethoxy)propane (Diisopropyl ether) | L | 0.5 | Clear, colourless liquid, very slightly soluble in water, miscible with ethanol (96 per cent); density: 0.723 to 0.728; bp: 67 °C to 69 °C. Ambalaj: 0.5L. |
| 13 | Diphenylamine | g | 25 | Conform Ph.Eur a white or almost white crystals, slightly soluble in water, soluble in ethanol (96 per cent). mp: about 55 °C. Ambalaj: 25g. |
| 14 | Ethyl acetate | L | 1 | Conform Ph.Eur a clear, colourless liquid, soluble in water, miscible with ethanol (96 per cent), assay ≥99.5% (GC); Relative density: 0.898 to 0.902; Refractive index: 1.370 to 1.373; Related substances conform Ph.Eur.; Residue on evaporation: maximum 30 ppm.; Water: max. 0.1%. Ambalaj: 1 L |
| 15 | Hexamethylenetetramine | g | 25 | Conform Ph.Eur a colourless, crystalline powder, very soluble in water. Ambalaj: 25 g |
| 16 | Iron(III) chloride hexahydrate | g | 50 | Conform Ph.Eur., 98.0-102.0% .Crystalline mass or orange-yellow or brownish-yellow crystals, very hygroscopic; Sulfates: maximum 100 ppm; Ferrous ions: max. 50 ppm; Ambalaj: 50 g |
| 17 | Methanol HPLC | L | 70 | ≥99,9%; Reag. Ph. Eur.; gradul gradient; corespunde cu Methanol R1 și R2 în conformitate cu Reag. Ph. Eur.; filtrat prin filtru de 0,2 μm; corespunde pentru utilizarea la echipamente UPLC/ UHPLC/ Ultra HPLC; colorație: ≤ 10 Hazen; apă: ≤ 0,02%;reziduu după evaporare: ≤ 2,0 mg/l; aciditate: ≤ 0,0002 meq/g; alcalinitate: ≤ 0,0002 meq/g; densitate: 0,791 – 0,793 (la 20 °C); temperatura de fierbere: 64 – 65 °C; Gradul gradient la 235 nm: ≤ 2,0 mAU, la 254 nm: ≤ 1,0 mAH; Absorbanță: ≤ 0,170 (la 225 nm); Transmisie : la 210 nm: ≥ 20%, la 220 nm: ≥ 60%, la 230 nm: ≥ 75%,- la 235 nm: ≥ 83%, la 250 nm: ≥ 95%,- la ≥ 260 nm: 98%; Fluorescență (ca chinină la 254 nm): ≤ 1,0 ppb, ≤ 0,5 ppb (ca chinină la 365 nm);Ambalaj: 2,5 litr. |
| 18 | Potassium hexahydroxoantimonate | g | 25 | Conform Ph.Eur, White or almost white, crystals or crystalline powder, sparingly soluble in water. Ambalaj: 25 g |
| 19 | Potassium permanganate | g | 100 | Conform Ph.Eur; Content: 99.0 per cent to 100.5 per cent.Substances insoluble in water: maximum 1.0 per cent.; Chlorides: maximum 200 ppm; Sulfates (2.4.13): maximum 500 ppm. Ambalaj: 100 g |
| 20 | Sodium carbonate anhydrous | kg | 1 | White or almost white powder, hygroscopic, freely soluble in water. When heated to about 300 °C it loses not more than 1 per cent of its mass. Ambalaj: 1 kg |
| 21 | Sodium dodecyl sulfate | g | 500 | Conform Ph.Eur, assay ≥99.0% (GC), Non-esterified alcohols: maximum 4.0 per cent; Sodium chloride and sodium sulfate: maximum 8.0 per cent for the total percentage content. Ambalaj: 500 g |
| 22 | Sodium sulfate | kg | 0.5 | Conform Ph.Eur.:Anhydrous disodium sulfate.Content: 98.5 per cent to 101.0 per cent (dried substance).Chlorides: maximum 450 ppm.Calcium: maximum 450 ppm.Iron : maximum 90 ppm.Magnesium: maximum 200 ppm.Loss on drying: maximum 0.5 per cent, determined on 1.000 g Ambalaj 0.5 kg |
| 23 | Sodium thiosulfate pentahydrate | g | 250 | Conform Ph.Eur. Assay 99.0-101.0%. pH: 6.0-8.4; Sulfates, sulfites: max.0.2%; Sulfides: conform Ph.Eur. Ambalaj: 250 g |
| 24 | Triethanolamine | mL | 100 | Content: 99.0 per cent m/m to 103.0 per cent m/m of total bases (anhydrous substance); Related substances conform Ph.Eur; Water (2.5.12): maximum 1.0 per cent, determined on 1.000 g.; Sulfated ash (2.4.14): maximum 0.1 per cent, determined on 1.0 g. Ambalaj: 100 ml |
| 25 | (−)-Menthone | mL | 25 | CAS: 14073-97-3. Colourless liquid, very slightly soluble in water, very soluble in ethanol (96 per cent); density: about 0.897; refractive index about 1.450; Content: minimum 90.0 per cent, calculated by the normalisation procedure. Ambalaj: 25 mL |
| 26 | (−)-β-Pinene (CAS: 18172-67-3) | mL | 5 | assay ≥95%, Ambalaj: 5 mL |
| 27 | (R)-(−)-α-Phellandrene (CAS: 4221-98-1) | ml | 1 | refractive index about 1.471; Content: minimum 95.0 per cent, calculated by the normalisation procedure. Ambalaj: 1 mL |
| 28 | 1-Fluoro-2,4-dinitrobenzene | g | 25 | Pale yellow liquid or crystals, soluble in propylene glycol; mp: about 29 °C.; Content: minimum 99.0 per cent, determined by gas chromatography. Ambalaj: 25 g |
| 29 | (+)-Menthofuran | ml | 1 | Conform Ph.Eur.: Slightly bluish liquid, very slightly soluble in water, soluble in ethanol (96%). density: about 0.965; refractive index: about 1.480; optical activity: about + 93; bp: 196 °C. Suitable for Gas chromatografy; Content: minimum 97.0 per cent, calculated by the normalisation procedure. Ambalaj: 1ml. |
| 30 | Isoniazid | g | 5 | Content: 99.0 per cent to 101.0 per cent (dried substance).; pH (2.2.3): 6.0 to 8.0 for solution S; Related substances conform Ph.Eur.; Loss on drying: max 0.5%, determined on 1.000 g by drying in an oven at 105 °C; Sulfated ash:max.0.1%, determined on 1.0 g.Ambalaj 5g |
| 31 | THIOACETAMIDE | g | 25 | Conform Ph.Eur. Crystalline powder or colourless crystals, freely soluble in water and in ethanol (96 per cent); mp: about 113 °C; Ambalaj 25 g |
| 32 | Neryl acetate | ml | 1 | Conform Ph.Eur a colourless, oily liquid, assay ≥93.0% (GC), density 0.907 g/mL at 20 °C. Ambalaj: 1 mL |
| 33 | Sodium sulfide nonahydrate | g | 100 | Conform Ph.Eur. Ambalaj: 100 g |
| 34 | (R)-(+)-Limonene | ml | 5 | Conform Ph.Eur Colourless liquid, practically insoluble in water, soluble in ethanol (96 per cent) ,assay ≥99.0%; density: about 0.84; refractive index: about 1.471-1,474; optical activity: about + 124. Ambalaj: 5ml |
| 35 | α-Terpineol | ml | 1 | Conform Ph.Eur; Colourless crystals, practically insoluble in water, soluble in ethanol (96 per cent). ,assay ≥97.0%; density: about 0.935; refractive index: about 1.483; optical activity: about + 92,5. Ambalaj: 1ml |
| 36 | Anisaldehyde | ml | 5 | Conform Ph.Eur;, Oily liquid, very slightly soluble in water, miscible with ethanol (96 per cent). bp: about 248 °C.; Content: minimum 99.0 per cent, calculated by the normalisation procedure. Ambalaj: 5ml |
| 37 | Citral (cis+trans) | ml | 5 | Conform Ph.Eur; Light yellow liquid, practically insoluble in water, miscible with ethanol (96 per cent) and with propylene glycol; Content of citral (neral + geranial): minimum 95.0 per cent, calculated by the normalisation procedure.Ambalaj: 5ml |
| 38 | (+/-)-Linalool | ml | 5 | Conform Ph.Eur; Mixture of two stereoisomers (licareol and coriandrol); density: about 0.860; refractive index: about 1.462; Content: minimum 98.0 per cent, calculated by the normalisation procedure. Ambalaj: 5ml |
| 39 | γ-Terpinene | ml | 5 | Conform Ph.Eur; Oily liquid; Content: minimum 93.0 per cent, calculated by the normalisation procedure. Ambalaj: 5ml |
| 40 | Methyl Orange | g | 25 g | Orange-yellow, crystalline powder, slightly soluble in water, practically insoluble in ethanol (96 per cent).; Schultz No. 176.Colour Index No. 13025. Ambalaj: 25g |
| 41 | Murexid | g | 5 | Conform Ph.Eur Brownish-red crystalline powder, sparingly soluble in cold water, soluble in hot water, practically insoluble in ethanol (96 per cent), soluble in solutions of potassium hydroxide or sodium hydroxide giving a blue colour. Ambalaj: 5g |
| 42 | Metanil yellow | g | 10 | Conform Ph.Eur a brownish-yellow powder, soluble in water and in ethanol (96 per cent), Colour Index No. 13065. Schultz No. 169. Ambalaj: 10g |
| 43 | Mordant blue 13 | g | 25 | chromic dark blue; CAS:1058-92-0; Ambalaj: 25g |
| 44 | Dithizone | g | 10 | Conform Ph.Eur A bluish-black, brownish-black or black powder, practically insoluble in water, soluble in ethanol (96 per cent). Ambalaj: 10g |
| 45 | Bromocresol green | g | 5 | Conform Ph.Eur; Brownish-white powder, slightly soluble in water, soluble in ethanol (96 per cent) and in dilute solutions of alkali hydroxides. Ambalaj: 5g |
| 46 | Tropaeolin OO | g | 25 | pH 1.2–3.2 violet-red/yellow; Ambalaj: 25 g |
| 47 | Eosin Y | g | 25 | indicator; CAS:17372-87-1; Ambalaj: 25g |
| 48 | Alizarin S. | g | 25 | Conform Ph.Eur an orange-yellow powder, freely soluble in water and in ethanol (96 per cent), Colour Index No. 58005, Schultz No. 1145. Ambalaj 25g |
| 49 | Methyl red | g | 25 | Conform Ph.Eur a dark-red powder or violet crystals, practically insoluble in water, soluble in ethanol (96 per cent); Schultz No. 250; Colour Index No. 13020; Ambalaj 25g |
| 50 | Methyl violet | g | 25 | Powder or crystals, pH 1-2, solubility: ethanol and water: 0.01 g/10 mL. Ambalaj 25g |
| 51 | Bromophenol blue | g | 5 | Conform Ph.Eur a light orange-yellow powder, very slightly soluble in water, slightly soluble in ethanol (96 per cent), freely soluble in solutions of alkali hydroxides. Ambalaj 5g |
| 52 | Bromocresol purple | g | 5 | Conform Ph.Eur a pinkish powder, practically insoluble in water, soluble in ethanol (96 per cent) and in dilute solutions of alkali hydroxides.Ambalaj: 5g |
| 53 | Bromophenol red | g | 5 | solid, impurities ≤5% Water (Karl Fischer), visual transition interval 4.7-6.3, yellow to purple, mp 230 °C (decomposition). Ambalaj: 5g |
| 54 | Congo red | g | 25 | Conform Ph.Eur a brownish-red powder, soluble in water, colour Index No. 22120. Ambalaj: 25g |
| 55 | Cresol red | g | 5 | Conform Ph.Eur a reddish-brown crystalline powder, slightly soluble in water, soluble in ethanol (96 per cent) and in dilute solutions of alkali hydroxides. CAS: 1733-12-6. Ambalaj: 5g |
| 56 | Methylene blue | g | 25 | Conform Ph.Eur a dark-green or bronze, crystalline powder, freely soluble in water, soluble in ethanol (96 per cent). Schultz No. 1038; Colour Index No. 52015. Ambalaj: 25g |
| 57 | phenolphthalein | g | 25 | Conform Ph.Eur a white or yellowish-white powder, practically insoluble in water, soluble in ethanol (96 per cent); assay: 98,0-101,0% (dried substance); Related substances (TLC): conform Ph.Eur; Chlorides max 100ppm; Sulfates: max 200ppm; Loss on drying: Not more than 0.5%; Sulfated ash: Not more than 0.1%. Ambalaj: 25g |
| 58 | 1,10-Phenanthroline chloride monohydrate | g | 10 | Conform Ph.Eur, White or almost white, crystalline powder, freely soluble in water, soluble in ethanol (96 per cent). mp: about 215 °C, with decomposition. Ambalaj: 10g |
| 59 | Copper (II) acetate monohydrate | g | 100 | Conform Ph.Eur: Blue-green crystals or powder, freely soluble in boiling water, soluble in water and in ethanol (96 per cent), slightly soluble in glycerol (85 per cent). Ambalaj 100g |
| 60 | Dichloroacetic acid | ml | 100 | Conform Ph.Eur.; Colourless liquid, miscible with water and ethanol (96 per cent). Density: about 1.566; Refractive index: about 1.466.; bp: about 193 °C. Ambalaj: 100ml |
| 61 | Tartaric acid | g | 100 | Conform Ph.Eur.. Ambalaj: 100g |
| 62 | Ethanol 96% | L | 100 | 95.1 % V/V - 96.9 % V/V; bp: about 78 °C; Acidity or alkalinity conform Ph.Eur. Relative density: 0.805 - 0.812; Residue on evaporation: maximum 25 ppm; Ambalaj: 1 L. |
| 63 | (-)-Borneol | g | 25 | Conform Ph.Eur,; Colourless crystals, readily sublimes, practically insoluble in water, freely soluble in ethanol (96 per cent) and in light petroleum.; mp: about 208 °C. Ambalaj 25 g |
| 64 | Bornyl acetat (CAS:5655-61-8) | ml | 1 | Conform Ph.Eur, Colourless crystals or a colourless liquid, very slightly soluble in water, soluble in ethanol (96 per cent); Suitable for GC. Ambalaj 1 ml |
| 65 | Bovine serum albumin | g | 10 | Bovine serum albumin containing about 96% of protein; White to light brownish-yellow powder; Water: max 3.0%, determined on 0.800 g. Ambalaj 10 g |
| 66 | Bromothymol blue | g | 5 | Conform Ph.Eur; Reddish-pink or brownish powder, practically insoluble in water, soluble in ethanol (96 per cent) and in dilute solutions of alkali hydroxides.Ambalaj 5 g |
| 67 | Chloroform | L | 5 | conform Ph.Eur. Ambalaj: 2,5L. |
| 68 | Copper sulfate pentahydrate | g | 250 | Conform Ph.Eur.:Content: 99.0 per cent to 101.0 per cent. Appearance: blue, crystalline powder or transparent, blue crystals.Solubility: freely soluble in water, soluble in methanol, practically insoluble in ethanol (96 per cent).Chlorides: maximum 100 ppm.Iron: maximum 100 ppm.Elemental impurities- conform Ph.Eur. Loss on drying: 35.0 per cent to 36.5 per cent. Ambalaj 250 g |
| 69 | Dimethyl sulfoxide | L | 2.5 | Conform Ph.Eur; ≥99.7 %. Ambalaj 2.5L |
| 70 | Disodium edetate dihydrate | g | 200 | Conform Ph.Eur; Content: 98.5 per cent to 101.0 per cent; pH: 4.0 to 5.5; Impurity (nitrilotriacetic acid) max.0.1%; Iron: maximum 100 ppm. Ambalaj 100 g |
| 71 | di-Potassium hydrogen phosphate trihydrate | g | 250 | Conform Ph.Eur; suitable for HPLC mobile phase. Ambalaj 250 g |
| 72 | di-Potassium hydrogen phosphate trihydrate | g | 250 | Conform Ph.Eur.: Colourless or white or almost white powder or crystals, freely soluble in water. Ambalaj 250 g |
| 73 | Ethanol absolute | L | 2.5 | ≥99.5 % V/V, conform Ph. Eur.;Acidity <0.0005meq/g, Alkalinity <0.0002meq/g,Absorbance at 240nm < 0.40 Absorbance at 250 to 260nm <0.30, Absorbance at 270 to 340nm <0.10. Ambalaj: 2.5L |
| 74 | Solvent Karl Fischer two component | L | 20 | solvent for volumetric Karl Fischer titration with two component reagents. Ambalaj: 2.5L. |
| 75 | Potassium sulfate | g | 100 | Conform Ph.Eur.; Content: 98.5% to 101.0% of K2SO4 (dried substance); Chlorides: maximum 40 ppm; Calcium: maximum 200 ppm.;Iron: maximum 10 ppm; Magnesium: max 20 ppm.; Sodium: max0.10%.Loss on drying: max 1.0%. Ambalaj 100 g |
| 76 | Sulfuric acid | L | 1 | purity: 95.0 % m/m to 97.0 % m/m; Chlorides: max. 0.5 ppm; Nitrates: max. 0.5 ppm; Ammonium: max.2 ppm; Arsenic: max. 0.02 ppm; Iron: max. 1 ppm; Heavy metals: max. 2 ppm; Residue on ignition: max. 0.001%. Ambalaj: 1L. |
| 77 | Titrant Karl Fischer one component | L | 1 | one-component reagent for volumetric Karl Fischer titration; titr-5 mg H₂O/ml. Ambalaj: 1L. |
| 78 | Silver nitrate | g | 200 | 99,0-100,5%; white or almost white, crystalline powder or transparent, colourless crystals; very soluble in water, soluble in ethanol (96 per cent); Foreign salts: max. 0.3%; Aluminium, lead, copper and bismuth- comply Ph.Eur.; Storage: in a non-metallic container, protected from light. Ambalaj:100g. |
| 79 | refractive index standart kit 3 | buc | 2 | 1-Methyl-naphthalene/apă; testat la 20°C; cu încertitudinea măsurării. |
| 80 | camphene | g | 50 | >90%. Ambalaj: 50g |
| 81 | picric acid (2,4,6-trinitrophenol) | g | 50 | conform PhEur. Ambalaj:50g |
| 82 | 2-octyl-1-dodecanol | mL | 25 | Appearance (Form) Liquid, GC (area %) > 96.5 %. Ambalaj: 25mL |
| 83 | cesium chloride | g | 50 | conform PhEur. Ambalaj:50g |
| 84 | hydrindantin dihydrate | g | 10 | >98%. Ambalaj: 10g |
| 85 | ethylene oxide solution | ml | 3 | 50mg/ml in methanol. Ambalaj: 1ml. |
| 86 | ethylene oxide solution | ml | 2 | 2mg/ml in dichloromethane. Ambalaj: 1ml. |
| 87 | tetraheptylammonium bromide | g | 10 | suitable for ion pair chromatography. Ambalaj: 10g |
| 88 | 1.8-cineole | ml | 5 | min.98,0%. Ambalaj: 5ml |
| 89 | isooctane | L | 1 | conform Ph.Eur. Ambalaj: 1 L. |
| 90 | Calconcarboxylic acid | g | 5 | Brownish-black powder, slightly soluble in water, very slightly soluble in acetone and in ethanol (96 per cent), sparingly soluble in dilute solutions of sodium hydroxide. Ambalaj: 5g |
| 91 | trans-anethole (CAS-4180-23-8) | ml | 25 | ≥99%. Amblaj: 25ml |
| 92 | barium chloride dihydrate | g | 100 | Conform Ph.Eur. Amblaj: 100 g |
| 93 | Boron Trifluoride Methanol Solution | ml | 100 | 14% solution of BF3 in methanol. Ambalaj: 100ml. |
| 94 | CALCIUM CHLORIDE dihydrate | g | 250 | Reag. Ph Eur.; 97.0 - 103.0 %; Sulfate (SO4): ≤ 300ppm; Fe (Iron) ≤ 10ppm; Magneziu și metale alcaline: ≤ 0.5 %; Ambalaj: 250 g. |
| 95 | potassium carbonate | g | 100 | Conform Ph.Eur.; Content: 99.0% to 101.0% (dried substance); Chlorides: maximum 100 ppm; Sulfates: maximum 100 ppm; Calcium: maximum 100 ppm; Iron: maximum 10 ppm; Ambalaj: 100 g. |
| 96 | (-)-fenchone (CAS: 7787-20-4) | ml | 5 | Oily liquid, miscible with ethanol (96 per cent), practically insoluble in water; refractive index: about 1.46; content: ≥98.0%.Ambalaj: 5 ml. |
| 97 | formamide | L | 0.25 | Purity: ≥99,5%; water ≤0.1%; Clear, colourless, oily liquid, hygroscopic, miscible with water and with ethanol (96%); Density: about 1.134; bp: about 210 °C; Ambalaj: 0,25L. |
| 98 | formic acid | L | 1 | conform Ph.Eur;; 98.0 % m/m to 100.5% m/m; Chlorides: maximum 10 ppm; Sulfates: maximum 50 ppm.; Sulfites: maximum 300 ppm.; Residue on evaporation: maximum 0.01%. Ambalaj: 1 L. |
| 99 | Geraniol (CAS: 106-24-1) | g | 25 | ≥98%; Ambalaj: 25g. |
| 100 | Geranyl acetate | ml | 1 | ≥98.0%; Ambalaj: 1ml. |
| 101 | glycerol | L | 1 | Content: 98.0 % - 101.0 % m/m; Relative density: 1.258 to 1.268; Acidity or alkalinity conform Ph.Eur. Refractive index: 1.470 to 1.475; Aldehydes: maximum 10 ppm; Esters conform Ph.Eur. Halogenated compounds: maximum 35 ppm; Sugars conform Ph.Eur.; Chlorides: maximum 10 ppm; Water: maximum 2.0 %; Sulfated ash: maximum 0.01%; Ambalaj: 1L. |
| 102 | Sodium hexanitrocobaltate(III) | g | 25 | conform Ph.Eur; Orange-yellow powder, freely soluble in water, slightly soluble in ethanol (96 per cent). Ambalaj 25 g |
| 103 | POTASSIUM HYDROXIDE | kg | 1 | Conform Ph.Eur.: Content: 85.0 per cent to 100.5 per cent of total alkali, calculated as KOH.Appearance: white or almost white, crystalline, hard masses, supplied as sticks, pellets or irregularly shaped pieces, deliquescent, absorbing carbon dioxide.Solubility: very soluble in water, freely soluble in ethanol (96 per cent).Carbonates: maximum 2.0 per cent. Chlorides: maximum 200 ppm.Phosphates: maximum 100 ppm.Sulfates: maximum 200 ppm.Aluminium: maximum 0.2 ppm.Iron: maximum 10 ppm.Sodium: maximum 1.0 per cent. Ambalaj 1 kg |
| 104 | Hydroxylamine hydrochloride | g | 50 | Conform Ph.Eur.: White or almost white, crystalline powder, very soluble in water, soluble in ethanol (96 per cent). Ambalaj 50 g |
| 105 | potassium iodate | g | 100 | Conform Ph.Eur; White or almost white, crystalline powder, soluble in water. Ambalaj: 100 g. |
| 106 | (+)-Isomenthol (CAS: 23283-97-8) | g | 1 | Colourless crystals, practically insoluble in water, very soluble in ethanol (96%). Optical rotation: aboout + 24, determined on a 100 g/L solution in ethanol (96%).; bp: about 218 °C; mp: about 80 °C. Ambalaj 1 g |
| 107 | Linalyl acetate (CAS: 115-95-7) | ml | 5 | min.95.0%; Colourless or slightly yellow liquid with a strong odour of bergamot and lavender. density: 0.895 to 0.912; Refractive index: 1.448 to 1.451; bp: about 215 °C. Ambalaj 5 ml |
| 108 | Menthyl acetate (CAS: 2623-23-6) | ml | 10 | conform Ph.Eur; Colourless liquid, slightly soluble in water, miscible with ethanol (96%); density: about 0.92; refractive index: about 1.447; bp: about 228 °C; assay: min.97.0%. Ambalaj: 10ml. |
| 109 | Ammonium molybdate tetrahydrate | g | 25 | conform Ph.Eur. Ambalaj: 25g. |
| 110 | Iodine monochloride | g | 5 | conform Ph.Eur; Black crystals, soluble in water, in acetic acid and in ethanol (96%); bp: about 97.4 °C. Ambalaj: 5g. |
| 111 | N,N-dimethylaniline | ml | 25 | Clear, oily liquid, almost colourless when freshly distilled, darkening on storage to reddish-brown, practically insoluble in water, freely soluble in ethanol (96 per cent); Refractive index: 1.558; Distillation range: Not less than 95% distils between 192°C and 194°C. Ambalaj: 25ml. |
| 112 | Nessler's reagent | ml | 500 | for determination of ammonia and ammonium compounds. Ambalaj: 500ml. |
| 113 | Ninhydrin | g | 10 | conform Ph.Eur.; White or very pale yellow, crystalline powder, soluble in water and in ethanol (96%). Ambalaj: 10g |
| 114 | Lead (II) nitrate | g | 100 | conform Ph.Eur. Ambalaj: 100g |
| 115 | Oxalic acid dihydrate | g | 50 | conform Ph.Eur; White or almost white crystals, soluble in water, freely soluble in ethanol (96%); Ambalaj: 50g |
| 116 | Sodum perchlorate monohydrate | g | 100 | conform Ph.Eur; Content: min 99.0% of NaClO4,H2O; White or almost white, deliquescent crystals, very soluble in water. Ambalaj: 100g |
| 117 | Phthaldialdehyde | g | 5 | conform Ph.Eur; Yellow, crystalline powder; mp: about 55 °C. Ambalaj: 5g |
| 118 | (+)-α-Pinene (CAS: 7785-70-8) | g | 5 | Content: min 99.0%; Liquid not miscible with water; density: about 0.859; refractive index: about 1.466; bp: 154 °C to 156 °C. Ambalaj: 5g |
| 119 | Polysorbate 20 | g | 100 | Conform Ph.Eur:Appearance: oily, yellow or brownish-yellow, clear or slightly opalescent liquid. Solubility: soluble in water, in anhydrous ethanol, in ethyl acetate and in methanol, practically insoluble in fatty oils and in liquid praffin.Relative density: about 1.10.Viscosity: about 400 mPa·s at 25 °C.Acid value: maximum 2.0.Hydroxyl value: 96 to 108.Peroxide value: maximum 10.0.Saponification value: 40 to 50, determined on 4.0 g.Composition of the fatty-acid fraction of the substance: conform Ph.Eur. Ethylene oxide and dioxan: maximum 1 ppm of ethylene oxide and 10 ppm of dioxan.Water: maximum 3.0 per cent, determined on 1.00 g.Total ash: maximum 0.25 per cent, determined on 2.0 g. Ambalaj 100 g |
| 120 | Potassium hexacyanoferrate(III) | g | 10 | conform Ph.Eur. Ambalaj: 5 g |
| 121 | Potassium thiocyanate | g | 10 | conform Ph.Eur; Colourless crystals, deliquescent, very soluble in water and in ethanol (96%). Ambalaj: 10 g |
| 122 | Sodium nitroprusside dihydrate | g | 5 | Content: 99.0%-100.5% (anhydrous substance); Chlorides: max 200 ppm; Insoluble matter: max 100 ppm; Ambalaj: 5g |
| 123 | Starch, soluble | g | 100 | Ambalaj: 100g |
| 124 | Iron(II) sulphate heptahydrate | g | 10 | Content: 98.0% to 105.0%; Chlorides): maximum 200 ppm.; Ferric ions: maximum 0.3%; Manganese: maximum 0.1%; Zinc: maximum 50 ppm. Ambalaj: 5g |
| 125 | Tannic acid | g | 5 | Yellowish or light-brown, glistening scales or amorphous powder, very soluble in water, freely soluble in ethanol (96 per cent), soluble in acetone. Ambalaj: 5g |
| 126 | Ammonium thiocyanate | g | 100 | conform Ph.Eur; Colourless crystals, deliquescent, very soluble in water, soluble in ethanol (96 per cent). Ambalaj: 100g |
| 127 | tri-sodium citrate dihydrate | g | 100 | Content: 99.0 per cent to 101.0 per cent (anhydrous substance).; Readily carbonisable substances-conform PhEur; Chlorides: maximum 50 ppm; Oxalates: maximum 300 ppm.; Sulfates: maximum 150 ppm.; Water: 11.0% to 13.0%. Ambalaj: 100g |
| 128 | Sodium tungstate dihydrate | g | 25 | White or almost white, crystalline powder or colourless crystals, freely soluble in water forming a clear solution, practically insoluble in ethanol (96%). Ambalaj: 25g |
| 129 | 2-aminoethanol | ml | 25 | Content: 99.0 per cent m/m to 101.0 per cent m/m (anhydrous substance); Water: max 0.25%. Ambalaj: 25 ml |
| 130 | Dichlorophenolindophenol, sodium salt. | g | 5 | conform Ph.Eur; Dark-green powder, freely soluble in water and in anhydrous ethanol. The aqueous solution is dark blue; when acidified it becomes pink. Ambalaj: 5g |
| 131 | 2,6-Dichloroquinone-4-chloroimide | g | 10 | Pale yellow or greenish-yellow crystalline powder, practically insoluble in water, soluble in ethanol (96 per cent) and in dilute alkaline solutions; mp: about 66 °C. Ambalaj: 10g |
| 132 | Lead(II) acetate trihydrate | g | 25 | reag. Ph. Eur.; Colourless crystals, efflorescent, freely soluble in water, soluble in ethanol (96 per cent). Ambalaj: 25g |
| 133 | perchloric acid | L | 0.5 | reag. Ph. Eur., 70.0-72.0%. Ambalaj: 0.5 L |
| 134 | acetaldehyde | ml | 10 | Clear, colourless flammable liquid, miscible with water and with ethanol (96 per cent).; density: about 0.788; Refractive index: about 1.332; bp: about 21 °C. Ambalaj: 10 ml |
| 135 | (−)-Isopulegol | ml | 1 | conform Ph.Eur., min.99%, for GC. Ambalaj: 1 ml |
| 136 | Levomenthol (CAS:2216-51-5) | g | 1 | conform Ph.Eur., min.98.0%, for GC. Ambalaj: 1 g |
| 137 | (+)-Pulegone | ml | 5 | conform Ph.Eur., min.98,0%, for GC. Ambalaj: 5 ml |
| 138 | (+)-Carvone | ml | 1 | conform Ph.Eur., min.98,0%, for GC. Ambalaj: 1 ml |
| 139 | β-Caryophyllene (CAS: 87-44-5) | ml | 1 | conform Ph.Eur., min.90,0%, for GC. Ambalaj: 1 ml |
| 140 | trans-nerolidol (CAS:40716-66-3) | g | 5 | conform Ph.Eur., min.90,0%, for GC. Ambalaj: 5 g |
| 141 | methyl anthranilate | g | 50 | conform Ph.Eur., min.95,0%, for GC. Ambalaj: 50 g |
| 142 | E,E- farnesole (CAS:106-28-5) | g | 1 | conform Ph.Eur., for GC. Ambalaj: 1 g |
| 143 | Lavender oil for peak identification | ml | 0,5 | conform Ph.Eur., for GC. Ambalaj: 0.5 ml |
| 144 | camphor racemic | g | 50 | conform Ph.Eur., min.95,0%. Ambalaj: 50g |
| 145 | cetyl alcohol CRS | mg | 300 | European Pharmacopoeia (EP) Reference Standard. |
| 146 | stearyl alcohol CRS | mg | 250 | European Pharmacopoeia (EP) Reference Standard |
| 147 | Methyl 2-methoxybenzoate | g | 25 | conform Ph.Eur. Ambalaj: 25g |
| 148 | Methyl 4-methoxybenzoate | g | 25 | conform Ph.Eur. Ambalaj: 25g |
| 149 | methyl laurate | g | 5 | 98% GC. Ambalaj: 5g |
| 150 | methyl miristate | g | 1 | 98% GC. Ambalaj: 1g |
| 151 | methyl palmitate | g | 1 | 98% GC. Ambalaj: 1g |
| 152 | methyl stearate | g | 1 | 98% GC. Ambalaj: 1g |
| 153 | Methyl arachidate | g | 1 | 98% GC. Ambalaj: 1g |
| 154 | methyl oleate | g | 5 | 98% GC. Ambalaj: 5g |
| 155 | methyl caproate | ml | 1 | for GC. Ambalaj: 1ml |
| 156 | methyl caprylate | ml | 5 | for GC. Ambalaj: 5ml |
| 157 | methyl decanoate | ml | 1 | 99.0% GC. Ambalaj: 1ml |
| 158 | Methyl eicosenoate | mg | 25 | for GC. Ambalaj: 25mg |
| 159 | Methyl behenate | g | 1 | for GC. Ambalaj: 1g |
| 160 | Methyl Palmitoleate | mg | 100 | for GC. Ambalaj: 100mg |
| 161 | Methyl linoleate | g | 1 | for GC. Ambalaj: 1g |
| 162 | Methyl linolenate | mg | 100 | for GC. Ambalaj: 100mg |
| 163 | Methyl tetracosanoate | mg | 100 | for GC. Ambalaj: 100mg |
| 164 | Sodium hexanesulfonate monohydrate | g | 50 | for ion-pair chromatography; Conform Ph.Eur. White or almost white powder; Content: minimum 99.0 per cent. Ambalaj: 50 g |
| 165 | Sodium sulfite | g | 50 | Conform Ph.Eur. White or almost white powder; Thiosulfates: max 0.1 per cent; Iron: max 10 ppm; Zinc: max 25 ppm; Assay: 95.0-100.5%; Ambalaj: 25g |
| 166 | Acetonitrile | L | 250 | 99,9%; Reag. Ph. Eur.; gradul gradient;corespunde cu Acetonitril pentru cromatografie și Acetonitril R1 în conformitate cu Reag. Ph. Eur.;filtrat prin filtru de 0,2 μm;corespunde pentru utilizarea la echipamente UPLC/ UHPLC/ Ultra HPLC;colorație: ≤ 10 Hazen;apă: ≤ 0,02%; reziduu după evaporare: ≤ 2,0 mg/l;aciditate:≤0,0002meq/g; alcalinitate: ≤ 0,0002 meq/g; densitate:0,78(la 20°C);indice de refracție(n 20/D):1,344, se distilează la 80–82 °C:≥95%(v/v) Gradul gradient la 210 nm: ≤ 1,0 mAU, la 254 nm: ≤ 0,5 mAH; Absorbanță: ≤0,10 (la 200 nm);Transmisie:- la 193 nm: ≥ 60%;- la 195 nm: ≥ 80%;- la ≥ 230 nm: ≥ 98%, Fluorescență (ca chinină la 254 nm): ≤ 1,0 ppb, ≤ 0,5 ppb (ca chinină la 365 nm);Ambalaj: 2,5 L |
| 167 | Ammonium sulfate | g | 100 | Conform Ph.Eur. Colourless crystals or white or almost white granules, very soluble in water, practically insoluble in acetone and in ethanol (96 per cent); pH: 4.5 to 6.0; Sulfated ash: maximum 0.1%; Ambalaj 100 g |
| 168 | Potassium bromate | g | 100 | Conform Ph.Eur. Ambalaj 100 g |
| 169 | Cerium sulfate tetrahydrate | g | 25 | Conform Ph.Eur.: Yellow or orange-yellow, crystalline powder or crystals, very slightly soluble in water, slowly soluble in dilute acids. Ambalaj 25 g |
| 170 | Hydrazine sulfate salt | g | 5 | Conform Ph.Eur.:Colourless crystals, sparingly soluble in cold water, soluble in hot water (50 °C) and freely soluble in boiling water, practically insoluble in ethanol (96 per cent). Content: minimum 99 per cent. Ambalaj 5 g |
| 171 | 1-Naphtholbenzein | g | 5 | Conform Ph.Eur.: Brownish-red powder or shiny brownish-black crystals, practically insoluble in water, soluble in ethanol (96 per cent) and in glacial acetic acid. Ambalaj 5g |
| 172 | Ammonium carbonate | g | 100 | Conform Ph.Eur.: White or almost white translucent mass, slowly soluble in about 4 parts of water. It is decomposed by boiling water. Ammonium carbonate liberates not less than 30 per cent m/m of NH3 (Mr 17.03). Ambalaj 100 g |
| 173 | Barium hydroxide octahydrate | g | 100 | Conform Ph.Eur.: Colourless crystals, soluble in water.Ambalaj 100 g |
| 174 | Cyclohexane | mL | 100 | Conform Ph.Eur.: Clear, colourless, flammable liquid, practically insoluble in water, miscible with organic solvents. d20 about 0.78. bp about 80.5 °C. Ambalaj 100 mL |
| 175 | Sodium periodate | g | 25 | assay: ≥99.0%, form: powder. Ambalaj 25 g |
| 176 | Buffer sol. pH 9,18 | mL | 250 | Bufer, standard de referință, trasabil la materiale standard de referință N.I.S.T.; pH 9,18 (25 °C); Ambalaj: 0.25 L. |
| 177 | Buffer sol. pH 1,68 | mL | 250 | pH 1,68 (25°C). Ambalaj: 0.25 L. |
| 178 | Buffer solution pH 10,01 | L | 0,5 | pH 10,01 (25°C). Ambalaj: 0,5 L. |
| 179 | Sodium nitrite | g | 100 | Conform Ph.Eur. assay ≥97.0%. Ambalaj: 100 g |
| 180 | Triethylamine | mL | 100 | Conform Ph.Eur. Colourless liquid, slightly soluble in water at a temperature below 18.7 °C, miscible with ethanol (96 per cent); density: about 0.727; viscosity: about 1.401; Assay≥99.5%; Ambalaj 100 mL |
| 181 | Tetrakis(decyl)ammonium bromide | g | 10 | suitable for ion pair chromatography, Assay assay ≥99.0%. Ambalaj: 10g |
| 182 | Sucrose | g | 5 | CRM for polarimeter cu incertitudinea (u); Ambalaj: 1 g |
| 183 | tri-Sodium phosphate anhydrous | g | 250 | Conform Ph.Eur.: White or almost white powder, hygroscopic. Ambalaj 250 g |
| 184 | Sodium hypophosphite monohydrate | g | 500 | Conform Ph.Eur.: Ambalaj 500 g |
| 185 | di-Sodium hydrogen phosphate anhydrous | g | 500 | Conform Ph.Eur. Ambalaj 500 g |
| 186 | Disodium tetraborate decahydrate | g | 25 | Conform Ph.Eur.: Content: 99.0 per cent to 103.0 per cent of Na2B4O7,10H2O.ppearance: white or almost white, crystalline powder, colourless crystals or crystalline masses, efflorescent.Solubility: soluble in water, very soluble in boiling water, freely soluble in glycerol. pH: 9.0 to 9.6. Sulfates: maximum 50 ppm.Ammonium: maximum 10 ppm.Calcium: maximum 100 ppm.Ambalaj: 25g |
| 187 | Zinc | g | 100 | volumetric standard; Conform Ph.Eur.: Content: minimum 99.9%. Ambalaj 100 g |
| 188 | Phenylhydrazine hydrochloride | g | 5 | Conform Ph.Eur: White or almost white, crystalline powder, becoming brown on exposure to air, soluble in water and in ethanol (96 per cent). mp: about 245 °C, with decomposition.Ambalaj 5 g |
| 189 | Pyridine anhydrous | mL | 100 | Conform Ph.Eur: Clear, colourless liquid, hygroscopic, miscible with water and with ethanol (96 per cent). bp: about 115 °C, water: max.0.01% Ambalaj 100 mL |
| 190 | Sodium chloride | g | 80 | volumetric standard, secondary reference material for argentometry. Ambalaj 80 g |
| 191 | Dibutylamine | mL | 50 | conform Ph.Eur. Ambalaj: 50ml |
| 192 | Diethylamine | mL | 100 | Content: minimum 99.5 per cent; Relative density about 0.71; bp: about 55 °C. Ambalaj: 100ml. |
| 193 | Hexadecyltrimethylammonium bromide | g | 100 | Assay: ≥98%; Ambalaj: 100g |
| 194 | tert-Butanol | mL | 500 | Assay: ≥99.0%; Ambalaj: 500 mL |
| 195 | Hexylamine | mL | 10 | Conform Ph.Eur.Colourless liquid, slightly soluble in water, soluble in ethanol (96 per cent); density: about 0.766; viscosity: about 1.418; bp: 127 °C to 131 °C; Assay: 99%; Ambalaj: 5 mL |
| 196 | dimethyl sulfoxide | L | 2.5 | conform Ph.Eur, for GC-HeadSpace. Ambalaj: 2.5L |
| 197 | Hydrogen peroxide 30 weight % solution | L | 100 | Content: 29.0% - 31.0% m/m; Acidity conform Ph.Eur.; Organic stabilisers: maximum 500 ppm; Non-volatile residue: maximum 2 g/L. Ambalaj: 2.5L. |
| 198 | Formaldehyde | ml | 500 | Conform Ph.Eur. Clear, colourless liquid; Content: 34.5 per cent m/m to 38.0 per cent m/m of formaldehyde; Sulfated ash: max 0.1 per cent; Ambalaj: 500mL |
| 199 | Trichloroacetic acid | g | 10 | conform PhEur; 98.0 % to 100.5%; Chlorides: maximum 100 ppm; Sulfated ash: maximum 0.1%. Ambalaj: 5 g. |
| 200 | Aluminium oxide basic | g | 100 | Conform Ph.Eur., for column chromatography, Particle size: 75 µm to 150 µm. pH : 9 to 10; Ambalaj: 100g |
| 201 | Sodium taurocholate hydrate | g | 1 | Solubility: H2O: 50 mg/mL, clear; Assay: ≥97.0%; impurities: ≤1% taurine, ≤2% cholic acid; Ambalaj: 1g. |
| 202 | Tin | g | 50 | Conform Ph.Eur. Silvery-white granules, soluble in hydrochloric acid with release of hydrogen; Arsenic: max 10 ppm; Ambalaj: 50 g |
| 203 | 4-(Dimethylamino)benzaldehyde | g | 25 | Conform Ph.Eur. White or yellowish-white crystals, soluble in ethanol (96 per cent) and in dilute acids; mp: about 74 °C; Ambalaj: 25 g. |
| 204 | 1,2-Naphthoquinone-4-sulfonic acid sodium salt | g | 10 | Assay: 97%; Ambalaj: 10 g |
| 205 | Chloramine T trihydrate | g | 5 | Conform Ph.Eur. White or slightly yellow, crystalline powder; pH: 8.0 to 10.0; Ambalaj: 5g. |
| 206 | Arsenic(III) oxide | g | 25 | Conform Ph.Eur. Crystalline powder or a white or almost white mass, slightly soluble in water, soluble in boiling water; Assay :≥99.0%; Ambalaj: 25 g. |
| 207 | (Ethylenedinitrilo)tetra-acetic acid | g | 100 | Conform Ph.Eur. White or almost white crystalline powder, very slightly soluble in water; mp: about 250 °C; Assay: ≥98.5%, Ambalaj: 100g |
| 208 | Sodium metabisulfite | g | 250 | Conform Ph.Eur. White or almost white, crystalline powder or colourless crystals; pH: 3.5 to 5.0; Iron: maximum 20 ppm; Content: 95.0 per cent to 100.5 per cent; Ambalaj: 250g |
| 209 | caffein | g | 1 | CRM for melting point cu incertitudinea. Ambalaj: 1g |
| 210 | vanilin | g | 1 | CRM for melting point cu incertitudinea. Ambalaj: 1g |
| 211 | Refractive index standart kit 2 | buc | 1 | toluen/apă; testat la 20°C; cu încertitudinea măsurării. |
| 212 | Holmium(III) oxide | g | 15 | conform Ph.Eur. Ambalaj: 5g. |
| 213 | Nicotinic acid | g | 2 | For equipment qualification, European Pharmacopoeia (EP) Reference Standard; Ambalaj: 1 g |
| 214 | fuchsin (CAS: 632-99-5) | g | 5 | Conform Ph.Eur., A mixture of rosaniline hydrochloride (Colour Index No. 42510; Schultz No. 780) and para-rosaniline hydrochloride (Colour Index No. 42500; Schultz No. 779). Ambalaj: 5g |
| 215 | ignitor for FID detector | buc | 2 | FID detector, Agilent 7890A |
| 216 | absorber tube with tubing nipple | buc | 5 | Height 155mm; Inner diameter 32mm; SGJ size B-14/15. |
| 217 | fused silica tubing | buc | 1 | I.D. 75 µm ± 3 µm / O.D. 280 µm ± 10 µm, 5 meters(LCP# FS-75) |
| 218 | fused silica tubing | buc | 1 | I.D. 50 µm ± 3 µm / O.D. 280 µm ± 10 µm, 5 meters(LCP# FS-50) |