Anexa nr. 1

| № | Sistem audio-video integrat | Unitate | Cant. |
| --- | --- | --- | --- |
| 1 | VIDEOPROIECTOR REZOLUTIE Nativa 1920x1080 ASPECT RATIO 16:9 Native, 16:10/4:3 Compatible LUMINOZITATE Min.5500 Lumeni CONTRAST Min.300.000:1 LAMPA Up to 30000 hours SURSA DE LUMINA Laser INTRARI/IESIRI Min. 2x HDMI v1.4a, 1*HDBaseT, VGA In, Audio In, Mic In, VGA Out, Audio Out, RJ45, RS232, 12V Trigger, 1*USB-A Power, 1*USB-B Service Port AUDIO Min 2*10W stereo ZOOM OPTIC 1.6x Manual RAPORTUL DE PROIECTARE 1.2:1~1.92:1 DISTANTA DE PROIECTIE – diapazon minim: 128 cm - 799cm CORECTIA TRAPEZOIDALA min. ± 30° Vertical ± 30° Horizontal LENS SHIFT min. Vertical: +25%; Horizontal: +10% GREUTATE max. 11kg CONSUM DE ENERGIE Max. 370W (Bright), 0.5W (Standby) PROJECTION METHOD 360° operation, 24/7 mode SECURITATE Kensington® lock port, security bar, password protection CONTROL LA DISTANTA Infra-Red Remote Control, Wired Control Certificare IP5X (rezistenta la praf) ACCESORII INCLUSE AC Power Cord, Infra-Red Remote Control, Batteries X 2 CULOAREA ALBA GARANTIE: minim 36 luni GARANTIA LAMPII: minim 60luni/12000H | buc | 1 |
| 2 | ECRAN DE PROIECTIE ELECTRIC Diagonală 120", Aspect 16:9 Motorized Sistem de control al ecranului – integrat in carcasa, inclusiv: - switch - sistem de control la distanta integrat (telecomanda fara fir) - Trigger +12V for projection system automatization - Port RS232 12V trigger cable includded – 10meters Tubular Motor Dual Wall and Ceiling Mountable Viewing Size (wxh) (mm) 2656 x 1494 Diagonal Viewing (inch) 120 Black Border R & L (mm) 50 Black Border Bottom (mm) 55 Black Border Top (mm) 300 Weight (Kgs) ~13.6kg Noise level: max. 45dB Dimensiunile carcasei (Inclusiv suportul de montare) 2983 x 108 x 113 Garantie: minim 12 luni | buc | 1 |
| 3 | KIT DE PRINDERE A VIDEOPROIECTORULUI Compatibil cu videoproiectorul inaltime ajustabila de la 57cm pana la 82 cm, rezistent la proiectoare cu masa de pana la 15 kg, posibilitate de rotire la +/- 30° a videoproiectorului, posibilitatea inclinarii videoproiectorului cu pana la 20°, posibilitatea instalarii pe tavan arcuit, cu canal pentru ascunderea cablului CULOAREA ALBA (in conformitate cu culoarea proiectorului) GARANTIE: minim 12 luni | buc | 1 |
| 4 | TV 55" FullHD, HDMI input, greutate, max.20kg, Garantie: minim 24 luni | buc | 4 |
| 5 | Kit de prindere de perete pentru TV 32-60" - cu posibilitatea actionarii de la telecomanda sau din aplicatie mobila. Include posibilitatea setarii a minim 2 pozitii, cea initiala, cea de lucru(intoarsa) Motorized TV Mount Material: Steel,Aluminum,Plastic Metal Sheet Thickness: Bracket Arm THK=1.5mm cold-rolled sheet (Bracket Arm THK=0.059" cold-rolled sheet) Surface Finish: Sand-blasted Aluminum Anodic Oxidation,Powder Coating Fit Screen Size: 32"-60" Mounting Hole Pattern : VESA & Universal VESA Compatible: 200x200,300x300,400x200,400x400,600x400 Max. VESA: 600x400mm Weight Capacity: up to 35kg (77lbs) Garantie: minim 12 luni | buc | 4 |
| 6 | Computer All-in-One PC - 23,8" FullHD Multi-Touch, Windows 10 Home SL Ru, CPU Intel® Core® i3-7100T sau analog, RAM 4GB DDR4, SSD 256GB M2., DVD-RW integrat, integrated Graphics, Wi-Fi, BT, Gigabit LAN, 135W PSU, KB/MS, Garantie: minim 24 luni | buc | 1 |
| 7 | Podium mobil digital Dimensiuni (mm) aprox: 688 X 744 X 1122 Greutate (kg) max: 65 kg Dimensiune frontală 32" cu media player inclus Rezoluție 1920x1080 FHD, 16:9 Luminozitate 350 cd/m2 Tip panel IPS Panel din față adaptabil pentru Media Player Panel din față cu Content Management Software Interfață de lectură Panel 22" Rezoluție 1920x1080 FHD Luminozitate 250 cd/m2 Panel Type IPS Aspect ratio 16:9 Touch Technology Capacitive Touch Points 10 points Lecturer Panel PC Specifications CPU Intel Core i3 or similar RAM 4 GB DDR4 HDD 120 GB SSD Operating System Windows Embedded 10 IoT Microfon – 2 buc Lungime 18" Conector de legătură 3-way XLR (Male) tip microfon Cardioid Frecvență de răspuns 50 -18 kHz Sensitivity (at 1,000Hz) -60dB ± 3dB Impedance 220Ω Power ON indicator LED Power requirement 2 x Phantom Power (48V) Intrări (on Speaker Panel) USB USB 2.0 x 4 (for internal PC) Ieșiri (Bottom of Podium) HDMI Ethernet XLR for microphone USB USB 2.0 x 2 (for internal PC) Sursă Sursă de intrare 230V 50Hz Consum 450W (max) Garantie: minim 12 luni | buc | 1 |
| 8 | AMPLIFICATOR CU MIXER DE SEMNAL INCORPORAT Putere de iesire – min.240 W Posibilitate impartire semnal in maxim 5 zone Intrari: minim 3 x balanced XLR, 3 x stereo RCA, 6 x 6,3mm jack socket, 1 x 3,5mm TRS Jack. Iesire: minim 6 x phoenix, 2 x 6.3mm Jack Frequency response – cel putin 80 Hz - 15 kHz THD –cel mult 0,5% la 1kHz putere S/N ratio - 75 Db Equalizer - 125 Hz, 250 Hz, 500 Hz, 1 kHz, 2 kHz, 4 kHz, 8 kHz, ± 12 dB Phantom power – 15V max. posibilitate de instalat pe masa sau in rack pentru boxe de tip 4Ohm, 25V, 70V si 100V reglare putere sunet intrari si master greutate.min9kg, max.10kg GARANTIE: minim 12 luni | buc | 1 |
| 9 | Incinta acustica Specificatii minime solicitate: Tipul – incastrabile in tavan Putere de iesire – min.25 W (8Ohm) Transformator 100V cu comutator pentru: 15 W – 7,5 W – 3,75 W - 8 Ω Sensibilitate: 89 dB (1 W/1 m) Frequency response – cel putin 80 Hz - 20 kHz Maximum SPL 101dB (Pmax/1m) la 15,694 kHz Dispersie conica la 120grade Dimensiuni max. diametru extern 240 mm, adancimea 95 mm Difuzoare incorporate de frecvente joase de 5" (130 mm) si de frecvente inalte de 0,5" (13 mm) Greutate – minim 0.9kg, max.1.3kg Capac magnetic care acoperă toata suprafața incintei acustice Culoarea alba | buc | 12 |
| 10 | Full Digital Networked DSP Conference Processor One CAT5e cable to transmit up 64 channels audio and other signals The microphone sensitivity of each unit and 16 level equalizer EQ can be adjusted independently The 16-level equalizer EQ to achieve sound or the whole conference field adjustment, limiter to prevent the sound suddenly become loud or low; noise gate of human voice can be identified; support AGC/AFC/ Mix (Auto-mix) technology Interfaces Front panel USB recording interface 3.5mm headphone for monitoring 1x RCA audio input 1x XLR audio input 1x RCA audio output 1x XLR audio output 6x Phoenix interface audio output 2x RJ45 digital audio connection 2x RJ45 for chairman unit, delegate unit, unit translators and other connection 1x RJ45 for wireless AP or router 1x DP9 male interface for external control 1x DP9 female interface for camera 1x three-pin phoenix interface for camera control keyboard 1x RJ45 for computer connection or network switcher 1x grounding screw 1x 100 ~ 240V DC Power Interface Parametri tehnici Sursă 110V / 220V AC Static consum 12W Consum maxim 150W Frecvență 20Hz ~ 20KHz Nivelul de zgomot (S / N)> 80dB Total harmonic distortion <0.05% Channel Crosstalk> 80dB Rackmountable Garantie: minim 12 luni | buc | 1 |
| 11 | Microfon digital cu fir Gooseneck microphone 410mm lenght Control & Indicators Each unit with two-color LED indicator on microphone, red for speaking 128x32 OLED display with volume bar, clock, time span of microphone activating, interpretation channels and language, MIC ON/OFF etc and other information. 2 groups of channels selecting buttons MIC ON/OFF buttons with indicators Chairman unit with priority button and consent button for the delegates Interfețe Pluggable microphone base 2x 3.5mm stereo headphone jack 1x 3.5mm stereo input jack 2x RJ45 connectors Parametri tehnici Button: Touchable interface, no physical buttons Sound Pickup: heart type capacitance Display: OLED display 128 × 32 Sensitivity: -46 dBV / Pa Maximum power consumption: 2.0W Directivity: 0 ° / 180 °> 20 dB (1 kHz) Headphone load: 16Ω headphone volume: 10mW Headphone jack: 3.5mm stereo Input impedance: 2kΩ Frequency response: 20 ~ 20000Hz Connection: CAT5 / CAT6 cable + snap Equivalent noise: 20dBA (SPL) Main material: ABS The maximum sound pressure: 125dB (THD <3%) Greutate: max 1.1kg (with microphone) Garantie: minim 12 luni | buc | 1 |
| 12 | Microfon digital cu fir Gooseneck microphone 410mm lenght Control și indicatori Fiecare unitate are două culori LED, roșu pentru vorbire, verde pentru a cere permisiune de la moderator de a vorbi 128x32 OLED display with volume bar, clock, time span of microphone activating, interpretation channels and language, MIC ON/OFF etc and other information. 2 groups of channels selecting buttons MIC ON/OFF buttons with indicators Interfețe Pluggable microphone base 2x 3.5mm stereo headphone jack 1x 3.5mm stereo input jack 2x RJ45 connectors Parametri tehnici Button: Touchable interface, no physical buttons Sound Pickup: heart type capacitance Display: OLED display 128 × 32 Sensitivity: -46 dBV / Pa Maximum power consumption: 2.0W Directivity: 0 ° / 180 °> 20 dB (1 kHz) Headphone load: 16Ω headphone volume: 10mW Headphone jack: 3.5mm stereo Input impedance: 2kΩ Frequency response: 20 ~ 20000Hz Connection: CAT5 / CAT6 cable + snap Equivalent noise: 20dBA (SPL) Main material: ABS The maximum sound pressure: 125dB (THD <3%) Greutate: max 1.1kg (with microphone) Garantie minim 12 luni | buc | 4 |
| 13 | Sistem de doua microfoane radio un transmițător și pentru 2 receptoare Carrier frequency range - 606 MHz - 621 MHz Channels – 32 selectable channels, automatic channel targeting Frequency response - 45 Hz - 18 kHz Type - Antenna diversity THD < 0,7 % S/N ratio > 105 dB Power supply – 12-15DC, 1.25A Outputs – Balanced XLR, unbalanced 6,3mm TS Jack Receiver way – two frequency conversion super heterodynes Antenna connector – BNC Material – steel enclosure, aluminium front panel Garantie: minim 12 luni | buc | 1 |
| 14 | SISTEM FARA FIR DE TRANSMITERE A SEMNALULUI VIDEO HDMI Compus din transmitator si receptor 2D Video Streaming 1080p24, 1080p50, 1080p60 1080i50, 1080i60 720p50, 720p60, 576p, 480p 3D Video Streaming Frame-pack: 1080p24, 720p50/60 Side-by-Side (half):1080i50/60, 720p50/60 Over-Under: 1080p24, 720p50/60 Audio Support up to 6 Mbps AC-3 and DTS Transmission Range 20 meters “In Room” Intrari/Iesiri transmitator 2x HDMI in (v1.4a 3D support) HDCP 1.2, 1 x HDMI out (v1.4a 3D support) HDCP 1.2 Alimentare transmitator 5V curent continuu Dimensiuni maxime 182 x 92 x 31 mm Intrari/Iesiri receptor 1 x HDMI (v1.4a 3D support) Alimentare receptor 5V curent continuu prin mini-USB Dimensiuni maxime 95 x 95 x 34 mm System Latency Mai putin de 1ms Antenna Omni-directional High Performance Internal Antennas Accessories Mini-USB Power Adaptor, DC Power Adapter, 2x HDMI (1.5m) Cable. Fixing Kit, Quick Start Guide, User manual, IR repeater, Remote control GARANTIE: minim 12 luni | buc | 1 |
| 15 | Transmitator semnal HDMI 1080p prin UTP HDMI extender prin cablu UTP cat.5e/cat.6 la distante de pana la 120m Support HDCP version - HDCP1.2 Input HDMI resolution - 480i@60Hz、480p@60Hz、576i@50Hz、576p@50Hz、720p@50/60Hz、1080i@50/60Hz、1080p@50/60Hz； Output HDMI resolution - 480p@60Hz、576p@50Hz、720p@50/60Hz、1080p@50/60Hz Support video color format - 24bit/Deep color 30bit/Deep color 36bit Support audio format - L/R stereo audio Max transmission rate - 10.2Gbps Audio signal transmission format - L/R stereo audio GARANTIE: minim 12 luni | buc | 3 |
| 16 | Receptor semnal HDMI 1080p prin UTP HDMI extender prin cablu UTP cat.5e/cat.6 la distante de pana la 120m Support HDCP version - HDCP1.2 Input HDMI resolution - 480i@60Hz、480p@60Hz、576i@50Hz、576p@50Hz、720p@50/60Hz、1080i@50/60Hz、1080p@50/60Hz； Output HDMI resolution - 480p@60Hz、576p@50Hz、720p@50/60Hz、1080p@50/60Hz Support video color format - 24bit/Deep color 30bit/Deep color 36bit Support audio format - L/R stereo audio Max transmission rate - 10.2Gbps Audio signal transmission format - L/R stereo audio GARANTIE: minim 12 luni | buc | 6 |
| 17 | Switch - 16-port switch 10/100M RJ45 ports, 1U 19-inch rack-mount cu carcasa metalică | buc | 1 |
| 18 | Convertor HDMI input up to 4K to HDMI output up to 4K + audio SPDIF + audio 3,5mm stereo | buc | 1 |
| 19 | Cablu UTP cat. 5e /0,50 mm cooper 4 pairs PVC | m | 600 |
| 20 | Set de cabluri audio, inclusiv conectoare | set | 1 |
| 21 | Cablu pentru conectarea boxelor 2x0.75mm | m | 100 |
| 22 | Cablu HDMI 1,8m v1.4 | buc | 10 |
| 23 | Rack de podea 18U 600х800x900, 19", negru, cu usa de sticla, cheie, lacate la toti peretii care pot fi deschisi, bloc de prize, minim 8, 2 polite cu adancime 450mm, bloc de ventilatoare | set | 1 |
| 24 | Servicii de trasare, montare, instalare, setare si punere în funcțiune a sistemului audio-video integrat | serv | 1 |