| - Specificaţii tehnice (F4.1) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Acest tabel va fi completat de către ofertant în coloanele 3, 4, 5, 7, iar de către autoritatea contractantă – în coloanele 1, 2, 6, 8]. |  |
| Numărul procedurii de achiziție______________din_________ |  |  |  |  |  |  |  |  |
| Denumirea procedurii de achiziție: Achiziții de valoare mică |  |  |  |  |  |  |  |  |
| Denumirea bunurilor/serviciilor | Modelul articolului | Ţara de origine | Produ-cătorul | Specificarea tehnică deplină solicitată de către autoritatea contractantă |  | Specificarea tehnică deplină propusă de către ofertant | Standarde de referinţă |  |
| 2 | 3 | 4 | 5 | 6 |  | 7 | 8 |  |
| Lotul 1 |  |  |  |  |  |  |  |  |
| Mixer |  |  |  | Compact Mixer/4x4 USB Interface with FX 4 in/4 out USB audio interface (24-bit/96kHz) Cubase LE software included; Cubasis LE app included 4 mic/line inputs with separate XLR and TRS jack sockets; 2 stereo inputs with TRS jack sockets 2 Guitar DI high impedance inputs, eliminating the need for DI boxes; internal FX Engine with Tap Tempo Lo-cut filter for cleaning up unwanted low frequency noise; 3 band EQ for easy creative and corrective tonal shaping Item Weight 5.1 pounds Product Dimensions 10.8 x 13.1 x 3.8 inches |  |  |  |  |
| Lotul 2 |  |  |  |  |  |  |  |  |
| Boxe |  |  |  | Boxa activa 2 cai Frecventa raspuns: 52 - 20.000 Hz [-10dB] Max SPL: 129 dB HF: 1" driver compresie Dispersie orizontala: 100° (85° sus/120° jos) Dispersie verticala: 85° (+ 25°/-60°) Con Asimetric LF: 12", bobina de 2" Amplificator Clasa D Putere RMS 600 W Putere max,: 1200 W DSP 56 bit Convertor la 24 bit/48 KHz 8 Preseturi Frecventa crossover: 2000 Hz LimiterPeak, RMS, Thermal Advanced DSP Functions Linear Phase FIR Filters Intrare: (XLR/Jack 6.3 mm) MIC/Line/instr. Iesire: 1x XLR tata (CH1 Link/Mix OUT) Cabinet polipropilena Grila otel Manere: 2 Puncte agatare: 2XM10 sus + 1x M10 spate Socket stativ de 36 mm Montare ca monitor la 45° Dimensiuni: 350 mm x 642 mm x 349 mm Greutate: 14.3 kg |  |  |  |  |
| Lotul 3 |  |  |  |  |  |  |  |  |
| Suport boxe |  |  |  | Height from 1,440 to 2,230 mm Height adjustment locking screw with push-button system Leg construction tube legs with cross braces Material steel Max. load capacity 50 kg Product Category Topline Rod combination 2-piece folding design Special features patented gradual height adjustment system; integrated shock absorber; mounting aid included Tube end(s) diameter 35 mm Type black Weight 5.91 kg |  |  |  |  |
| Lotul 4 |  |  |  |  |  |  |  |  |
| Microfon Wireless |  |  |  | Receivers Dimensions approx. 200 x 42 x 127 mm Weight approx. 680g Power supply 12 V DC nom. / 300 mA Housing metal, microphone: robust ABS AF frequency response 50 to 16,000 Hz (–3 dB) Microphone Dimensions approx. 260 x 50 mm Weight approx. 245g Operating time approx. 10 hrs Power supply 2 AA size batteries, 1.5 V AF frequency response 80-16,000 Hz Microphone condenser Frequency response 50 to 16,000 Hz (–3 dB) THD, total harmonic distortion ≤ 0.9% Pick-up pattern super-cardioid Signal-to-noise ratio ≥ 103 dBA RF output power 10 mW Switching bandwidth up to 24 MHz Modulation wideband FM Output voltage 1/4“ (6.3 mm) jack socket (unbalanced): +6 dBu, XLR socket (balanced): +12 dBu Frequency 8 frequency banks, each with up to 12 factory-preset channels Frequency range A: 548-572 MHz GB: 606-630 MHz B: 614-638 MHz C: 766-790 MHz D: 794-806 MHz E: 821-832 MHz, 863-865 MHz K: 925-937.5 MHz Sensitivity < 3 μV at 52 dB(A)rms S/N Squelch switchable from 3 dBμV to 28 dBμV (via Pilot Tone) Synchronization 2.4 GHz, Low Power OQPSK (only active during synchronization) Input sensitivity 1.8 mV / Pa Audio adjustment range 40 dB, adjustable in 5-dB steps Line / Mic level 20 dB, switchable Temperature Range operation: 0°C to +40°C, storage: –20°C to +70°C Receiver Principle double superheterodyne |  |  |  |  |
| Lotul 5 |  |  |  |  |  |  |  |  |
| Di Box Pasiv |  |  |  | carcasă: construcție rezistentă, metalică (otel inoxidabil) dimensiuni: l= 103 mm; w= 75 mm; h= 49 mm intrare și iesire nebalansată: jack 6.3 mm ieșire balansată: XLR mamă cu 3 pini nivel maxim de intrare fără atenuare: +10 dBu (2.5 V) / 20 kΩ nivel maxim de intrare cu atenuare 20: +30 dBu (25 V) / 40 kΩ nivel maxim de ieșire: -10 dBu (250 mV) / 200 Ω răspuns in frecvențe: 10 Hz - 30 kHz (+/- 0.3 dB) distorsiune armonică: <0.1 % |  |  |  |  |
| Lotul 6 |  |  |  |  |  |  |  |  |
| Cablu |  |  |  | SST0-MRR-M0-036 + CC 302 (2 metri) |  |  |  |  |
| Lotul 7 |  |  |  |  |  |  |  |  |
| Cablu |  |  |  | (MY206) XLR, 2 metri |  |  |  |  |
| Lotul 8 |  |  |  |  |  |  |  |  |
| Cablu |  |  |  | XLR (MY206), 20 metri |  |  |  |  |
| Semnat:_______________ Numele, Prenumele:_____________________________ În calitate de: ________________ Ofertantul: _______________________ Adresa: ______________________________ - Specificații de preț (F4.2). [Acest tabel va fi completat de către ofertant în coloanele 5,6,7,8, iar de către autoritatea contractantă – în coloanele 1,2,3,4,9,10]. Numărul procedurii de achiziție______________din_________. Denumirea procedurii de achiziție: Achizitii de valoare mica. Cod CPV; Denumirea bunurilor/serviciilor; Unitatea de măsură; Canti-tatea; Preţ unitar (fără TVA); Preţ unitar (cu TVA); Suma fără TVA; Suma cu TVA; Termenul de Livrare/prestare; Clasificație bugetară (IBAN). 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. Lotul 1. 32210000-8; Mixer; buc; 1; 16.12.2019; MD05TRPBAA314110A15841AB. Lotul 2. 32210000-8; Boxe; buc; 2; 16.12.2019; MD05TRPBAA314110A15841AB. Lotul 3. 32210000-8; Suport boxe; buc; 2; 16.12.2019; MD05TRPBAA314110A15841AB. Lotul 4. 32210000-8; Microfon Wireless; buc; 2; 16.12.2019; MD05TRPBAA314110A15841AB. Lotul 5. 32210000-8; Di Box Pasiv; buc; 1; 16.12.2019; MD05TRPBAA314110A15841AB. Lotul 6. 32210000-8; Cablu; buc; 1; 16.12.2019; MD05TRPBAA314110A15841AB. Lotul 7; MD05TRPBAA314110A15841AB. 32210000-8; Cablu 2 m.; buc; 4; 16.12.2019. Lotul 8; MD05TRPBAA314110A15841AB. 32210000-8; Cablu 20 m.; buc; 2; 16.12.2019. TOTAL. Semnat:_______________ Numele, Prenumele:_____________________________ În calitate de: ______________ Ofertantul: _______________________ Adresa: ________________________________________________________; 16.12.2019. |  |