| Anexa nr. 22 la Documentația standard nr. 115 din 15.09.2021 nr. ## Specificaţii tehnice |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Acest tabel va fi completat de către ofertant în coloanele 2, 3, 4, 6, 7, iar de către autoritatea contractantă – în coloanele 1, 5,]. |  |  |
| Numărul procedurii de achiziție: ocds-b3wdp1-MD-1753881961729 din |  |  |  |  |  |  |  |  |
| Obiectul achiziției: Centru informațional de date |  |  |  |  |  |  |  |  |
| Denumirea bunurilor/serviciilor | Denumirea modelului bunului/serviciului | Ţara de origine | Producătorul | Specificarea tehnică deplină solicitată de către autoritatea contractantă | Specificarea tehnică deplină propusă de către ofertant | Standarde de referinţă |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  |
| COMPARTIMENTUL SERVERE |  |  |  |  |  |  |  |  |
| Infrastructura Servere |  |  |  | Infrastructură computing: 3 x Computing Servers with following configuration: 2 x CPU min. 2.3GHz, min. 32Cores, min. 144MB cache, DDR5 RAM 512GB DDR5-6400 RDIMM. min. 32 slots. Raid Controller with Raid 0, Raid 1 support. Boot disks min. 2 x 480GB SSD, used for hypervisor. Network card: 4 x 25Gb Intelligent Ethernet, RoCEv2 and NVMe/TCP support, with possibility to divide in logical ports. Security: TPM Module 2.0 included. Fans: Redundant fans. Power Supply, Redundancy: 6 x min. 2600W, Titanium Efficiency, hot swap, N+1. Power cords included. Network interconnect: Redundant network connectivity module, with min. 16 x 100Gb Ethernet ports, dedicated management ports, console. Management: Remote KVM for each server, centralized management for all servers from a single dashboard, updates (including firmware) policies, security policies, operation automation and orchestration, integration with 3rd party host OS and monitoring systems. Scalability: future computing node updates without additional cost for network components, PSU, FANs. Architecture: All components must be compatible, validated as an architecture with full interoperability. Warranty: 3 years for software updates, hardware replacement on-site. Equipment must be registered at vendor support portal and attached to company profile for at least future access to support renewals, updates, notification, recommendations. |  |  |  |  |
| COMPARTIMENTUL STOCARE ȘI BACKUP |  |  |  |  |  |  |  |  |
| Sistem de stocare All Flash |  |  |  | All Flash Storage System, NVMe. Dual controller, with high availability and automatic failover. Chassis, 2U rackmount: dual controller and min. 24 x NVMe disk slots. Architecture: scalable and resilient architecture, clustered controllers with non-disruptive failover in case of failure or component updates. System must be scalable with more than 4 nodes. Disk included: min. 24 x 3.8TB NVMe disks, hot swap. Host ports per system: min. 6 x 10/25Gb Ethernet with support of NVMe/TCP, RDMA ; 8 x 32Gb FC. Storage protocols included: Block (at least iSCSI, FCP) and NAS (SMB, NFSv4). Storage features included: storage space efficiency mechanisms (data deduplication and compression), thin provisioning, LUN/Volume resize, LUN/Volume clone, LUN/Volume QoS, redirect on write snapshots and clones, replication (synchronous and asynchronous), antiransomware protection, MFA, multitenancy. All features must be covered with licenses. Redundancy: dual controller, redundant fans, power supply, management. Management and monitoring: Monitoring and management solution included for day one operation and activity, providing at least: performance indicators (latency, IOPS, throughput) for storage node, disks, cache, host ports, volumes/shares, headroom information, predictive information for performance and space grow, recommendations for system optimization or best proactive validation, alerts, proactive actions. Warranty: 3 years for software updates, hardware replacement on-site. Equipment must be registered at the vendor support portal and attached to the company profile for at least future access to support renewals, updates, notification, recommendations. ProActive support 24/7, up to 30-minute call to repair. |  |  |  |  |
| COMPARTIMENTUL REȚEA ȘI CONECTIVITATE |  |  |  |  |  |  |  |  |
| Switch rețea |  |  |  | Data Center network switches Chassis: rackmount chassis 19" with included rackmount kit. Architecture: redundant, non-blocking ports, low latency performance with L2 and advanced L3 routing protocols included. Ports: total of 72x10/25Gbps SFP28 ports, with possibility of using aggregated min. 4x40/100Gbps QSFP ports. Included SFP: min. 24x10Gb SFP+ SR modules, min. 8x25Gb SFP SR modules Performance: min. 3.4Tbps, converged and lossless ethernet Included features: IEEE 802.1Qbb (Priority Flow Control);IEEE 802.1Qaz (Enhanced Transmission Selection); DCBX (Data Center Bridging Exchange Protocol); Routing included (BGP, OSPF, IS-IS) overlay included (VXLAN EVPN) VLAN tagg. (802.1Q),LACP; Trunking; STP, RSTP; Link Aggregation, vPC or equivalent; MACSec (AES-256) included; QoS. Management: dedicated console port, 1x100/1000Base-T out-of-band Redundancy: (1+1) hot swappable PSU, FANs (N+1). Warranty: 3 years for software updates, hardware replacement on-site. Equipment must be registered at vendor support portal and attached to company profile for at least future access to support renewals, updates, notification, recommendations., certificat CE/ISO |  |  |  |  |
| COMPARTIMENTUL INFRASTRUCTURĂ ȘI MATERIALE AUXILIARE |  |  |  |  |  |  |  |  |
| UPS tip tower, 10 kW |  |  |  | Modular UPS Characteristics: online, min. 10kVA 3-phase, double conversion eff. min. 96%. Autonomy: min. 11 minutes at 8KW load. Battery modules: min. 2 x 16x9Ah VRLA long life. Management: SNMP/Web. Formfactor: rackmount, up to 12U Warranty: 2 years, on-site with next business day parts replacement from local ZIP stock. |  |  |  |  |
| Rack server 42U |  |  |  | Rack server 42U, 19”, min. 800x1000mm, perforated door, grounding kit, wheels, cable organizer, EN 61587-1/CE |  |  |  |  |
| Filtru de alimentare pentru rack |  |  |  | Power distribution unit Type: Rack PDU, Monitored on LAN, 0U space. Characteristics: 16A, 220V/380V with 3m x IEC60309, sockets min. 30xC13, additional 6 x C19. Warranty: 2 years. |  |  |  |  |
| Organizator de cabluri pentru rack |  |  |  | Organizator cabluri frontal/vertical, compatibil rack 19”, min. 12 treceri, CE/ISO |  |  |  |  |
| COMPARTIMENTUL LUCRĂRI ȘI SERVICII ASOCIATE |  |  |  |  |  |  |  |  |
| Lucrări de instalare și configurare echipamente |  |  |  | Instalare fizică a serverelor, rețelei, storajului, montare în rack, conectare cabluri, testare, validare funcțională, configurare IP și management, etichetare, standard EN/ISO |  |  |  |  |
| Instruire personal IT |  |  |  | Instruire tehnică pentru personal IT privind administrarea, monitorizarea și mentenanța platformei, certificat de participare, EN/ISO |  |  |  |  |
| Suport tehnic post-implementare |  |  |  | Suport la distanță/telefonic pentru întrebări, troubleshooting, consultanță timp de 12 luni după livrare, inclus în garanție, EN/ISO |  |  |  |  |

Semnat:_______________ Numele, Prenumele:_____________________________ În calitate de: ________________

Ofertantul: _______________________ Adresa: __________________________

________________________________________________________________________________________________