CAIER DE SARCINI

privind achiziționarea echipamentului pentru implementarea proiectelor CPORT, VIGILANT, POLIIICE și FALCON

| Nr. lot/ poziție | Denumirea bunurilor/ serviciilor/ lucrărilor Cod CPV | Specificarea tehnică deplină solicitată, Standarde de referință, Certificate de conformitate/calitate |
| --- | --- | --- |
| 1. | Stație de lucru (computer) cu 2 monitoare, mouse, tastatură | CPU; Last Generation (Launch Date Q1, 2023 or later), min. 10 cores, base/turbo frequency min. 2.1 GHz/5.2 GHz, cache memory min. 30 MB. RAM; Minimum 32 GB (2 x 16 GB), DDR5-4800, 4.5 GHz ECC Registered Memory. Memory slots:; Minimum 4 DIMM slots with support for Dual Channel mode. Storage (internal drive):; "Minimum 1 TB SSD, read/write speed (6000/4000 MB/s); Minimum 4 TB SATA 3.5"" HDD, 7200 rpm.". Expansion slots:; minimum 2x PCI-E 3.0 (maximum height, full-length) and 2x slots must be free; 1x PCI. Graphics card:; Minimum 8 GB GDDR6 (NVIDIA T1000 or equivalent); min. 128-bit Memory width and 160GB/s Mem. Bandwidth, min. 2x digital ports with support not less than 4K resolution each (compatibil with 2 monitors); 2 x HDMI and DisplayPort. Audio:; integrated audio. Ports:; Minimum 8 x USB 3.2 ports (4x front, 4x rear); 2x RJ45; 2x HDMI and DisplayPort with multi-stream support (compatibil with 2 monitors); 1x line input, 1x line/headphone output, 1x headphone, 1x microphone; 1x Universal audio port. Network board:; Integrated (with support 10/100/1000Mbps). Current supply:; Power supply at least 700 W with an efficiency of at least 90% - active PFC function, SCHUKO type connector. Acoustic characteristics:; max. 30 dBA. Security management:; Trusted Platform Module 2.0 security chip; USB connect/disconnect; login/BIOS setup password; combination lock + sensor, which ensures locking of the hood of the main block and notification in case of unauthorized access to the inside of the computer, which can be managed through BIOS (opening/closing) or through remote access. Optical drive; CD/DVD-RW (slim). Keyboard:; Eng/Rus USB same brand as workstation. Mouse:; USB optical, same brand as the workstation. Pre-installed operating system; Windows 11 Pro for Workstations. Office; Licensed Microsoft Office pro min. 2021. Monitor; Cureved display monitor, Diagonal of at least 34 inches, IPS/VA/OLED, Resolution 3440x1440@75Hz or better, Brightness min. 400 nits, Contrast Static / Dynamic 1500:1 / 5 mln:1; sRGB min. 99%; Anti-reflection screen; Low Blue Light (Hardware Solution); Tilt; Rotation; Adjustable height (0-130mm); VESA support; Input signal: 1x HDMI; 1x DisplayPort; 1x Type-C port (DP 1.2 alt mode); min 2x USB Type-A ports (1x port must support charging function); Integrated Stereo speakers (min. 3W per channel); DP->DP min. 1.8m Cable, USB Type-C->USB Type-C min. 1.8m Cable, AC Cable min. 1.8m; Power supply 100-240 VAC 50/60 Hz;. Guarantee; Minimum 60 months for components, labor and service at the workshop keep your drive service 2+1 years.. Perioada de garanție; min. 36 de luni. Anul de producere:; Nu mai vechi de 1 an din momentul livrării. Authorized service in the Republic of Moldova. |
| 2 | Laptop | Display:; 14"; Resolution min. 2880 x 1800; Brigtness min. 400 nits; Anti-Glare; 10-point touch support; Viewing Angle 170°/170° Display 360 degree rotation, tablet mode.. CPU:; Base Frequency min. 1.9GHz; Turbo Frequency min. 4.9GHz; L3 Cache min. 12MB; min. 12 core; Launch Date by Manufacturer Q1-2023. RAM:; Min. 16GB DDR5. Storage:; Min. 1Tb M2 NVMe PCIe 4.0. Video:; Integrated or discret. Connectivity:; WiFi 6E + BT v5.3; RJ45 or USB Type-C to RJ45 adapter. Multimedia:; webcamera min. 1080P with IR and Windows Hello support; Privacy Shutter; Stereo Speakers, Dual Array Microphone with Noise reduction software. I/O Ports:; 2x USB Type-C 3.2 Gen 2, (DisplayPort™ 1.4 / Power Delivery 3.0); 1x USB Type-A 3.2 Gen 1; Audio Combo port; minim HDMI 1.4. Battery and Power supply:; Battery min. 65Whr; USB Type-C power adapter. Weight:; max. 1.5kg. OS:; Microsoft Windows 11 Pro, OEM license, pre-installed on the factory (the activation key for Microfot Windows has to be written in the BIOS). Application SW:; Microsoft Office 2021 Home&Business (eng), Retail version in the Original Microsoft Package. Input Devices:; Backlit spill-resistant Keyboard with Numeric keypad (Eng/Rus); Touchpad or Clickpad; Mouse wireles or BT with USB adapter + USB keyboard.. Laptop Case:; 14" laptop Backpack or Bag. Docking Station:; USB 3.2 Type-C, 3x USB 3.2 Type-A, Gigabit Ethernet port (same manufacturer as laptop or compatible). |
| 3 | Stație de lucru (computer) cu 2 monitoare, mouse, tastatură | Processor:; min. 16 nuclee, 33 MB cache, frecvență de bază/Turbo min. 3,1GHz/4,7GHz; TDP min. 200W; Cerințe minime pentru dezvoltator. Storage Controller:; suportă nivelurile RAID 0,1,10,5; Cerințe obligatorii ale dezvoltatorului. Storage:; 1x 2Tb NVMe PCIe 4.0 SSD; SSD sau HDD 7200 rpm min. Volum efectiv de 2x4TB efectiv 8 TB. Memory:; min. 128GB DDR5 în modul Dual Channel; min. 4x sloturi DIMM trebuie să fie libere pentru upgrade-uri viitoare; Cerințe minime pentru dezvoltator. Graphics Card:; Discrete min. 12 GB de memorie GDDR6 min. 2 interfețe DP sau HDMI (copatibil cu monitor); Cerințe recomandate dezvoltatorilor pentru a asigura moduri diferite în timpul testării. Expansion Slots:; slot PCIe Gen4 x16 trebuie să fie disponibil pentru componente suplimentare; Cerințe recomandate dezvoltatorilor pentru a asigura testarea cu alte componente ale sistemului. Network:; 1xGigabit Ethernet. Ports:; Minimum 8 x USB 3.2 ports (4x front, 4x rear); 1x RJ45; 2x HDMI and DisplayPort with multi-stream support (compatibil with 2 monitors); 1x line input, 1x line/headphone output, 1x headphone, 1x microphone; 1x Universal audio port; Cerințe recomandate pentru dezvoltatori pentru testarea în diferite mpduri de funcționare și de transfer de date. Power Supply:; Putere maximă posibilă disponibilă pentru modelul propus cu un randament minim de 90%; Capscitatea maximă a sursei de alimentare pentru a asigura o sarcină mai mică asupra sursei de alimentare în timpul condițiilor de funcționare de vârf. Keyboard:; Fără fir (Eng/Rus). Mouse; Laser fără fir. OS:; Microsoft Windows 11 Pro Plus for Workstation. Soft:; Microsoft Office 2021 Proffesional (Eng). Security:; TPM 2.0; Autentificare înainte de pornire; Senzor de intruziune cu activare prin BIOS; Porturile USB pot fi dezactivate/activate prin BIOS. Warranty:; Min. 2+1 ani de service oficial al producătorului (la fața locului, NBD,păstrați unitatea); numărul piesei trebuie să fie indicat. 2x Monitor:; Dimensiunea ecranului: min. 27”; Micro Bezel pentru 3 laturi. Rezoluție nativă:2560x1440@75Hz. Tip de panou: IPS. Luminozitate:min. 350 nits. Suport pentru monitor: Reglarea înălțimii - până la 150 mm. Interfețe: 4x porturi USB Type-A.x (min. 1x încărcare 7W li mai mult); porturi de afișare; HDMI si DP. Garanție:3 ani de service oficial al producătorului ( la fața locului, NBD); numărul piesei trebuie să fie indicat. Adjustable desk 2 display mounting arm (rotate, tilt,swivel), 17”-27”, up to 7 kg; Suport de birou pentru monitoare cu braț pentru două monitoare;Brațe din aluminiu sau metal; Suportă 2x monitoare (cu diagonală de 27”și greutate de min. 7 kg fiecare; Interval de înclinare 30* sau mai bine; Rotire pivotantă 90*; Suport VESA 100x100 mm; Managementul cablurilor încorporat; Suportul de montare a monitorului trebuie să suporte instalarea pentru suprafețe cu grosimi cuprinde între 15 și 80mm; Garanție 4 ani de garanție cu înlocuirea dispozitivelor defecte. Note; Tastatura, Mouse-ul și monitoarele trebuie să fie de același producător ca și stația de lucru; Toate componentele stației de lucru și software-ul trebuie să fie instalate și testate în fabrică; Cheile de activare ale sistemului de operare trebuie să fie scrie cu BIOS în fabrică; Software-ul de aplicație (MS Office) înpachetul original Microsoft sau instalat în fabrică și cu cheia de activare BIOS; trbeuie specificate numerele de referință ale tuturor pieselor și componentelor stației de lucru; includerea la stația de lucru a unor piese sau componente neoriginale este motiv de descalificare a ofertei; Garanția oficială pentru echipamentele oferite (stații de lucru, monitoare) trebuie să fie asigurată prin intermediul centrului oficial de service al producătorului de pe teritoriul Republicii Moldova; Pentru monitoaarele Braț Standuri trebuie să fie asigurat stocul necesar de produse pentru eventuala înlocuire în perioada de garanție. |
| 4 | Tableta | Type; Tablet. Processor; Intel, min. 10 nuclee, 12 MB cache, frecvență de bază/Turbo min. 1.7 GHz/4,7GHz; TDP min. 15W. Min. 12 Generation.. RAM; min. 16Gb. SSD; min. 512Gb. Display; min. Resolution: 2880 X 1920. Screen: min. 13”. Touch: min. 10-point multi-touch. Security; including face sign-in. Video/cameras; Front-facing camera with at least 1080p full HD video 10.0MP rear-facing autofocus camera with al least 1080p HD and 4k video. Connections; at least 1xUSB-C with USB 4.0,. Network and connectivity; Wi-Fi 6E: 802.11ax compatible. Minimum Bluetooth Wireless 5.1 technology or equivalent. Accessory. USB-C. Arc Mouse. Power supply. Keyboard; Signature Keyboard with Slim Pen Black; the latest generation compatible with proposed device; Pen integration: Slim Pen storage and charging; Magnetic interface; Accelerometer sensor;. Battery; Battery Capacity Min (WH) at least 45 Wh.. Up to 15.0 hours of typical device usage. Software; Microsoft Windows min. ver. 11 (preinstalled). Warranty; 36 month. Docking stations; Docking stations for tablet;. Designed for devices with USB-C® with USB 4.0/.. Connections: min. 1 front-facing USB-A 3.1; min. 1 front-facing USB-C; min. 2 rear-facing USB-A 3.1; min. 2 rear-facing USB-C; Warranty at least 1-year;. |