Annex nr.3

” Republican Veterinary Diagnostic Center” P.I.

Public Bidding

Procurement of transponders (microchips)

Technical Specifications

| Name of the goods | Description | Transponder tuning fork | Quantity |
| --- | --- | --- | --- |
| Transponders: - Horses | According to „TECHNICAL REQUIREMENTS” | 498040000030001-498040000032000 | 2000 |
| Total |  |  |  |

TECHNICAL REQUIREMENTS

characteristics of transponders (microchips) of the horses species

- General technical requirements

1.1 All horses, donkeys and descendants obtained by crossing them will be identified by applying an injectable transponder;

1.2 Injected transponders must meet the following requirements:

            - be a passive read-only transponder without an internal source of electricity, consisting of an integrated circuit - a microchip in which a unique numeric code is encoded - and an antenna, both included in a water-repellent material that can be of glass or ceramic;

            - the identification code can be read only when the transponder is activated by the signal transmitted by the reader;

            - the shell of the injected transponder is composed of complex substances, which ensure a fast encapsulation and prevent their migration;

            - the uniqueness of the identification code is guaranteed by the provider;

            - the unique identification code can not be copied or modified;

            - the unique identification code programmed only by the manufacturer during the realization of the transponder;

            - the uniqueness of the unique identification code verified by the manufacturer during the construction of the transponder.

- The shape and appearance of the transponders

2.1. Material - biocompatible glass;

2.2. Subcutaneous anti-migration treatment;

2.3. Color - transparent;

2.4. Shape - cylindrical.

- The size of the transponders

The transponders will have the following dimensions: - length - 11.5 mm (± 4.5%), diameter - 2.12 mm (± 4.5%).

- Resistance

            - water resistance - according to IP 68;

            - Vibration resistance - according to IEC 68 PART 2-47;

            - shock resistance - according to IEC 68 PART 2-29.

- Technical performance requirements

            - Use HDX or FDX-B technology in accordance with ISO 11784 and ISO 11785 standards;

            - Be legible with ISO 11785 compliant readers;

            - The reading distance should be: at least 12 cm, when reading with a portable reader and at least 50 cm, when reading with a stationary reader;

            - To transmit the correctly stored code in conditions of temperature between -20°C and + 40°C and humidity conditions between 40% rH and 80% rH;

            - Activation frequency - 134.2 kHz;

            - The code of the transponder (injected) for the identification of horses, donkeys and descendants obtained by crossing them will consist of 15 characters, having the following meaning:

- the first 3 characters indicate the country code, according to the ISO 3166 standard, for the Republic of Moldova this being 498;

- the next digit is 0 (zero);

- the following digit will represent the species code of the animal, this being 4;

- the following 10 characters after the country code (498), the digit 0 (zero) and the species code (4) represent the unique identification code.

Example

| Type of means of identification | Identification code | Interpretation of the identification code |
| --- | --- | --- |
| Transponder injected | 498040099999999 | 498 - country code; 0 - required number; 4 - species code for horses, donkeys and descendants obtained by crossing them; 00 – series number of 99999999 codes; 99999999 - animal serial number (unique code). |

- Implant / syringe

The injector used for the electronic identification of equidae must have the following characteristics:

- to be a disposable device;

- have an ergonomic format for precise and safe handling during implantation;

- be provided with specific components;

- with the role of preventing accidental, involuntary action;

- with the role of protection, isolation of the implant needle;

- be labeled with the unique registration code of the electronic identification tag.

- Technical requirements for packaging of transponders (microchips)

10 pieces in a box with the indication of the tuning fork (each transponder in a separate package with a transparent part and 6 adhesive labels inscribed with the unique code of the chip).

Technical drawing for separately packaged transponder:

              - The front part;				b) 	The back part.