|  | Anexa nr. 22 la Documentația standard nr._____ din “____” ________ 20___ Specificaţii tehnice |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [Acest tabel va fi completat de către ofertant în coloanele 2, 3, 4, 6, 7, iar de către autoritatea contractantă – în coloanele 1, 5,]. |
| Procedura de achiziție: ________din_________ |  |  |  |  |  |  |  |  |  |
| Obiectul achiziției: Achiziționarea comutatoarelor (REPETAT). |  |  |  |  |  |  |  |  |  |
| Nr. | Denumirea bunurilor/serviciilor | Denumirea modelului bunului/serviciului | Ţara de origine | Produ-cătorul | Specificarea tehnică deplină solicitată de către autoritatea contractantă |  | Specificarea tehnică deplină propusă de către ofertant | Standarde de referinţă |  |
|  | 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |  |
|  | Bunuri/servicii |  |  |  |  |  |  |  |  |
|  | LOTUL 1 |  |  |  |  |  |  |  |  |
| 1.1 | Comutator gestionat cu 28 porturi inclusiv 4 SFP (Small Form-factor Pluggable) |  |  |  | Conform Caietului de Sarcini (pct. 3) - 28x ports (24x 10/100/1000 Ethernet PoE+ ports; 4x 1/10Gbps SFP+-based ports) - Switching bandwidth: 128 Gbps - Forwarding rate: 95 Mpps - Advanced Layer 2 and basic Layer 3 features - Available PoE power: 195W. Support IEEE 802.3af PoE and IEEE 802.3at PoE+ (up to 30W per port) - Management: RMON, SNMP, TELNET, SSH, Console access - Network Security: Access control list (ACL L2/L3/L4, MAC,IP,Port based); Port security; IP Source Guard; BPDU guard; Storm control - Authentication: MAC/Certificate/Web-based 802.1X authentification, RADIUS server support - Supported protocols: SPAN/RSPAN, UDLD - 802.1Q VLAN, min 4k active VLANs; - Stacking up to 8 members; - IPv4/IPv6 static routes, Routing Information Protocol (RIP), 990 unicast routes, 1024 IPv4/IPv6 ACL - Standards: EEE 802.1D Spanning Tree Protocol, IEEE 802.1p, IEEE 802.1Q VLAN, IEEE 802.1w, IEEE 802.1X, IEEE 802.3ad, IEEE 802.3af and IEEE 802.3at, IEEE 802.3ah (100BASE-X single/multimode fiber only), IEEE 802.3x full duplex on 10BASE-T, 100BASE-TX, and 1000BASE-T ports, IEEE 802.3 10BASE-T, IEEE 802.3u 100BASE-TX, IEEE 802.3ab 1000BASE-T, IEEE 802.3z 1000BASE-X, RMON I and II standards, SNMP v1, v2c, and v3, IEEE 802.3az, IEEE 802.3af PoE and IEEE 802.3at PoE+ - IP Multicast and IGMP support - Unicast MAC addresses: 16000 |  |  |  |  |

Semnat:_______________ Numele, Prenumele:_____________________________ În calitate de: ________________

Ofertantul: _______________________ Adresa: __________________________