| Annex no. 22 to the Standard Documentation from Order of the Minister of Finance no. 115 of September 15, 2021 ## Technical proposal |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [This table will be completed by the bidder in columns 2, 3, 4, 6, 7, and by the contracting authority - in columns 1, 5]. |
| open auction |  |  |  |  |  |  |  |
| Procurement machine for metal processing. |  |  |  |  |  |  |  |
| Name of goods | Name of offered goods | Country of origin | Manufacturer | Full technical specification requested by the Contracting Authority |  | Full technical specification offered by bidder | Reference standards |
| 1 | 2 | 3 | 4 | 5 |  | 6 | 7 |
| Milling machine with 5 axes for metal processing. |  |  |  | Fully cast-iron bed Servomotors for direct axis movements with direct torque transmissions Hardened steel bearing blocks Double ball screws with preloaded nut Automatic lubrication system for ball guides and ball screws Thermal expansion compensation system High precision sensors of B and C axis angular position Internal autotransformer (354-480 V) Min. Working stroke: X - 600 mm, Y - 400 mm, Z - 400 mm Displacement from spindle nose to table: 102 - 508 mm Min. feed speed when cutting: 16.0 m/min Min. Axis rapid traverse 22.0 m/min Min. motor operation in X,Y-axis 14000 H, in Z-axis 18000 H B-axis (inclined): +120º to -35º Min. rotary speed 50 °/s Min. torque 2500 Nm Min. Braking torque 1200 Nm C axis (rotation): 360º Min. workpiece diameter: 450mm Min. rotary speed 50°/s Min. torque 2500 Nm Braking torque 1220 Nm Positioning accuracy in X, Y, Z axes: ± 0,005 mm Accuracy of repeatability of positioning along the X, Y, Z axes: ± 0.0025 mm Accuracy of positioning of the B and C axes: ±30 arc sec. Accuracy of repeatability of positioning of the B and C axes: ±15 arc sec. Air consumption: Min.113 l/min, operating pressure: 6.9 bar Power supply: 3ph, 380V, 50Hz Electrical power consumption: Min. 28 kVA Line voltage fluctuations max ± 5% Tool Changer: min - 30 tools Through-Spindle Coolant: min - 1000 psi (69 bar) Operating temperature range: from 5 ºC to 40 ºC |  |  |  |
| Turning machine with 2 axes for metal processing |  |  |  | Fully cast-iron bed Servomotors for direct torque transmission on axes Hardened steel bearing blocks of guides Double ball screws with preloaded nut Automatic lubrication system for ball guides and ball screws Thermal expansion compensation system Internal transformer (354-480 V) Min. workpiece diameter 500 mm Min. workpiece diameter (with BMT65 turret) 330 mm Min. machining length (depending on the fixture) 800 mm Distance between centers Min 800 mm Min. traverse of axes: X - 200 mm, Y ± 51 mm, Z - 800 mm Min. Rapid traverse axes: X,Y - 12 m/min, Z - 24 m/min Min. feed force axes: X - 18000 H, Y - 10000 H, Z - 22000 H Slide positioning accuracy ± 0,005 mm Carriage positioning repeatability ± 0,0025 mm Power supply: 3ph. 380V, 50Hz Min. Power consumption - 27 kVA Line voltage fluctuations - not more than ± 5% Compressed air consumption – Min. 113 l/min, operating pressure in the network - 6.9 bar Spindle rpm: min 3400 Spindle power: 30 hp (22.4 kW) spindle nose: A2-6 Coolant Tank: min 208 liter with 1 hp (0.75 kW) pump Automatic Tool Presetter, manual and automatic modes. User-friendly interface. Operating temperature range - 5 - 40 ºС |  |  |  |
| Signed:_______________ Name, Surname:_____________________________ Position: ________________ Bidder: _______________________ Bidder's Address: __________________________ |  |  |  |  |  |  |  |