CAIET DE SARCINI

pentru achiziția echipamentului Audio

| Nr lot | Cod CPV | Denumirea bunului | Unitat de măsură | Cant | Descrierea bunurilor și specificarea tehnică (cerințe minime tehnice) | Valoarea estimată |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 32330000-5 | Boxă audio | Bucată | 5 | Boxa activă cu accesorii Equipment: 12" and 1" speakers Biamped switching power supply amplifier with 700 Watt LF+200 Watt HF RMS Class-D Frequency range: 48 - 20,000 Hz Beam angle: 90° x 60° Maximum SPL cont / peak: 125 / 131 dB Integrated DSP with 4 switchable EQ presets Monitor angle : 12°, 40° and 55° Mic Line Switch HP filters XLR input and output Volume controller Plastic housing 2x M10 flying points 35 mm Flange for uprights | 33333.33 |
| 2 | 32330000-5 | Wireless In-Ear Monitor Kit | Bucată | 1 | Pro Wireless In-Ear Monitor Kit includes a wireless transmitter, 3 wireless bodypack receivers, 3 battery pack and one charger. Include a 1/4 wave antenna, a power supply. Wireless Transmitter (1pcs) Performance Transmission Range – 90 m RF Output Power – 10, 20, 30 mW RF Output Impedance – 50 Ohms (typical) Audio Input Connector Type – 1/4” TRS (6.35 mm) Polarity – Tip positive with respect to ring Configuration – Electronically balanced Impedance – 40 kOhms (actual) Nominal Input Level – Switchable, +4 dBu, -10 dBV Maximum Input Level - +4 dBu: +22 dBu; -10 dBV: +12.2 dBu Pin Assignments – Tip=hot, Ring=cold, Sleeve=ground Phantom Power Protection – Up to 60 VDC Audio Output Connector Types – 1/4” TRS (6.35 mm) Configuration – Electronically balanced Impedance – Connected directly to inputs General Antenna – 1/4 wave, detachable Power Requirement – 12 to 15 VDC, 260 mA maximum Wireless Bodypack Receiver (3pcs) RF Sensitivity – Active: 2.2 μV (at 20 dB SINAD) Image Rejection - > 90 dB Adjacent Channel Rejection - > 60 dB Intermodulation Attenuation - > 50 dB Blocking - > 60 dB Audio Output Power-80mW per output Minimum Load Impedance – 16 Ohms Headphone Output – 1.8” (3.5 mm), stereo Continuous Battery Life – 4 to 5.5 hours, AA batteries Dual Docking Recharging Station (1 Pcs) equivalent SBC200 Fits P3RA, P9R, P10R, UR5, ULX-D and QLX-D series For two Li-Ion batteries and/or hand-held or pocket transmitters Can be linked together Includes power supply for up to 4 units Battery pack (3 Pcs): Rechargeable Lithium-ion Battery (4.2V/1240mAh) Equivalent to SB900B | 50000.00 |
| 3 | 32330000-5 | Lavalier microphone system | Bucată | 2 | Dual Wireless System with two Bodypack Transmitters and two microphones (Set) Frequency range: S50 Receiver (1 pcs) Form Factor – Tabletop / Rackmount Mounting Options – Rackmount (with Included Hardware) Rackmount Size – 1 RU (Full Rack) Antenna – 2 x 1/4 Wave External, BNC (Rear-Mount) Number of Audio Channels – 2 Audio I/O – 2 x XLR 3-Pin Male Balanced Mic/Line Output 2 x 1/4” TRS Female Balanced Output Gain Range – 0 to +21 dB (3 dB Steps) Pad – 30 dB Audio Output Level - -18 to +42 dB (Balanced, Unbalanced) Frequency Response – 20 Hz to 20 kHz +1/-2 dB (Dependent on Mic) Impedance – 1/4” Outputs: 1.3 Kilohms (Balanced); 670 Ohms (Unbalanced) XLR Outputs: 400 Ohms (Balanced); 150 Ohms (Balanced) Approximate RF Sensitivity - -97 dBm Adjacent Channel Isolation - >70 dB AC/DC Power Adapter – 15 VDC at 600 mA, Center-Positive (Included) AC Input Power – 100 to 240 VAC, 50 / 60 Hz Display & Indicators – LCD (AF Level, Channel, Frequency, Group, RF Level, Transmitter Battery Status) Housing – Steel Transmitter (3 pcs) Form Factor – Bodypack RF Output Power – 1 / 10 mW Audio I/O – 1 x TA4/Mini XLR 4-Pin Male Unbalanced Input (Lockable) Gain Range – 0 +21 dB Sync Method – IR Antenna – 1/4 Wave External, Fixed Power Requirements – Battery Battery Type – 2 x AA; 1 x SB 903 Battery Charging Time – 1.25 to 5.5 Hours USB/Lightning Connectivity – None Display & Indicators – LCD (Battery Status, Channel, Group) Housing – ABS, Plastic Microphone (3 pcs) Form Factor – Lavalier Sound Field – Mono Capsule – Electret Condenser Polar Pattern – Omnidirectional Pad – None High-Pass Filter – None Indicators – None Windscreen – Foam Performance Frequency Range – 50 Hz to 17 kHz Maximum SPL – 125 dB SPL Impedance – 1800 Ohms Sensitivity - -40 dBV/Pa Dynamic Range 102.5 dB Equivalent Noise Level – 22.5 dB SPL A-Weighted Connectivity Output Connectors – 1 x TA4/Mini XLR 4-Pin Female (TQG) Cable Length – 1.22 m Physical Color – Black Mounting – Alligator Clip, Tie Tac Clip (Included) | 78333.33 |
| 4 | 32330000-5 | Microphone Podcast Bundle | Bucată | 3 | Microphone - Dynamic Speaker and Recording Microphone - Polar pattern: Cardioid - Frequency range: 50 – 20,000 Hz - Sensitivity -59.0 dB (1.12 mV) - Bass roll-off switch - Mid-boost switch - Impedance: 150 ohms - Shielding against electromagnetic interference - Swivel mount - XLR connector - Includes switch cover plate and windscreen Microphone Arm - Table microphone arm with cable guide - Cables up to 8 mm in diameter can be attached to the underside of the arms with a clip system - Aluminium construction - Locking centre joint - 3/8» Rhreaded connection with 5/8» adapter - Table clamp up to 56 mm or table feed-through up to 45 mm (20 mm hole) - Length upper arm: 43 cm - Length lower arm: 43 cm - Colour: Black - Microphone Cable Length: 3 m | 22500.00 |
| 5 | 32330000-5 | Handheld Microphone system | Bucată | 1 | The handheld microphone system offers two presenters or performers a dual-channel wireless handheld microphone system that’s dependable, and features crystal-clear, digital sound quality. Built around a rugged, dual-channel, single-rackspace receiver. System Wireless Technology – Digital UHF Number of RF Channel Frequencies – 1840 Included Transmitters – 2 x Handheld Diversity - True Digital Diversity RF Frequency Band – S50: 823 MHz – 865 MHz RF Channel Scanning – Auto-Scan Max Operating Range – 100 m Max Transmitters per Band – 32 Sample Rate/Resolution – 24-Bit Latency – 3.2 ms Dynamic Range – 120 dBA (Typical) Receiver (1 pcs) Receiver Type – Rackmount Mounting Options – Rackmount (with Included Hardware) Rackmount Size – 1 RU (Full Rack) Antenna - 2 x 1/4 Wave External, BNC (Rear-Mount) Number of Audio Channels – 2 Audio I/O – 2 x XLR 3-Pin Male Balanced Mic/Line Output; 2 x 1/4” TRS Female Balanced Output (Lockable) Gain Range – 0 to +21 dB (3 dB Steps) Pad – 30 dB Audio Output Level - -18 to +42 dB (Balanced, Unbalanced) Phantom Power Protection – On 1/4”, XLR Outputs Frequency Response – 20 Hz to 20 kHz +1/-2 dB (Dependent on Mic) Network I/O – 1 x RJ45 (10/100 Mb/s) Network Protocols – DHCP, Manual IP Adjacent Channel Isolation - >70 dB Power Requirements – AC/DC Power Adapter AC/DC Power Adapter – 15 VDC at 600 mA, Center-Positive (Included) AC Input Power – 100 to 240 VAC, 50 / 60 Hz Display & Indicators – LCD (AF Level, Channel, Group, RF Level, Transmitter Battery Status) Transmitter (3 pcs) Transmitter Type – Handheld RF Output Power - 1 / 10 mW Audio Input Level - 8.2 dBV Gain Range – 0 +21 dB Frequency Response – 20 Hz to 20 kHz (Dependent on Mic) Sync Method – IR Antenna - Internal Single-Band Helical Power Requirements – Battery Battery Type – 2 x AA; Display & Indicators – LCD (Battery Status, Channel, Group, Power) Housing - Aluminum Beta87A condenser capsule Microphone directional characteristic: Supercardioid | 41666.66 |
| 6 | 32330000-5 | Condenser Microphone | Bucată | 4 | Condenser Microphones Ideal for overhead, hihat, percussion, acoustic guitar General Specifications Audio frequency bandwidth 20 – 20000 Hz Equivalent noise level 18 dB-A Sensitivity 9 mV/Pa Signal to Noise 76 dB-A Preattenuation Pad -10; -20 dB Bass cut filter 75; 150 Hz Electrical impedance 200 Ohms Recommended load impedance 1000 Ohms Polar Pattern Cardioid Powering Interface Voltage 9 to 52 V Current 2 mA Audio Output Type Balanced XLR Gender Male Contacts 3-pin -CK 1 capsule -Includes windshield W90 and stand adapter SA60 | 15000.00 |
| 7 | 32330000-5 | EARPHONE | Bucată | 10 | EARPHONE -Transparent coiled tube (acoustic tube) -Application area: Stage – Theatre – Broadcast -Detachable cable -Cable:Length 1.5m (59») -Plugs: 3.5mm Right Angle Phone | 12500.00 |
| 8 | 32330000-5 | Lavalier Microphone | Bucată | 20 | Omnidirectional Lavalier Microphone with TA4F Connector -Modular design with interchangeable electret condenser capsule -Directional characteristic: Omnidirectional -4-Pin mini XLR (TQG) connector for wireless applications -Frequency response: 50 Hz – 17,000 kHz -Output impedance: 1.8 kOhm -Open circuit sensitivity: -45.5 dBV/Pa (10 mV) -Max. SPL (sound pressure level): 130.5 dB -Inherent noise (equivalent noise level): 28.0 dB (A) -Power supply: +5 V (1.5 – 10 V) -Cable length: 1.2 m Includes a windscreen that controls breath noise, 2 different clip-on tie pins and a bag | 33333.34 |
|  | Total: |  |  |  |  | 286666.66 |