TECHNICAL SPECIFICATION

| Microscope for the Examination of Travel Documents with Advanced Features |  |
| --- | --- |
| Functionalities and Basic Characteristics | The microscope shall be intended for the advanced examination of the authenticity of travel documents. General characteristics: Maximum dimensions: 455 × 455 × 605 mm; Maximum weight: 26 kg; Computer interface: USB 2.0. Manual or remote control: magnification; light sources; light filters. |
| Technical, Quality and Operational Characteristics | Magnification: minimum, through eyepiece: 95×; minimum, digital (on monitor): 250×. Video camera: type: CMOS (color); minimum resolution: 3 Mp (2048 × 1536 px). Light sources: All light sources shall be LED with brightness control. Incident mode: UV 365 nm, IR 870 nm, IR 950 nm, visible white, 455 nm, 470 nm, 505 nm, 530 nm, 590 nm, 627 nm; Transmitted mode: UV 365 nm, visible white, broadband infrared; Oblique mode: visible white (4 sources from different directions), IR (4 sources from different directions). Physical sources: minimum number of white lights: 35 pcs; minimum number of IR lights: 35 pcs. Filter sources (camera), minimum: bandpass 420–1100 nm; IR low-pass 700 nm; IR high-pass 650, 670, 700 nm. Operating systems supported: Windows 10/11, 32/64 bit; Certification: ISO 9001, CE; Documentation - The operating manual shall be provided in Romanian, English, and Russian; Warranty and After-Sales Service Warranty period: not less than 5 years from commissioning. All warranty servicing expenses shall be borne exclusively by the Supplier, including transportation of the equipment to and from the service center, if required. Additional Requirements: The Supplier shall ensure the provision of prototype equipment upon request. |