# ANUNȚ DE PARTICIPARE

privind achiziționarea Reutilarea studioului nr. 5 – Proiectoare LED

prin procedura de achiziție: Licitație deschisă

- Denumirea autorității contractante: IP Compania „Teleradio-Moldova”

- IDNO: 1004600050558

- Adresa: str. Miorița 1, mun. Chișinău

- Numărul de telefon/fax: 022 721047, 022 723352, 022 406935, 068760600, 068088984

- Adresa de e-mail și de internet a autorității contractante: trm@trm.md, www.trm.md

- Adresa de e-mail sau de internet de la care se va putea obține accesul la documentația de atribuire: e-mail: achizitii@trm.md, (documentația de atribuire este anexată în cadrul procedurii în SIA RSAP , MTender).

- Tipul autorității contractante și obiectul principal de activitate (dacă este cazul, mențiunea că autoritatea contractantă este o autoritate centrală de achiziție sau că achiziția implică o altă formă de achiziție comună): Instituție Publică

- Cumpărătorul invită operatorii economici interesați, care îi pot satisface necesitățile, să participe la procedura de achiziție privind livrarea/prestarea/ următoarelor bunuri /servicii/lucrări:

Lotul unic Proiectoare LED

Echipamente de scenă şi lumini

| Nr. d/o | Cod CPV | Denumirea bunului | Descrierea bunurilor și specificarea tehnică (cerinte minime tehnice) | C-te. | Unitatea de măsură |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1.1 | 44211100-3 | Schele, traverse 4 pct (350cm) | Feature: Straight Lenght : 350 cm Tailings: 2 Snake brace : 8,8 mm Tube pitch: 29 cm (outside) with conical connector set (Quicklock) included (4 x conical connector, 4 x 2 steel pins and 2 splints) Main tube diameter: 50 mm Wall thickness: 2mm Brace diameter: 20mm Material: AlMgSi F31 TÜV tested | 18 | buc |  |
| 1.2 | 44211100-3 | Schele, traverse 4 pct (250cm) | Feature: Straight Lenght : 250 cm Tailings: 2 Snake brace : 8,8 mm Tube pitch: 29 cm (outside) with conical connector set (Quicklock) included (4 x conical connector, 4 x 2 steel pins and 2 splints) Main tube diameter: 50 mm Wall thickness: 2mm Brace diameter: 20mm Material: AlMgSi F31 TÜV tested | 4 | buc |  |
| 1.3 | 44211100-3 | Schele, traverse 4 pct (150cm) | Feature: Straight Lenght : 150 cm Tailings: 2 Snake brace : 8,8 mm Tube pitch: 29 cm (outside) with conical connector set (Quicklock) included (4 x conical connector, 4 x 2 steel pins and 2 splints) Main tube diameter: 50 mm Wall thickness: 2mm Brace diameter: 20mm Material: AlMgSi F31 TÜV tested | 4 | buc |  |
| 1.4 | 44211100-3 | Schele, traverse 4 pct (100cm) | Feature: Straight Lenght : 100 cm Tailings: 2 Snake brace : 8,8 mm Tube pitch: 29 cm (outside) with conical connector set (Quicklock) included (4 x conical connector, 4 x 2 steel pins and 2 splints) Main tube diameter: 50 mm Wall thickness: 2mm Brace diameter: 20mm Material: AlMgSi F31 TÜV tested | 4 | buc |  |
| 1.5 | 44211100-3 | Schele, traverse 4 pct (90° 3-Way Corner) | Feature: Special element, 90° 3-Way Corner Lenght : 50 cm Tailings: 3 Snake brace : 8,8 mm Tube pitch: 29 cm (outside) with conical connector set (Quicklock) included (8 x conical connector, 8 x 2 steel pins and 2 splints) Main tube diameter: 50 mm Wall thickness: 2mm Brace diameter: 20mm Material: AlMgSi F31 TÜV tested | 4 | buc |  |
| 1.6 | 44211100-3 | Schele, traverse 4 pct (3-Way T-Piece) | Feature: Special element, 3-Way T-Piece Lenght : 50 cm Tailings: 3 Snake brace : 8,8 mm Tube pitch: 29 cm (outside) with conical connector set (Quicklock) included (8 x conical connector, 8 x 2 steel pins and 2 splints) Main tube diameter: 50 mm Wall thickness: 2mm Brace diameter: 20mm Material: AlMgSi F31 TÜV tested | 10 | buc |  |
| 1.7 | 44211100-3 | Schele, traverse 4 pct (Floor Plate) | Feature: Floor Plate Tailings: 1 Tube pitch: 29 cm (outside) with half-conical connector set (quick coupler) (4 x 1 steel pin and 1 splint) Material: AlMgSi F31 TÜV tested | 8 | buc |  |
| 1.8 | 44211100-3 | Grip | Clamp for pipe to rail connection and rail -Clamp to connect pipe to rail parallel or rectangular.For easy installation of rail system to an existing studio pipe grid Ø MAX 50, echivalent cu FF3602. -Rail systems in each length and shape are possible by connecting 2 or more standard rails, length: 4000mm, hight: 610mm, width: 345mm, echivalent cu Echivalent cu FF6004 | 32 | Buc |  |
| 1.9 | 44211100-3 | Grip | Double carriage for Rail 60. Double carriage with spring-loaded brake pads on each section. To release the brake pull down on wire with operating pole. Used to connect a mobile rail to a fixed rail. Echivalent t cu FF3235 | 37 | buc |  |
| 1.10 | 44211100-3 | Grip | Carriage with brake and 16mm spigot. Spring-loaded brake pad holds carriage in position. To release the brake pull down on wire with operating pole. Compatible with Rail 60. Echivalent cu FF3229 | 36 | buc |  |
| 1.11 | 44211100-3 | Grip | Alignment connector for rail 60. Two steel plates to connect rails end to end. Hardware includedNot suitable for moving rails. Echivalent cu FF3208 | 8 | buc |  |
| 1.12 | 44211100-3 | Grip | End stop (kit of 4). Mount the end stops in the holes at the end of each rail to prevent carriages leaving the rail. Echivalent cu FF3661 | 24 | buc |  |
| 1.13 | 44211100-3 | Grip | Rail 60 cable carriage. Made to support up to 3 electrical cables from 8 to 16mm diameter on rail type 60. Cables held by standard cable tie, not supplied Echivalent cu FF3226V | 40 | buc |  |
| 1.14 |  | Instalare | Lucrările de montare, precum și operațiunile de instalare, ajustare, testare; Materiale și elemente pasive/active necesare pentru montarea construcției. Fixarea construcție așezat orizontal/vertical (podea/perete). Sistemul va fi predat la cheie; Garanția minim 3 ani. | 1 | buc |  |
| Echipamente de iluminare |  |  |  |  |  |  |
| Nr. d/o | Cod CPV | Denumirea bunului | Descrierea bunurilor și specificarea tehnică (cerinte minime tehnice) | C-te. | Unitatea de măsură |  |
| 2.1 | 31520000-7 | LED Panel Bi-color | Bi-Color Ultra Soft LED Features: DMX control with LCD display High performance light source with excellent colorimetry Bi-colour Ultra Soft LED Studio Light Can be mounted horizontally or vertically Dimmer control Barn doors included Rear heat dissapation to keep the light cool Control brightness, colour and colour temperature Specifications: Brightness Control: DMX and Dimmer Colour Temperature: 3200K - 5600K Remote Control Type: DMX CRI (Ra): 95 Dimensions: 710 x 330mm Light Type: Studio AC Input Power: 100 to 240 VAC, 50 / 60 Hz Power: 144 (Halogen Equivalent 1008 Watts) Mount conection 16MM BUSHING Echivalent cu LG-T1440MC | 32 | buc |  |
| 2.2 | 31520000-7 | LED Panel Bi-color | Bi-Color Ultra Soft LED Features: DMX control with LCD display High performance light source with excellent colorimetry Bi-colour Ultra Soft LED Studio Light Can be mounted horizontally or vertically Dimmer control Barn doors included Rear heat dissapation to keep the light cool Control brightness, colour and colour temperature Specifications: Brightness Control: DMX and Dimmer Colour Temperature: 3200K - 5600K Remote Control Type: DMX CRI (Ra): 95 Dimensions: 1375 x 330 mm Light Type: Studio AC Input Power: 100 to 240 VAC, 50 / 60 Hz Power: 288 (Halogen Equivalent 2016 Watts) Mount conection 16MM BUSHING Echivalent cu LG-T2880LMCIII | 2 | buc |  |
| 2.3 | 31520000-7 | Proiector angular LED | LED Ellipsoidal Spot is a high-powered LED-based fixture with a single 230W warm white LED light source that exceeds a Tungsten 750W light source and is well suited for film studios, broadcast applications, theaters, and more. The fixture can be retrofitted to lamp-based ellipsoidals without losing the investment into a current dimming system. Optical Include Ellipsoidal HD Lens Tube (36°, Black) (echivalent cu OHDLENS36) Light Source: 1 LEDs (WW) 230 W, (3.4 A) Life Expectancy: 100,000 Hours Color Temperature (at Full): 3334 K CRI: 97 PWM Frequency: 600, 1200, 2000, 4000, 25,000 Hz Dynamic Effects Dimmer: Electronic or External Dimmer Rack Shutter/Strobe: Electronic Focus: Manual Strobe Rate: 0 to 18 Hz Dim Modes: 5 Dim Curves: 4 Control Control Protocol: DMX, RDM, Dimmer DMX Channels: 1, 2, or 3 Modes/Personalities: 3 Personalities (UNO, DOS, STD.P) Connections Power Input: Neutrik powerCON Power Output: Neutrik powerCON Data Connectors: 3- and 5-Pin XLR Power Connection: Edison Plug to Neutrik powerCON Cable Length (Power): 1.5 m Power Input Voltage: 120/230 VAC, 50/60 Hz (Switchable) Power Linking: 6 Units at 120 V; 12 Units at 230 V Color: Black Echivalent cu ED-200WW | 6 | buc |  |
| 2.4 | 31520000-7 | Proiector LED | A beautifully soft and flat field of warm white light with high CRI and a motorized zoom range of 16° to 65°, making it a versatile LED replacement for aging incandescent six-inch Fresnels. A virtually quiet operation and smooth 16-bit dimming control make it a great choice for theatrical and/or studio applications. Warm white Fresnel with output comparable to a 6”, 650 W Fresnel Soft and even field of light 16-bit dimming resolution for smooth fades Virtually silent operation for use in any situation Motorized zoom range between 16° to 65°, manually or via DMX Optical Light Source: 1 LED (WW) 70 W, (3.6 A), 50,000 hours life expectancy Color Temperature (at full): 3096 K CRI: 97 Beam Angle: 16° to 48° Field Angle: 25° to 65° Zoom Range: 16° to 65° Lumens (25°): 2,787 Lumens (65°): 4,266 Illuminance (25°): 1,290 lux @ 5 m Illuminance (65°): 252 lux @ 5 m Selectable PWM: 600 Hz, 1,200 Hz, 2,000 Hz, 4,000 Hz, 6,000 Hz, 25,000 Hz Control Protocol(s): DMX, RDM DMX Channels: 1, 2, 2 or 5 Modes/Personalities: 4 personalities (1, 2, 2, 5) Connections Power Connection: Edison (Local) plug to Neutrik powerCON Power Input: Neutrik powerCON Power Output: Neutrik powerCON Data Connectors: 3- and 5-pin XLR Cable Length (power): 5 ft (1.5 m) Dynamic Effects Dimmer: Electronic Shutter/Strobe: Electronic Zoom: Motorized Strobe Rate: 0 to 30 Hz Dim Modes: 4 Include Barndoor and full-size half-coupler Echivalent cu F-145WW | 5 | buc |  |
| 2.5 | 31527260-6 | Pantograf | PANTOGRAPH TOP 2C W/Type 4 Spring (From 3kg to 8kg COUNTER BALANCED WEIGHT) TECHNICAL SPECIFICATIONS Lifting rope - 2mm wire rope Safety factor rope - 12 Self weight - 4.6kg Max working load - 18kg Closed length - 27cm Max extension - 200cm TOP ATTACHMENT - 16MM - 5/8” BUSHING (Echivalent cu FF3548) BOTTOM ATTACHMENT - 16MM - 5/8” PIN (Echivalent cu FF3546) Echivalent cu FF3512A86 | 19 | buc |  |
| 2.6 | 31527260-6 | Pantograf | PANTOGRAPH TOP 2C W/2 Type 4 Springs (From 8kg to 14kg COUNTER BALANCED WEIGHT) TECHNICAL SPECIFICATIONS Lifting rope - 2mm wire rope Safety factor rope - 12 Self weight - 4.6kg Max working load - 18kg Closed length - 27cm Max extension - 200cm TOP ATTACHMENT - 16MM - 5/8” BUSHING (Echivalent cu FF3548) BOTTOM ATTACHMENT - 16MM - 5/8” PIN (Echivalent cu FF3546) Echivalent cu FF3512B86 | 6 | buc |  |
| 2.7 | 31527260-6 | Clema | Aluminum Coupler with 5/8" (16 mm) Spigot Load capacity: min 50 kg For pipes 42-52 mm (16 mm) Baby spigot Echivalent cu C4462 MP | 21 | buc |  |
| 2.8 | 31527260-6 | Bară DMX și priza LED | Bar for LED-PARs or other DMX spotlights Crossbar for the distribution of electrical power and DMX signals to up to five spotlights Inputs: –Lockable input socket (Power Twist) for power supply .–DMX input socket Outputs: –5 safety chassis sockets with hinged lid, individually fused –6 DMX output sockets –Lockable output socket (Power Twist). Includes 2 attachment screws Power supply cable (Power Twist) included Echivalent cu LED Power & DMX Bar | 10 | buc |  |
| 2.9 | 31527260-6 | DMX Cablu | The sturdy digital 110 ohm cable for the transmission of digital DMX light control signals. Inner conductor: 2 Inner conductor: 0,22 mm² Shielding: Copper spiral shielding Jacket material: PVC | 300 | m |  |
| 2.10 | 31527260-6 | DMX mufa | XLR, 3-pole, male | 60 | buc |  |
| 2.11 | 31527260-6 | DMX mufa | XLR, 3-pole, female | 15 | buc |  |
| 2.12 | 31527260-6 | DMX mufa | XLR, 5-pole, male | 5 | buc |  |
| 2.13 | 31527260-6 | DMX mufa | XLR, 5-pole, female | 50 | buc |  |
| 2.14 | 31527260-6 | Fisa | Cable Plug (Male), Connector cross-sections up to 3G 2.5 mm², Rated voltage: 250 ~, Rated current: 16 A, | 50 | buc |  |
| 2.15 | 31527260-6 | Priza | Cable Plug (Female), Connector cross-sections up to 3G 2.5 mm², Rated voltage: 250 ~, Rated current: 16 A, | 50 | buc |  |
| 2.16 | 31527260-6 | Cablu | 3-Wire: 3x 1.5 mm², Shielding Material Rubber, Rated voltage: 250 ~ | 300 | m |  |
| 2.17 | 31527260-6 | Cutie | Cutie pentru 1x36 automate (Extern), Grad de protective - IP20, Mod de instalare - Extern | 1 | buc |  |
| 2.18 | 31527260-6 | Întrerupător automat | Întrerupător automat 3P 50A (C) 4,5ka, Număr de poli – 3, Curent nominal - 50 А, Tipul de tensiune, A - AC (alternativ), Tensiunea nominala - 230/400 V, Gradul de protecţie - IP20 | 1 | buc |  |
| 2.19 | 31527260-6 | Întrerupător automat | Întrerupător automat 1P 16A (C) 4,5ka, Număr de poli – 1, Curent nominal - 16 А, Tipul de tensiune, A - AC (alternativ), Tensiunea nominala - 230/400 V, Gradul de protecție - IP20 | 15 | buc |  |
| 2.20 | 31527260-6 | Corp de iluminat | Corp de iluminat led industrial 200W, Puterea (W) - 200 W Flux luminos (Lm) - 22000 Lm Temperatura culoare (K) - 6500 K Unghi fascicul luminos (°) - 120° Tensiune (V) - 180-245 V Frecvență - 50…60 Hz Indice de redare a culorilor (Ra) - R ≥ 80 Grad de protecție (IP) - IP65 | 10 | buc |  |
| 2.21 |  | Instalare | Instalarea și punerea în funcțiune a sistemului de iluminare. Materialele și elementele pasive/active necesare pentru interconectarea echipamentelor și punerea în funcțiune a acestora. Lucrările de montare, precum și operațiunile de instalare, ajustare, testare; Sistemul va fi predat la cheie. Garanția hardware minim 2 ani; | 1 | buc |  |
|  | Total: |  | 2025916,67 |  |  |  |

- Admiterea sau interzicerea ofertelor alternative: Nu se admite

- Termenii și condițiile de livrare/prestare/executare solicitați: Livrarea bunurilor se va realiza în termen de 50 de zile de la intrarea în vigoare a contractului, iar prestarea serviciilor în decurs de 10 de zile de la solicitarea Cumpărătorului.

- Termenul de valabilitate a contractului: 31.12.2021

- Scurta descriere a criteriilor privind eligibilitatea operatorilor economici care pot determina eliminarea acestora și a criteriilor de selecție; nivelul minim (nivelurile minime) al (ale) cerințelor eventual impuse; se menționează informațiile solicitate (DUAE, documentație):

| Nr. | Denumirea documentului/cerințelor | Mod de demonstrare a îndeplinirii cerinţei: | Obl. |
| --- | --- | --- | --- |
| 1 | Oferta financiară | Formularul (F 3.1) semnat cu semnătură electronică avansată calificată | Da |
| 2 | Specificații tehnice | Formularul ( F 4.1) semnat cu semnătură electronică avansată calificată | Da |
| 3 | Specificații de preț | Formularul ( F 4.2) semnat cu semnătură electronică avansată calificată | Da |
| 4 | Garanția pentru ofertă | În cuantum de 1% din valoarea ofertei fără TVA Garanția pentru ofertă sau formularul garanției bancare (F3.2), Semnat cu semnătura electronică avansată calificată | Da |
| 5 | Formular standard al Documentului Unic de Achiziții European (DUAE) | Semnat cu semnătură electronică avansată calificată | Da |
| 6 | Dovada înregistrării persoanei juridice, în conformitate cu prevederile legale din țara în care ofertantul este stabilit | Certificat/decizie de înregistrare a întreprinderii; extras din Registrul de Stat al persoanelor juridice din care sa rezulte adresa actuala și obiectul de activitate al societății –Semnate cu semnătură electronică avansată calificată | Da |
| 7 | Certificat privind lipsa sau existenta restantelor fata de bugetul public național | Semnat cu semnătură electronică avansată calificată | Da |
| 8 | Informații generale despre ofertant | Formularul informativ despre ofertant, semnat cu semnătură electronică avansată calificată | Da |
| 9 | Garanția de bună execuție | Garanția de bună execuție, completată în conformitate cu formularul F 3.3. – Original sau transfer pe contul autorității contractante. Notă: Va fi prezentată la semnarea contractului de către operatorul econmomic învingător. | Da |
| 10 | Raport financiar | Ultimul raport financiar - Semnat cu semnătură electronică avansată calificată | Da |
| 11 | Certificat de atribuire a contului bancar eliberat de banca deţinătoare de cont după data punerii în aplicare a codurilor IBAN | Varianta scanată de pe original confirmată prin semnătura electronică avansată calificată | Da |
| 12 | Numărul de înregistrare | Numărul de înregistrare din „Lista producătorilor” conform Regulamentului privind deșeurile de echipamente electrice și electronice aprobat prin H.G. nr. 212 din 07.03.2018 | Da |
| 13 | - DECLARAŢIE - privind confirmarea identității beneficiarilor efectivi și neîncadrarea acestora în situația condamnării pentru participarea la activităţi ale unei organizaţii sau grupări criminale, pentru corupţie, fraudă şi/sau spălare de bani. | Întocmită conform Ordinului Ministrului Finanțelor nr. 145 din 24 noiembrie 2020. Varianta scanată de pe original confirmată prin semnătura electronică avansată calificată Notă: Va fi prezentată de către operatorul econmomic învingător în termen de 5 zile de la data comunicării rezultatelor procedurii de achiziție publică | Da |
| 14 | Minim ani de experienţă specifică în livrarea bunurilor şi/sau serviciilor similare | Ofertantul va avea minim 5 ani de experienţă specifică în livrarea bunurilor similare. Pentru demonstrarea îndeplinirii acestei cerinţe opertaorul economic va anexa acte doveditoare (contracte executate sau in derulare/ scrisori de recomandare, etc.) Varianta scanată de pe original și aplicată semnătura electronică avansată calificată. | Da |

IMPORTANT: În conformitate cu Legea privind achizițiile publice nr.131 din 03.07.2015 și anume, art.65 alin. (4), prezentarea ofertei presupune în mod obligatoriu depunerea „ofertei tehnice (F4.1), ofertei financiare (F4.2), formularului DUAE și garanția pentru ofertă”.

- Tehnici și instrumente specifice de atribuire (dacă este cazul specificați dacă se va utiliza acordul-cadru, sistemul dinamic de achiziție sau licitația electronică): Licitație electronică,

3 runde, valoarea pasului minim 1%

- Criteriul de evaluare aplicat pentru adjudecarea contractului: Prețul cel mai scăzut pe lot și corespunderea cerințelor caietului de sarcini.

- Termenul limită de depunere/deschidere a ofertelor: Conform SIA RSAP

- Adresa la care trebuie transmise ofertele sau cererile de participare:

Ofertele sau cererile de participare vor fi depuse electronic prin intermediul SIA RSAP

- Termenul de valabilitate a ofertelor:  60 de zile

- Locul deschiderii ofertelor: SIA RSAP Mtender

Ofertele întîrziate vor fi respinse.

- Limba sau limbile în care trebuie redactate ofertele sau cererile de participare: limba de stat. Operatorii economici nerezidenți vor prezenta înscrisurile însoțite de traducere în limba română sau rusă.

- Respectivul contract se referă la un proiect și/sau program finanțat din fonduri ale Uniunii Europene: Nu

- Denumirea și adresa organismului competent de soluționare a contestațiilor:

Agenția Națională pentru Soluționarea Contestațiilor

Adresa: mun. Chișinău, bd. Ștefan cel Mare și Sfânt nr.124 (et.4), MD 2001;

Tel/Fax/email: 022-820 652, 022 820-651, contestatii@ansc.md

- Data publicării anunțului de intenție sau, după caz, precizarea că nu a fost publicat un astfel de anunţ: BAP nr. 14 din 23.02.2021

Conducătorul grupului de lucru:  						 Sorocean Tatiana