SPECIFICAȚII TEHNICE

| Nr. lot | Tip |  |  | Descrierea | Cant. | Garanția, ani | Pret unitar | Suma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Calculator staționar |  | Carcasa | Black, ATX/MicroATX, USB 3.0x1, USB 2.0x1, HD Audio x1, mic x1, 3x3,5 drive bays, 1x2,5 drive bays, 1x5,25 drive bays, buclăpentrulacătpeambeleprti. | 70 | 3 | 6265 | 438550 |
|  |  |  | Blocdealimentare | ATX 12 V, v2.31, 650W, 220-240V, 20+4 pin 1PCS/CPU 12V 4+4pin, 4 xSata, 4xPCI-E, 1x120 mm cooler/Active PFC, Inteligent fan speed control, PWM auto fan speed temp-controlled, 21 dBA. |  |  |  |  |
|  |  |  | Placamamă | ATX, 1151, Suport windows 7/10 x32/x64, DDR4 2xSlots, 1xVGA, 1xDVI, 1xHDMI, 2xPCI-E X1, 1xPCI-E x16, PCI, 4xSATAIII 6.0 Gb/s, 2xUSB 3.1, 4xUSB2.0/GLAN RJ-45 with led/ Audio HD, all solid state capacitors. |  |  |  |  |
|  |  |  | Procesor | 2x3.3GHz, 3Mb, 1151, 8GT/s, HD Graphics, 14 nm, 54W, DDR4, DDR3L, support ECC, (Dual core)/Suport windows 7/10 x32/x64. |  |  |  |  |
|  |  |  | Cooler CPU | 1151/ fan 120x25 mm/900-1600 rpm/55,5 cfm/125 W/ 17.8-26,1 dBA/ input 1,56 W. |  |  |  |  |
|  |  |  | RAM | 4 GB DDR4 2400 MHz PC19200. |  |  |  |  |
|  |  |  | HDD | HDD 500GB, SATA-III (6 Gb/s), 7200RPM, 32MB, 3,5". |  |  |  |  |
| 2 | Calculator staționar asamblat la uzina producătorului |  | Carcasa | Carcasaşicomponente interne asamblate la uzinaproducătoruluiDestkopBlack .tower, 5.25” ODD bays, 3.5” external bays, internal bays 3.5”, 1x2.5”, USB2.0, USB3.0, audio jack. | 18 | 3 | 16440 | 295920 |
|  |  |  | Bloc de alimentare | 300 W. |  |  |  |  |
|  |  |  | Placamamă | 1xPCI, 2x PCI Express x1, 1xPC Express x16, DVI, VGA, 4xUSB 2.0, 4x USB 3.1, Audio IN/OUT, Gigabit Ethernet, No WLAN , No ODD. |  |  |  |  |
|  |  |  | Procesor și Cooler CPU | CPU 2x3.5 Ghz, 3 MB, 1151, Cooler CPU. |  |  |  |  |
|  |  |  | RAM | Memory 4GB (Up to 32 GB of Dual-channel) DDR4 2400 MHz. |  |  |  |  |
|  |  |  | HDD | 1 ТВ 3.5-inch 7200 RPM. |  |  |  |  |
|  |  |  | Monitor (cuboxeincluse) | În set cu blocul de sistemaasamblat de la uzinaproducătorului. 21,5” black 1920x1080 full HD, twisted nematic+film, LED, 200 cd/m2, mat, 5 ms, 100000000:1, unghiulobservăriipelaturile 100/70, 1xD-sub, 1xDVI, 1xHDMI, ajustareapoziţieiecranuluiînclinare, 22 W, 60 Hz, cablu VGA, DVI, cablu de alimentare, garanția 3 ani (VGA, DVI, HDMI). |  |  |  |  |
|  |  |  | Tastieră | Mouse, keyboard încomplet cu blocul de sistemasamblat de la uzinaproducătorului. |  |  |  |  |
|  |  |  | Mouse |  |  |  |  |  |
|  |  |  | Sistemdeoperare licențiat | Microsoft Windows 10 pro x32/x64, En. |  |  |  |  |
| 3 | Laptop | Laptop |  | CPU min 8 MB Cache SmartCache, up to 4.00GHz, # of Cores 4, # of Threads 8. RAM min 8 GB DDR4. Stocare: min SSD 256 GB sau min 128 GB SSD + HDD min 500GB. Ecran: 15,6”,Full HD 1920x1080. Unitateoptică: DVD-RW. Porturi: min 2 X USB 3.0, GLAN RJ 45, HDMI, Audio. | 3 | 3 | 16940 | 50820 |
|  |  | Sistem de operare licențiat |  | Microsoft Windows 10 pro x32/x64, En. |  |  |  |  |
| 4 | Monitor (cuboxeincluse) | Monitor BLACK 23,8", IPS-ADS LED, 250 cd/m2, 1000:1, 10M:1, 178/178 grade vizibilitatea, 5 ms, audio inclus 2x2 W, VGA, DVI, HDMI, Cablu VGA, Cablu DVI, Cablu HDMI, bloc de alimentareîncorporat. |  |  | 70 | 3 | 3000 | 210000 |
| 5 | Tastieră | Keyboard BLACK ErgoComfort, RoundEdge Keycaps PS/2 (US+RO+RU), lungime fir 180 cm, ajustareînălţime, compatible Win 7/8/10, rezistență la umeditate. |  |  |  |  | 100 | 7000 |
| 6 | Mouse | Black, Optical Wheel 1000 dpi, 2 taste+Scroll button, PS/2, lungime fir 180 cm. |  |  |  |  | 100 | 7000 |
| 7 | UPS | Line Interactive/600w/1000va/220-240V/ AVR/CPU controled, battery 12v/7A, 2puteri de ieşire/alarmăsonoră, protecţie la suprasarcină, protecţie la puls de înaltătensiune, protecţieîmpotrivascurtcircuitelor. |  |  |  |  | 1000 | 70000 |
| 8 | Surge | Surge Protector 5 Sockets, 5 m, BLACK, plăcibimetalice, protecție la suprasarcină, corpantiincendiar. |  |  |  |  | 100 | 7000 |
| 9 |  | Surge Protector for RACK, out: min 6 Sockets euro, in: IEC-320-C14 (male). |  |  | 10 |  | 300 | 3000 |
| 10 |  | Surge Protector for RACK, out: min 6 Sockets euro, in: euro plug. |  |  | 10 |  | 300 | 3000 |
| 11 | Imprimantă | alb-negru/tehnologie laser, A4, 60-163 g/m2/18 foi/min/posibilitate de printarepe carton, foifotografice, etichete, etc/32 MB</port USB2.0/compatibilitatea windows 7/10 x32/x64, 600x600dpi, <50db, black. |  |  | 5 |  | 3200 | 16000 |
| 12 |  | color/tehnologie laser, A4, 40000, foi\luna, 4 culori, 600x600 dpi, 20 foi/min, 60-220 g/m2, posibilitate de printarepe carton, foifotografice, etichete, 4 cartușe, 16MB, ethernet, USB 2.0, 430 W, 52 db. Suplimentar set de cărtușe de rezervă. |  |  | 3 |  | 7300 | 21900 |
| 13 | Scaner | Scaner A3, overhead, simplex, color, lens reduction optics/color CCDx1,White LED+lens illumination x2, 285 to 218 dpi(horizontal scaning), 283 to 152 dpi(vertical scaning), Scanning speed(A3 landscape) 3 seconds, USB 2.0/3.0, compatibil cu windows 7/10 x32/x64. |  |  | 1 |  | 1900 | 1900 |
| 14 |  | Scaner, USB 2.0, compatibil Windows 7/10 x32/x64, A4, 4800x4800, 19200x19200 dpi, 48 bit/24 bit, alimentare USB, Posibilitate de scanare a dosarelor de dimensiunimari. |  |  | 3 |  | 18000 | 54000 |
| 15 | Boxe | USB-Power, black, 2x1,2w, 30-20kHz, 65dB, BlueLighting. |  |  | 20 |  | 450 | 9000 |
| 16 | Switch | Port Standards & Functions IEEE 802.3 10BASE-T Ethernet, IEEE 802.3u 100BASE-TX Fast Ethernet, IEEE 802.3ab 1000BASE-T Gigabit Ethernet, 802.3ae 10 GbE, IEEE 802.3x Flow Control for Full-Duplex Mode, Auto-negotiation; Number of Ports 48 10/100/1000 Mbps, 2 Gigabit SFP, 2 10G SFP+; Network Cables UTP Cat. 5, Cat. 5e (100 m max.) EIA/TIA-568 100-ohm STP (100 m max.) Full/Half Duplex Full/half duplex for 10/100 Mbps and Gigabit speed Media Interface Exchange Auto or configurable MDI/MDIX Performance Switching Capacity 140 Gbps Transmission Method Store-and-forward MAC Address Table 16,000 entries per device MAC Address Update Up to 512 static MAC entries Enable/disable auto-learning of MAC addresses Maximum 64 bytes Packet Forwarding Rate 104.16 Mpps Packet Buffer Memory 3 MB per device Software Features Stackability • Virtual Stacking Support • D-Link Single IP Management • Up to 32 devices per virtual stack • Up to 20G stacking bandwidth • Physical Stacking • Supports Duplex Chain/Ring topology • Up to 40G stacking bandwidth full duplex • Up to 6 units per stack L2 Features• MAC Address Table: 16K • Flow Control • 802.3x Flow Control • HOL Blocking Prevention • Jumbo Frame up to 9000 Bytes • IGMP Snooping • IGMP v1/v2 Snooping • IGMP v3 awareness • Supports 512 IGMP groups • Supports 128 static multicast addresses • IGMP per VLAN • Supports IGMP Snooping Querier • Host-based IGMP Snooping Fast Leave • MLD Snooping • Supports MLD v1/v2 awareness • Supports 512 groups • Supports 128 Static Multicast Addresses • Per VLAN MLD Snooping • Host-based MLD Fast Leave • MLD Snooping Querier • Spanning Tree Protocol • 802.1D STP • 802.1w RSTP • 802.1s MSTP • Loopback Detection • 802.3ad Link Aggregation • Max.32 groups per device/8 ports per group • Port Mirroring • Support 4 mirroring groups • One-to-One, Many-to-One • Supports Mirroring for Tx/Rx/Both • Multicast Filtering • Forwards all unregistered groups • Filters all unregistered groups • LLDP, LLDP-MED VLAN • 802.1Q Tagged VLAN • 4K VLAN Groups • Configurable VID: 0~4094 • Asymmetric VLAN • Auto Voice VLAN • Auto Surveillance VLAN Quality of Service (QoS) • 802.1p Quality of Service • Queue Handling • Strict • Weighted Round Robin (WRR) • 8 queues per port • Bandwidth Control • Port-based (Ingress/Egress, min. granularity for 10/100/1000 BASE-T ports is 64 Kb/s) • CoS based on • 802.1p priority • VLAN • MAC address • Ether type • IP address • DSCP • Protocol type • TCP/UDP port number • DSCP of IPv6 Traffic Class • IPv6 flow label L3 Features • ARP • 256 Static ARP • Supports Gratuitous ARP • IPv6 Neighbour Discovery (ND) • Default Routing • Static Routing • 64 IPv4 Static Route Entries • 32 IPv6 Static Route Entries Access Control List (ACL) • ACL based on • 802.1p priority • VLAN • MAC address • Ether type • IP address • DSCP • Protocol type • TCP/UDP port number • DSCP of IPv6 Traffic Class • IPv6 flow label• ACL Actions • Permit • Deny• Max. 256 access list• Max. 768 rules• Single or multiple ports (each rule)• Time-based ACL• ACL Statistics Security• Port Security • Supports up to 128 MAC addresses per port• Broadcast/Multicast/Unicast Storm Control• Dynamic ARP Inspection• Static MAC• D-Link Safeguard Engine• DHCP Server Screening• ARP Spoofing Prevention • Max. 64 entries• SSH • Supports v2 • Supports IPv4/IPv6• SSL • Supports v1/v2/v3 • Supports IPv4/IPv6• Traffic Segmentation• IP-MAC-Port Binding • DHCP snooping • IP Source Guard • Dynamic ARP inspection • DHCPv6 Guard • RA Guard • IPv6 Snooping • IPv6 Source Guard• DoS Attack Prevention AAA• Compound Authentication• 802.1X Port and MAC-based Authentication • Supports RADIUS and Local Server • Supports EAP, OTP, TLS, TTLS, PEAP• Web-based Access Control (WAC) • Port-based Access Control • Host-based Access Control • Dynamic VLAN Assignment• MAC-based Access Control (MAC) • Port-based Access Control • Host-based Access Control • Dynamic VLAN Assignment• Japan Web-based Access Control (JWAC) • Port-based Access Control • Host-based Access Control • Dynamic VLAN Assignment OAM• Cable Diagnostics• Factory Reset Management• CLI• Telnet Server• TFTP Client• IPv6 Neighbor Discovery• Configurable MDI/MDIX • SNMP• Supports v1, v2c, v3• SNMP Trap• System Log • Max.10,000 log entries• DHCP Client• D-Link Network Assistant support• SNTP• ICMPv6• IPv4/v6 Dual Stack• DHCP Auto Configuration• RMON v1 D-Link Green 3.0 Technology• Power Saving by: • Link Status • LED or Port Shutoff• System Hibernation mode • Time-based PoE (PoE model only) AC Input 100 to 240 VAC 50/60 Hz internal universal power supply Maximum Power Consumption 38.4 W Standby Power Consumption 27.6 W Smart Fan Quantity 2 Acoustics 44.2 dB(A) Heat Dissipation 130.944 BTU/hr Operation Temperature-5 to 50°C (23 to 122°F) Storage Temperature -20 to 70°C (-4 to 158°F) Operation Humidity 0% to 95% non-condensing Storage Humidity 0% to 95% non-condensing Dimensions 440 x 250 x 44 mm (17.36 x 9.84 x 1.73 inches) 19” standard rack mounting width, 1U height Weight 2.40 kg Diagnostic LEDs Power/Stacking ID/Fan (per device), Link/Activity/Speed (per 10/100/1000 Mbps port), Link/Activity/Speed (per Gigabit SFP port), Link/Activity/Speed (per 10G SFP+ port) Certifications CE, FCC, C-Tick, VCCI, BSMI, CCC Safety cUL, CB Supported SFP Modules: Supported WDM SFP Transceivers DEM-331T: 1000BASE-LX, Wavelength Tx: 1550 nm, Rx: 1310 nm, single-mode, 40 km DEM-331R: 1000BASE-LX, Wavelength Tx: 1310 nm, Rx: 1550 nm, single-mode, 40 km DEM-330T: 1000BASE-LX, Wavelength Tx: 1550nm, Rx 1310nm, single-mode, 10km DEM-330R: 1000BASE-LX, Wavelength Tx: 1310nm, Rx: 1550nm, single-mode, 10km Supported SFP+ Transceivers DEM-431XT: 10GBASE-SR SFP+ Transceiver (without DDM), 80m: OM1 & OM2 MMF, 300m: OM3 MMF DEM-431XT-DD: 10GBASE-SR SFP+ Transceiver (with DDM), 80m: OM1 & OM2 MMF, 300m: OM3 MMF DEM-432XT: 10GBASE-LR SFP+ Transceiver (without DDM), 10km DEM-432XT-DD: 10GBASE-LR SFP+ Transceiver (with DDM), 10km DEM-433XT: 10GBASE-ER SFP+ Transceiver (without DDM), 40km DEM-433XT-DD: 10GBASE-ER SFP+ Transceiver (with DDM), 40km DEM-434XT: 10GBASE-ER SFP+ Transceiver (without DDM), 80km DEM-435XT: 10GBASE-LRM SFP+ Transceiver (without DDM), 220m DEM-435XT-DD: 10GBASE-LRM SFP+ Transceiver (with DDM), 220m DEM-436XT-BXD: 10GBASE-LR SFP+ Transceiver. Wavelength: Tx: 1330nm Rx: 1270nm (without DDM), 20km DEM-436XT-BXU: 10GBASE-LR SFP+ Transceiver. Wavelength: Rx: 1330nm Tx: 1270nm (without DDM), 20km Supported SFP+ Direct Attach Stacking Cables DEM-CB100S: 10-GbE SFP+ 1 m Direct Attach Cable DEM-CB300S: 10-GbE SFP+ 3 m Direct Attach Cable DEM-CB700S: 10-GbE SFP+ 7m Direct Attach Cable. |  |  | 2 |  | 10000 | 20000 |
| 17 | 10 G SFP + 5m DAC | 10 Gbps SFP+ 5m DAC (Direct Attach Stacking Cables), Connector Type SFP+ to SFP+, compatible with the HP NC523SFP Network Card. |  |  | 4 |  | 1000 | 4000 |
| 18 | UPS | 230V Rackmount,Line-interactive,Pre-Installed SmartSlot Card AP9630,Output Power Capacity 4000 Watts/5000VA, Max Configurable Power 4000 Watts/5000VA, Nominal Output Voltage 230V, Output Voltage Note Configurable for 220:230 or 240 nominal output voltage, Output Voltage Distortion Less than 5% at full load,Output Frequency(sync to mains) 47-53 Hz for 50 Hz nominal,Other Output Voltages 220,240V, Output Connections (2) IEC 320 C19 (Battery Backup), (4) IEC Jumpers(BatteryBackup), (8)IEC 320 C13,Nominal Input Voltage 230V, Input Frequency 50/60Hz +/- 3Hz(auto sensing),Input Connections Hard Wire 3-wire,Input voltage range for main operations 160-286V,Input voltage adjustable range for mains operation 151-302V,Battery Type VRLA, Typical recharge time 3 hour,Replacement Battery RBC55,RBC Quantity 2,Interface Port DB-9 RS-232,SmartSlot,Pre-Installed SmartSlotCards AP9630,Control panel LED status display with On Line: OnBattery: ReplaceBattery and Overload indicators, AudibleAlarm Audible and visible alarms:configurable delays, Surge energy rating 480 Joules, Filtering Full time multi-pole noise filtering:0,3% IEEE surge let-through: zero clamping response time:meets UL 1449, Maximum Height 222.0 mm,Maximum Width 483.0 mm, Maximum Depth 666.0 mm,Rack Height 5U, Net Weight 97.73 kg, Color Black |  |  | 1 |  | 50000 | 50000 |
| 19 | Dulap comunicaţional | Dulap 42U, 800X1000X2000, ușa(față/spate) perforată, unitate de ventilare cu min 4 ventilatoare, 2 X bloc de prize electrice 19” (6 prize, 1U, 16A, cu filtrușisiguranță), raft staționar 3 buc, 4 X tije de instalare a dispozitivelor (2X față, 2X spate), 2 X organizator de cabluriverticali, complet de roțipivotante. |  |  | 1 |  | 10000 | 10000 |
| 20 | Sistem de operare | Microsoft Windows 10 pro x32/x64, En, licența nelimitată. |  |  | 50 | - | 11570 | 578500 |
| 21 | Aplicație de prelucrare imagine | Adobe Photoshop CC 32/64,, offline activator, termen nelimitat, compatibilitatea Microsoft Windows 7/10 x32/x64. |  |  | 5 | - | 18 000 | 90000 |
| 22 | Aplicație de video redactare | Edius Pro 9 32/64, offline activator, termen nelimitat, compatibilitatea Microsoft Windows 7/10 x32/x64. |  |  | 2 | - | 6300 | 12600 |
| 23 | Aplicație prelucrare text | Microsoft Office 2016 Home and Business 32/64, offline activator, termen nelimitat, compatibilitatea Microsoft Windows 7/10 x32/x64. |  |  | 20 | - | 8000 | 160000 |
| 24 |  | Microsoft Office 2019 Home and Business 32/64, offline activator, termen nelimitat, compatibilitatea Microsoft Windows 7/10 x32/x64. |  |  | 1 | - | 11570 | 11570 |

Termen de livrare: 30 zile calendaristice de la înregistrarea contractului la Trezorerie.