SPECIFICATION CONDITIONS

Object: Single-stage anti-hail missiles, with mortar reactive launching type

Contracting authority: Special Service for Active Influences on hydrometeorological processes

- General description.

Solid fuel single-stage anti-hail missiles, with mortar reactive launching type, intended for active influence on natural hail processes in order to minimise hail falls.

- Requirements for necessary technical characteristics:

- Maximum length – no more than 1200 mm.

- Missile diameter – 60 mm.

- Missile weight – no more than 5.0 kg.

- Freezing composition weight (active smoke composition) – no more than 0.4 kg.

- Number of active ice-forming nuclei, generated by the missile warhead:

- At temperature minus 10 °С – not less than 1.2х1016.

- Maximum flight path height – no more than 8000 m.

- Effective action radius – not less than 7.5 km.

- Ignition time of the active smoke pyrotechnic cartridge – no more than 5 sec.

- Launch capability via modified anti-hail installations type ТКБ-040, ТКБ-04-01, launch (impulse) voltage12 V, impulse launch duration 20 msec.

- Number of missiles in one packing box – no more than 4 pieces.

- Missile warranty period – at least 3 years, 6 months of which being in the field conditions, under the eaves.

- During the warranty period, the missiles will retain their normal characteristics and reliability:

- in the range of air temperatures at the ground surface 0° to at least +45°С;

- at a wind speed at the ground surface of at least 25.0 m/sec;

- at the relative humidity of the ambient air up to 98%;

- after missiles launching, the missiles from the neighbouring guide of the launching facility will be in reliable condition;

- after at least 20 reloads of the launch facility.

- Presence of four self-destruction points.

- Probability of safe use for the population, at the level of confidence 0.9 to be at least 0.9998.

- Presence of scientific expertise on freezing capacity of crystallization reagent from specialised laboratories from other countries (stationary and dynamic regime).

- Requirements on cost/price calculation.

The lowest price complying with technical requirements.

- Definitions.

Freezing capacity – the ability of a series of substances to initiate the onset of the solid phase by placing them in an overcooled cloudy environment (usually as aerosols).