TECHNICAL SPECIFICATION

| Portable Device for Verifying the Authenticity of Travel Documents – Level 1 with Advanced Capabilities |  |
| --- | --- |
| Functionalities and Basic Characteristics | The portable device shall be designed for the rapid verification of the authenticity of travel documents containing security elements. It shall provide the following functionalities: - detection of the main security printing features; - comparison of object dimensions; - control of typographic line width; - detection of mechanical and chemical alterations in the structure of the document; - examination of documents under different light spectra. Maximum dimensions: 103 × 61 × 46 mm Maximum weight: 205 g Power supply: Built-in Li-pol battery, rechargeable via USB Type-C Transport packaging: Protective carrying case made of Cordura-type material Display OLED |
| Technical, Quality and Operational Characteristics | Optical magnification: minimum 15× Examination of documents under white light: incident/top, oblique, coaxial, and OVD (hologram/kinegram) visualization Examination under UV light: 254, 313, 365, 400 nm (LED) Examination under IR light: high-intensity 980 nm and incident 850 nm Flashlight mode: white light for transparent documents and 365 nm UV for quick verification of an entire document page at a glance Field of view (minimum diameter): 11+/-1 mm Built-in module for detection and display of RFID type (embedded in travel documents) Certification: The equipment shall be certified in accordance with: ISO 9001, CE; Documentation - The operating manual shall be provided in Romanian, English, and Russian; Warranty and After-Sales Service: - Warranty period: at least 5 years from commissioning. - All warranty servicing expenses shall be borne exclusively by the Supplier, including transportation of the equipment to and from the service center, if required. Additional Requirements: Supply of a prototype equipment upon request. |