Anexă

CAIET DE SARCINI

| Item No. | Description / Specification of Goods (minimum) |  | Unit of measure | Quantity |
| --- | --- | --- | --- | --- |
|  | LOT I |  |  |  |
| 1 | Personal Computers (PC) |  | piece | 1 |
|  | Form factor: | Mini Desktop Form factor with possibility mountable to the back of the monitor via VESA standard |  |  |
|  | Processor: | Last Generation, min. Six cores, min. 2,0 GHz Base frequency; memory support min. DDR4-2933MHz |  |  |
|  | Memory: | min. 8GB DDR4-2933MHz; 2x SODIMM slots |  |  |
|  | Internal Storage: | 2x M.2 2280 PCIe NVMe SSD min. 256GB in RAID 1 |  |  |
|  | Interfaces: | min. 2x DP v1.4; HDMI v2.0; 2x USB 3.2 Gen2 Type-C; 4x USB 3.2 Gen2 Type-A |  |  |
|  | Network: | Gigabit Ethernet; Wi-Fi 6 802.11AC + BT5.0 |  |  |
|  | Audio: | internal speaker; 2x Audio ports |  |  |
|  | Graphics: | integrated; support min.2x monitors with resolution 1920x1080@60Hz |  |  |
|  | Keyboard: | USB (Eng/Rus) |  |  |
|  | Mouse: | USB Laser |  |  |
|  | Power Supply: | External |  |  |
|  | Accessories: | VESA mount Kit |  |  |
|  | Security: | TPM Module; Power-on and setup password (BIOS) |  |  |
|  | Monitor: | min. 23.8" IPS; FHD (1920x1080); Height Adjustable; VESA support; DP and HDMI; integrated speakers or speaker bar; Contrast min. 1000:1 (static), 5mln:1 (Dynamic); Brightness min. 250 nits |  |  |
|  | Operation System: | Microsoft Windows 10 Pro |  |  |
|  | Warranty: | min. 5 Years On-site official Manufacturer warranty with Defective Media Retention service for PC |  |  |
|  | Note: | all components of the set must be from the same manufacturer |  |  |
|  | LOT II |  |  |  |
| 1 | Network attached storage |  | piece | 1 |
|  | Type | Dual Controller iSCSI NAS |  |  |
|  | Form Factor | Rack 2U-5U, incl, Rack kit cu Cable Management Arm |  |  |
|  | Storage Controller | 2x Fully redundant hot swappable controllers, memory min. 128GB per controller |  |  |
|  | Interface | Dual 10G iSCSI ports per controller; Network Management Port, min 2x USB 3.0. |  |  |
|  | RAM | min. 128GB |  |  |
|  | Storage capacity | 12x min. 12TB SAS 12 GB/s 7.2k rpm 3.5"; support up to 48x HDD 3.5" with additional storage enclosures. |  |  |
|  | Supported possibility | FTP, Web interface for management, NFS, Support RAID 1, 0, 10, 5, 6, support connectivity to domen controller, possibility to create domen controler |  |  |
|  | Power Supply | 2x Fully redundant hot swappable Power Supplies |  |  |
|  | Management | Centralized Management Console |  |  |
|  | LAN | 2x 10 Gb/s ports SFP+, 4x 1Gb/s |  |  |
|  | Extra cable | 2x 10 Gb/s ports SFP+, 2 Meters |  |  |
|  | Warranty | 3 years official manufacturer warranty |  |  |
|  | LOT III |  |  |  |
| 1 | Graphics Workstation |  | piece | 1 |
|  | MB | cu posibilitatea de a instala min. 2 procesoare |  |  |
|  | CPU | Familia de procesoare: min.1 procesoare cu 28 nuclee la o frecvența de bază de min. 2,7 GHz și cu memorie cache de min. 38,5 MB |  |  |
|  | RAM | Minim 128 GB, 2933MHz, DDR4, ECC Registered Memory |  |  |
|  | Sloturi de memorie: | minim 12 slot-uri cu suport pentru mod Dual Channel |  |  |
|  | Stocare (unitate internă): | 1x NVMe M.2 SSD cu capacitatea 1To viteza citire/ scriere (6000/4000 MB/s) |  |  |
|  | Sloturi de expansiune: | minim 2x PCI-E 3.0 (înălțime maximă, full-lungime) și 2x sloturi trebuie sa fie libere; 1x PCI |  |  |
|  | Controler Hard Disk: | suport rată de transfer date 6.0 Gb/s (min. 8x SAS, 2x SATA porturi), suport RAID 0, 1, 10 |  |  |
|  | Placă grafică: | 1 x slot PCI-E 3.0, minim capacitate memorie 24 GB, GDDR6 – 384bit, 448 GB/s, suport 4 porturi digitale cu rezoluție de min.1920x1200, sistem de răcire activă cu radiator sau cooler |  |  |
|  | Audio: | audio integrat |  |  |
|  | Porturi: | Minim 10x porturi USB (6x porturi USB 2.0; 4x porturi USB 3.1 sau toate USB 3.0); 2x RJ45; 2x Display port sau HDMI cu suport pentru multi-flux; 1x intrare linie, 1x ieșire linie/cască, 1x cască, 1x microfon; 1x IEEE 1394a (fata) |  |  |
|  | Placă de reţea: | 2x 1 GbE integrat (cu suport iAMT, WOL, PXE 2.1, auto MDIX, Cablu de diagnostic avansat) |  |  |
|  | Alimentare curent: | Sursă de alimentare minim 1700 W cu o eficiență de minim 90% – funcție PFC activă |  |  |
|  | Caracteristice acustice: | max. 30 dBA |  |  |
|  | Gestiunea securității: | cip de securitate Trusted Platform Module 1.2; USB conectare/deconectare; parolă la conectare/setare BIOS; combinație lacăt + senzorul, care asigura blocare capotului a blocului principal si notificare in caz de acces neautorizat la interiorul calculatorului, care poate fi administrata prin BIOS (deschiderea/închiderea)sau prin acces de la distanta |  |  |
|  | Tastatura: | Eng/Rus USB același brand ca şi stația de lucru |  |  |
|  | Mouse: | USB optical, același brand ca şi stația de lucru |  |  |
|  | Sistem de operare preinstalat | Windows 10 Professional 64-bit |  |  |
|  | Garanție | Minim 58 luni pentru componente, manoperă şi servicii la atelier serviciu keep your drive 5 ani |  |  |
|  | LOT IV |  |  |  |
| 1 | Color printer |  | piece | 1 |
|  | Type: | Color Laser Printer |  |  |
|  | Print Speed (Color): | up to 38 ppm (A4) |  |  |
|  | Print quality: | 1200x1200 dpi standard resolution (HW); should be capable with PANTONE color system |  |  |
|  | Print margins: | max. 4mm per each side |  |  |
|  | Duplex printing: | Automatic |  |  |
|  | Duty cycle (monthly, A4): | min. 80,000 pages |  |  |
|  | Memory: | min. 1GB + min.4GB Flash memory or HDD/SSD with AES256 encryption |  |  |
|  | Standard print languages: | PCL6, PDF |  |  |
|  | Interfaces: | Gigabit Ethernet port, Hi-speed USB, USB Host port |  |  |
|  | Paper handling (Input): | min. 500-sheet + 100-sheet; support additional Tray for min. 500 sheet |  |  |
|  | Paper size and weight: | A4, A5; min. 60 g/m² and up to 200 g/m² postcard media type |  |  |
|  | Cartridges: | All-in-one cartridge type, support color cartridges with min. 9000 pages yeld |  |  |
|  | Warranty: | 3 Years On-site official manufacturer warranty |  |  |
|  | LOT V |  |  |  |
| 1 | Scanner for PC |  | piece | 1 |
|  | Type: | Desktop Flatbed/ADF scanner |  |  |
|  | Scan Speed (ADF, Color): | up to 40 ipm (A4) |  |  |
|  | Scan quality: | min. 1200x1200 dpi optical resolution; 600x600 dpi optical resolution (ADF); up to 256/48-bits |  |  |
|  | Scanner features: | Auto detect size; Auto orient; Background smooth and removal; Edge erase; Delete of blank page; Merge of pages and document separation; Two-side scanning support |  |  |
|  | Duty cycle (daily, A4): | min. 1,500 pages |  |  |
|  | Scan file format: | PDF and searchable PDF; TIFF; JPEG; PNG |  |  |
|  | Interfaces: | Hi-speed USB |  |  |
|  | Paper handling (ADF): | min. 50-sheet |  |  |
|  | Paper size: | A4, A5 |  |  |
|  | Warranty: | 3 Years On-site official manufacturer warranty |  |  |