SPECIFICATIONS / TECHNICAL SPECIFICATIONS

Object: Modern and ecological transport units for the development of an integrated urban transport system in the municipality of Ungheni

Contracting authority: Centre Regional Development Agency

Beneficiary: Ungheni City Hall, Ungheni district, mun. Ungheni, str. Naţională nr. 7

General conditions:

All transport units must be new.

The warranty period must be at least 2 years or 150 000 km from the date of receipt of the transport units.       The supplier is obliged to ensure the training of the beneficiary's staff.

The supplier is obliged to supply spare parts and aggregates for the repair of transport units, for a fee, for a period of at least 10 years after the expiry of the warranty.

Transport units must be properly equipped so that the service operator can continue to ensure the safety of passengers.

The transport units must comply with national and EU requirements (as applicable) in order for the service operator to be able to obtain all the necessary authorisations for the operation of the public passenger transport service.

The buses will be intended for operation in temperate-continental climate areas and will ensure reliable operation in the following ambient conditions:

•	Ambient temperature - 30 °C ... + 45 °C;

•	Maximum relative humidity 100 % RH at + 25 °C;

•	The atmospheric pressure between 866 ... 1066 kPa;

•	Altitude from sea level (0 m) to a maximum of 400 m;

•	Outdoor agents: dust, rain, fog, mud, snow, frost, ice, salt water, petroleum products, anti-slip materials and solutions.

| Item no. | CPV Code | Name of the requested goods | Unit of measurement | Quantity | Required full specification, reference standard |
| --- | --- | --- | --- | --- | --- |
| 1. | 34121500-6 | Small capacity buses for urban public transport | Pcs | 14 | Year of manufacture: starting withYear 2024 Body type – self-supporting or non-load-bearing Dimensions Total length: 5000 – 7500 mm; Width: max. 2700 mm (without exterior mirrors); Height: max. 3100 mm Interior dimensions Height: minimum 1850 mm; Width: minimum 1850 mm; Minimum width of the open door access space: minimum 700 mm Maximum permissible total weight: maximum 8 500 kg Ecological standard: minimum EURO 5 Body type: with anti-corrosion processing Servo or hydraulic steering system Engine type: diesel, four-stroke turbocharger Engine power: 100 - 140 kW Gearbox: automatic or mechanical Cubic capacity: 2000 - 3800 cm³ Safety systems at least ABS; EBD; ESP Minimum power 130 Hp Central locking Speed limiter (optional) Passenger access door with driver-controlled automatic opening Electric rear-view mirrors with blind spot elimination, equipped with heating elements Fuel tank with min. capacity 80 liters Ramp for people with disabilities, electrically or manual operated Adjustable driver's seat Ventilation and heating system: Air conditioning system. Air System Warranty Conditionally must not be less than the general warranty for the bus. The side windows of the salon must ensure its natural ventilation through sliding glass at the top. Depending on the type of size (range) of the bus, it will have at least one air hatch, with opening . Number of seats: minimum 15 people on seats Stop button at the request of passengers (optional) Durable upholstery and upholstery, easy to maintain and clean Minimum 1 exterior indicator with electronic display for informing passengers and indicating the route, placed at the front and optionally on the side and rear Information system: Electronic information system (front and optional side, rear, interior). LED technology Passenger counting system (optional) Indoor and outdoor video surveillance system E-ticketing system (optional) All inscriptions on the inside and outside of the buses will be in Romanian. The exterior painting and all the inscriptions according to the legislation in force (tire pressure, safety exits, places for people with reduced mobility, wheelchairs, etc.) will be carried out by the winning bidder according to the legislative prescriptions in force. The specific design elements regarding the exterior painting of the body will be established in agreement with the beneficiary and will include the identity elements, symbols and/or logo of the City Hall and the inscription "Financed from the National Fund for Regional and Local Development, through the RDA Center " |
| 2. | 34121500-6 | Medium-capacity buses for urban public transport | Pcs | 4 | Year of manufacture – starting with Year 2024 Body type – self-supporting / non-load-bearing, city bus Dimensions Length: 8,000 – 12,500 mm; Width: max. 2550 mm (without exterior mirrors); Height: max. 3300 mm Interior dimensions Height: minimum 2100 mm; Width: minimum 1900 mm; Height of the access platform in living room: maximum 370 mm; Minimum width of the space Open door access: (two-leaf door) minimum 1100 mm. Minimum width of the space Open door access: (single-leaf door) minimum 550 mm. Maximum permissible total weight: 19000 kg Ecological standard - minimum EURO 5 Body type – Body with anti-corrosion processing Servo or hydraulic steering system Engine type: diesel, four-stroke turbocharger Engine power: min 110 kW Gearbox: Automatic / mechanical Cubic capacity: maximum 7700 cm³ Safety systems: minimum ABS; EBD; ESP Minimum power 150 Hp Centrallocking Speed limiter (optional) Double passenger access door, with automatic opening controlled by the driver Electric rear-view mirrors with blind spot elimination Fuel tank with min. capacity110 liters Ramp for people with disabilities, preferably electrically operated Adjustable driver's seat Ventilation and heating system: Air conditioning system. Air System Warranty Conditionally must not be less than the general warranty for the bus. The side or upper windows of the salon must ensure its natural ventilation through sliding glass at the top. Depending on the type of size (range) of the bus, it will have at least one ventilation hatch. Number of seats , minimum transport capacity: 50 people, of which at least 18 people on seats Stop button at the request of passengers Durable upholstery and upholstery, easy to maintain and clean Minimum 1 external indicator with electronic display for informing passengers and indicating the route, placed in front, and optionally on the right side. Information system: Electronic information system (front, side, rear, interior). LED technology Passenger counting system (optional) Indoor and outdoor video surveillance system E-ticketing system (optional) The dashboard will be equipped with an on-board computer with a multifunctional digital display The front indicator (and optionally the side and rear indicators) must display the line number, the starting point and the final destination, optionally displaying the intermediate route. All inscriptions on the inside and outside of the buses will be in Romanian. The exterior painting and all the inscriptions according to the legislation in force (tire pressure, safety exits, places for people with reduced mobility, wheelchairs, etc.) will be carried out by the winning bidder according to the legislative prescriptions in force. The specific design elements regarding the exterior painting of the body will be established in agreement with the beneficiary and will include the identity elements, symbols and/or logo of the City Hall and the inscription "Financed from the National Fund for Regional and Local Development, through the RDA Center " |

*Note: Payments will be made by transfer based on e-invoices, within 45 days after receipt of the handover-receipt documents, and invoices, signed and accepted by the Co-investor. It is forbidden to make advance payments (advance) by the contracting authority.