CAIET DE SARCINI

| Item No. | Description / Specification of Goods |  |  | Unit of measure | Quantity | Yes/No (please provide detailed description of offered goods) |
| --- | --- | --- | --- | --- | --- | --- |
|  | LOT I |  |  |  |  |  |
| 1 | Personal Computers (PC) |  |  | piece | 65 |  |
|  | Form factor: | Mini Desktop Form factor with possibility mountable to the back of the monitor via VESA standard |  |  |  |  |
|  | Processor: | min. Generation Y2020, min. Six cores, min. 2,9GHz Base frequency, TDP min. 65W; min. 12 MB cache; memory support min. DDR4-2666MHz |  |  |  |  |
|  | Memory: | min. 8GB DDR4-2666MHz; 2x SODIMM slots |  |  |  |  |
|  | Internal Storage: | 2x M.2 2280 PCIe NVMe SSD min. 256GB in RAID 1 |  |  |  |  |
|  | Interfaces: | min. 3 video digital port (1x DP v1.4; 1x HDMI v2.0, 1x DP or HD-MI) (without adapter); 1x USB 3.2 Gen2 Type-C; 3x USB 3.2 Gen2 Type-A; 1x USB 3.2; |  |  |  |  |
|  | Network: | Gigabit Ethernet; Wi-Fi 5 802.11AC + BT5.0 |  |  |  |  |
|  | Audio: | internal speaker; 2x Audio ports |  |  |  |  |
|  | Graphics: | integrated; support min.2x monitors with resolution 1920x1080@60Hz |  |  |  |  |
|  | Keyboard: | USB (Eng/Rus) |  |  |  |  |
|  | Mouse: | USB Laser |  |  |  |  |
|  | Power Supply: | External |  |  |  |  |
|  | Accessories: | VESA mount Kit for fastening to the monitor |  |  |  |  |
|  | Security: | Discrete TPM Module; Power-on and setup password (BIOS) |  |  |  |  |
|  | Monitor: | min. 23.8" IPS; FHD (1920x1080); Height Adjustable; VESA support; DP and HDMI; integrated speakers or speaker bar; Contrast min. 1000:1 (static), min. 3mln:1 (Dynamic); Brightness min. 250 nits |  |  |  |  |
|  | Operation Sistem: | Microsoft Windows 10 Pro OEM (key writen in BIOS) |  |  |  |  |
|  | Warranty: | min. 5 Years On-site official Manufacturer warranty with "Keep Your Drive/Deffective Media Retention" service for PC |  |  |  |  |
|  | Note: | all components of the set must be from the same manufacturer |  |  |  |  |
| 2 | Server |  |  | piece | 1 |  |
|  | Type: | Rack 2U |  |  |  |  |
|  | Processor: | 2x processors; min. Eighteen cores, min. 2,3GHz Base frequency; min. 24MB cache; min. DDR4-2666MHz memory support |  |  |  |  |
|  | Memory: | min. 512GB DDR4-2666MHz (12x32GB); min. 24x DIMM slots |  |  |  |  |
|  | Storage: | 2x M.2 2280 PCIe SSD min. 900GB Read Intencive; min. 10x HDD 2,4TB SAS 12G; support installation up to 16x 2,5" |  |  |  |  |
|  | Controller: | SAS/SATA 12G; min. 16x internal connection; min 4GB FBWC; RAID 0/1/5/6/10/50/60 and FIPS 140-2 Level 1 support; Battery should be included |  |  |  |  |
|  | Network: | 4x Gigabit Ethernet; 2x 10 Gigabit Ethernet SFP+transiver |  |  |  |  |
|  | Slots: | min. 1x PCIE x8 and 2x PCIEx16 must be free |  |  |  |  |
|  | HBA: | min. 2x single port 16Gbit Fibre Channel HBA |  |  |  |  |
|  | Power Supply: | 2x Redundand PS with min 1500W each module |  |  |  |  |
|  | Fans: | Full q-ty of Fans in redundand mode |  |  |  |  |
|  | Management: | Remote console, virtual keyboard and mouse; support virtual media and folders |  |  |  |  |
|  | Rail Kit: | Rail Kit with Cable Management Arm |  |  |  |  |
|  | Warranty: | min. 5 Years On-site official Manufacturer warranty |  |  |  |  |
| 3 | Color printer |  |  | piece | 4 |  |
|  | Type: | Color Laser Printer |  |  |  |  |
|  | Print Speed (Color): | up to 38 ppm (A4) |  |  |  |  |
|  | Print quality: | 1200x1200 dpi standard resolution (HW); should be capable with PANTONE color system |  |  |  |  |
|  | Print margins: | max. 4mm per each side |  |  |  |  |
|  | Duplex printing: | Automatic |  |  |  |  |
|  | Duty cycle (monthly, A4): | min. 80,000 pages |  |  |  |  |
|  | Memory: | min. 1GB + min.4GB Flash memory or HDD/SSD with AES256 encryption |  |  |  |  |
|  | Standard print languages: | PCL6, Adobe PostScript3, PDF |  |  |  |  |
|  | Interfaces: | Gigabit Ethernet port, Hi-speed USB, USB Host port |  |  |  |  |
|  | Paper handling (Input): | min. 500-sheet + 100-sheet; support additional Tray for min. 500 sheet |  |  |  |  |
|  | Paper size and weight: | A4, A5; min. 60 g/m² and up to 200 g/m² postcard media type |  |  |  |  |
|  | Cartridges: | All-in-one cartridge type, support color cartridges with min. 9000 pages yeld |  |  |  |  |
|  | Warranty: | 3 Years On-site official manufacturer warranty |  |  |  |  |
| 4 | Scaner for PC |  |  | piece | 4 |  |
|  | Type: | Desktop Flatbed/ADF scanner |  |  |  |  |
|  | Scan Speed (ADF, Color): | up to 40 ipm (A4) |  |  |  |  |
|  | Scan quality: | min. 1200x1200 dpi optical resolution; 600x600 dpi optical resolution (ADF); up to 256/48-bits |  |  |  |  |
|  | Scanner features: | Auto detect size; Auto orient; Background smooth and removal; Edge erase; Delete of blank page; Merge of pages and document separation; Two-side scanning support |  |  |  |  |
|  | Duty cycle (daily, A4): | min. 1,500 pages |  |  |  |  |
|  | Scan file format: | PDF and searchable PDF; TIFF; JPEG; PNG |  |  |  |  |
|  | Interfaces: | Hi-speed USB |  |  |  |  |
|  | Paper handling (ADF): | min. 50-sheet |  |  |  |  |
|  | Paper size: | A4, A5 |  |  |  |  |
|  | Warranty: | 3 Years On-site official manufacturer warranty |  |  |  |  |
| 5 | Router |  |  | piece | 1 |  |
|  | Type: | Modular Router, Rack 1U or 2U |  |  |  |  |
|  | Slots: | min. 2x open slots for future upgrade |  |  |  |  |
|  | Interfaces: | min. 2x Gigabit (1000BASE-T) WAN ports, 4x Gigabit (1000BASE-T) LAN ports, USB port, Console port; support LTE connection |  |  |  |  |
|  | Memory: | min. 1GB SDRAM |  |  |  |  |
|  | Throughput: | min. 600 Mbp/s |  |  |  |  |
|  | Protocols: | IPv4, IPv6, static routes, RIP and RIPv2, (OSPF, BGP, BGP Router Reflector, Intermediate System-to-Intermediate System (IS-IS), Multicast Internet Group Management Protocol Version 3 (IGMPv3), Protocol Independent Multicast sparse mode (PIM SM), PIM Source Specific Multicast (SSM), RSVP, CDP, ACL, DHCP, DNS, RADIUS, MPLS, Layer 2 and Layer 3 VPN, IP sec, Bidirectional Forwarding Detection |  |  |  |  |
|  | Security: | IPSec VPN; Zone based firewall; TACACS+; ADP; IPS |  |  |  |  |
|  | Traffic management: | QoS; Weighted random early detection |  |  |  |  |
|  | Cryptographic Algorithms: | DES, 3DES, AES-128/256; MD5; SHA |  |  |  |  |
|  | Warranty: | 1 Year On-site official manufacturer warranty |  |  |  |  |
| 6 | Switch |  |  | piece | 1 |  |
|  | Type: | Managed Switch, Rack 1U |  |  |  |  |
|  | Interfaces: | min. 24x Gigabit (1000BASE-T) LAN ports, 4x 10 Gigabit SFP+ ports; Console port; |  |  |  |  |
|  | Memory: | min. 2GB SDRAM; 2GB Flash memory |  |  |  |  |
|  | Routing Switching capacity: | min. 128 Gbp/s |  |  |  |  |
|  | Throughput: | min. 600 Mbp/s |  |  |  |  |
|  | Protocols: | Protocols and Services: Static IPv4/IPv6 routing; IGMP; DHCP; LACP; Static ARP; support of internal loopback testing; support min. 512 VLANs simultaneously (all necessary licenses should be included); |  |  |  |  |
|  | Security: | IPSec VPN; Zone based firewall; TACACS+; ADP; IPS |  |  |  |  |
|  | Traffic management: | QoS; Weighted random early detection |  |  |  |  |
|  | Warranty: | 5 Years On-site official manufacturer warranty |  |  |  |  |
|  | LOT II |  |  |  |  |  |
| 1 | Barcode printer |  |  | piece | 65 |  |
|  | Type: |  | Desktop barcode printer |  |  |  |
|  | Print method: |  | Thermal Transfer, Direct Thermal |  |  |  |
|  | Print width: |  | min. 105 mm |  |  |  |
|  | Print length: |  | min. 4mm |  |  |  |
|  | Print speed: |  | min. 4” IPS |  |  |  |
|  | Ribbon: |  | Wax, wax/resin, resin |  |  |  |
|  | Ribbon length: |  | 300 m long |  |  |  |
|  | Ribbon width: |  | max. 110 mm |  |  |  |
|  | Print resolution: |  | min.: 300dpi |  |  |  |
|  | Media type |  | Continuous form, gap labels, black mark sensing, label length set |  |  |  |
|  | Memory: |  | SDRAM min. 16MB, Flash min. 8MB |  |  |  |
|  | Internal fonts: |  | min. Bitmap, TTF |  |  |  |
|  | Print of codes: |  | 1-D and 2-D Bar codes |  |  |  |
|  | Functions: |  | Black mark sensor, Gap sensor, Tape end sensor etc. |  |  |  |
|  | Interfaces: |  | min.: USB 2.0, Ethernet, Serial port |  |  |  |
|  | Software driver: |  | min. Windows; Windows Server, Linux etc. |  |  |  |
|  | Warranty: |  | min. 2 Year On-site official manufacturer warranty |  |  |  |
| 2 | Barcode Scanner for PC |  |  | piece | 65 |  |
|  | Type: |  | Barcode handheld cordless scanner |  |  |  |
|  | Decode capability |  | min.: 1-D, 2-D |  |  |  |
|  | Light Source: |  | LED |  |  |  |
|  | Scanner sensor: |  | LED |  |  |  |
|  | Imager sensor |  | min. 640x480 pixels |  |  |  |
|  | Scan modes |  | trigger, sense, continuous mode |  |  |  |
|  | Drop |  | min.1.0 m |  |  |  |
|  | Connectivity: |  | wireless |  |  |  |
|  | Radio |  | 2.4 GHz |  |  |  |
|  | Communication distance |  | ≥50 m (in open space) |  |  |  |
|  | Impute Voltage |  | ±5VDC |  |  |  |
|  | Interface: |  | min. USB |  |  |  |
|  | Battery type |  | Li-Ion, min. 2Ah |  |  |  |
|  | Battery life |  | min. 12 hours |  |  |  |
|  | Operation temperature |  | -20℃ to +50℃ |  |  |  |
|  | Environmental Sealing |  | min. IP42 |  |  |  |
|  | Warranty: |  | min. 3 Years On-site official manufacturer warranty |  |  |  |
| 3 | Barcode printer ribbon |  |  | Piece | 260 |  |
|  | Designed: |  | To print all kind of barcodes for thermal transfer printers |  |  |  |
|  | Type: |  | Ribbon on rolls |  |  |  |
|  | Ink Coated Side |  | Out |  |  |  |
|  | Ribbon ink |  | Wax/Resin |  |  |  |
|  | Width |  | min. 108mm max. 110mm |  |  |  |
|  | Color |  | Black |  |  |  |
|  | Ribbon length: |  | 300 m; |  |  |  |
|  | Compatible |  | Compatible with Barcode printer Lot II, Item No.1 and Item No.4. |  |  |  |
| 4 | Ticket for barcode printer |  |  | Piece | 1300 |  |
|  | Type |  | labels on rolls |  |  |  |
|  | Destination |  | rolls for thermal transfer printers |  |  |  |
|  | Color |  | white |  |  |  |
|  | Paper |  | with permanent adhesive |  |  |  |
|  | Width |  | 52 mm |  |  |  |
|  | Length |  | 30 mm |  |  |  |
|  | No. labels/roll |  | min. 4000 labels on a roll, two labels on a rows |  |  |  |
|  | Compatible |  | Compatible with Barcode printer Lot II, Item No.1 and Item No.3. |  |  |  |