Liceul Teoretic cu profil de arte "Elena Alistar"

CAIET DE SARCINI

ACHIZIȚIE SISTEM SUPRAVEGHERE VIDEO INTEGRAT

CAIET DE SARCINI

PENTRU  CEREREA OFERTEI  DE  PREŢ

Introducere:

Caietul de sarcini face parte integrantă din documentaţia pentru atribuirea contractului şi

constituie ansamblul cerinţelor pe baza cărora se elaborează de către fiecare ofertant

propunerea tehnică. Caietul de sarcini conţine specificaţii tehnice, care vor fi considerate ca

fiind minimale. În acest sens, orice ofertă prezentată, care se abate de la prevederile Caietului de sarcini, va fi luată în considerare, dar numai în măsura în care propunerea tehnică presupune asigurarea unui nivel calitativ superior cerinţelor minimale din Caietul de sarcini. Ofertele care nu satisfac cerinţele caietului de sarcini vor fi declarate oferte neconforme şi vor fi respinse.

Denumirea achiziției: Achiziționarea și montarea sistemului de supraveghere video și sistem de limitare a accesului pe teritoriul și în incinta la Liceul Teoretic cu profil de arte "Elena Alistar" amplasat pe Bd. Decebal 74.

Amplasarea obiectului:                                                                                                                                                                                         Mun. Chisinau, Sector Botanica Bd. Decebal nr. 74

- Termenul de efectuare: 40 zile calendaristice din momentul semnării contractului.

- Modul de finalizare a lucrărilor: Executantul este dator să notifice investitorului data terminării tuturor lucrărilor prevăzute în contract, printr-un document scris, confirmat de reprezentantul investitorului pe șantier, pentru desemnarea comisiei de recepție la terminarea lucrărilor.

- Documentația de referință:

  - Planul Cadastral:

5.2. Normativele în vigoare privind sistemele de securitate.

6. Cerințe de bază:

- 6.1. Participanții asigură efectuarea luсrărilоr în conformitate cu Codul urbanistic .

  - Împreună cu sistemul să fie livrate accesoriile necesare instalării.

  - Toate cerinţele tehnice sunt minimale şi obligatorii. Toate echipamentele ofertate trebuie să fie compatibile cu standardele de alimentare cu energie electrică disponibile în Moldova: 220V c.a.  la 50 Hz. Toate echipamentele ofertate trebuie să fie noi şi neutilizate anterior. Ofertanţii vor include în ofertă, în original, un document emis de către producătorul echipamentelor ofertate care să certifice că modelele echipamentelor ofertate sunt în producţie (nu sunt declarate EOL) la data ofertei.

  - Operatorul economic adjudecat câștigător este obligat să aprobe, avizeze și coordoneze execuția lucrărilor, darea lor în exploatare cu Beneficiarul, autoritățile și instanțele de resort.

Operatorii economici participanti la licitatie trebuie sa detie Certificat MAF( Manufacturer Authorization Form) de la producatorul de echipament, si centru de deservire sau contract cu centrul de deservire autorizat de producator.

  - Ofertantul are obligația de a executa lucrările cu folosirea materialelor certificate în RM.

  - Tot utilajul trebuie să fie certificat, precum şi să dispună de toată documentația tehnică (specificaţia, descrierea, instrucțiuni pentru utilizatori).

  - Toate deficiențele, viciile apărute în pe parcursul termenului de garanție vor fi remediate de către Ofertant imediat, dar nu mai târziu de 15 (cincisprezece) zile lucrătoare de la momentul notificării.

  - Ofertantul se obligă să execute toate lucrările și volumele incluse în caietul de sarcini și proiectele de execuție.

  - Vizitarea obiectivului supus îndeplinirii lucrărilor din caietul de sarcini, corespunderea caietului de sarcini și volumele de lucrări, sресifiсând data cînd acesta a făcut cunoștință cu locul (obiectul) efectuării luсrărilоr. În cazul depistării lucrărilor neprevăzute sau neconformități a caitului de sarcini cu lucrările ce urmează a fi îndeplinite ofertantul este obligat să înștiințeze Beneficiarul în scris în termeni cît mai restrînși pentru clarificări.

  - Ofertantul garantează calitatea lucrărilor îndeplinite în termen de 2 ani din data semnării Procesului-Verbal de recepție la terminarea lucrărilor și înlătură defectele pe tot termenul de garanție din cont propriu dacă acestea sunt legate de nerespectarea normelor în vigoare și regulilor tehnice pe parcursul îndeplinirii lucrărilor sau de calitatea materialelor folosite.

  - Termenul de garanție pentru toate echipamentele de baza furnizate trebuie să fie minim de 36 de luni din data semnării Procesului-Verbal de recepție.

  - Autoritatea contractantă nu efectuează plăți anticipate în calitate de avans.

  - Achitarea lucrărilor executate se va efectua de către Autoritatea contractantă în baza procesului-verbal de executare a lucrărilor și facturii fiscale în termen de 10 zile calendaristice din data înregistrării facturii fiscale  în registrul Beneficiarului. Executantul lucrărilor va emite factura fiscală Beneficiarului în termen de 5 zile calendaristice după semnarea de către părți a Procesului-Verbal de executare a lucrărilor și/sau Actului de recepție finală a lucrărilor.

  - Ofertele vor fi prezentate cu semnătură și ștampilă autorizată a firmei.

  - Lista materialelor și echipamentelor, conform proiectului, sunt indicate în Anexa nr.1, parte a acestui document.

- Ofertantul prezintă actele confirmative care atestă dreptul de activitate inclusiv executarea lucrărilor în domeniul respectiv.

- Operatorii economici vor avea acces la toate cabinetele si documentatiile tehnice detinute de beneficiar pe durata executarii lucrarilor.

Anexa nr. 1

Sistemul supraveghere video

| Nr.crt. | Denumirea Utilajului si a Materialelor | Unitatea de masura | Volum |
| --- | --- | --- | --- |
|  | Cabluri |  |  |
|  | Cablu extern FTP Cat.6 4P CU Solid Outdoor, cu tross de susținere, HF (Halogen Free) Cablu FTP Categoria 6 destinat instalațiilor exterioare, prevăzut cu 4 perechi torsadate și ecranaj general din folie de aluminiu pentru protecție împotriva interferențelor electromagnetice (EMI). Conductori realizați din cupru masiv (Solid Copper) cu diametrul de 0,57 mm (23 AWG). Echipat cu tross metalic integrat pentru montaj aerian și susținere mecanică suplimentară. | m | 2100 |
|  | Cablu FTP Cat.6 4P CU, HF ( Halogen Free) Interior | M | 560 |
|  | Cablu optic cu 4 Fire Atenuare: 1310nm ≤0,36 db/km Atenuare: 1550nm ≤0,22 db/km Rezultatul testului: Trecut | m | 260 |
|  | Cablu Alimentare VVGnG 3x1.5 LS Secțiune 1.5 mm² Numărul de fire 3 * Culoare Negru Tip material Cupru Temperatura mediului ambiant la exploatare de la -50°С pînă la +50°С Temperatura maximă de încalzire a firelor +70°С | m | 150 |
|  | Materiale |  |  |
|  | Materiale de Fixare cablu si echipament | Set | 50 |
|  | Conector terminal de fibra optica 2.5 | buc. | 4 |
|  | Box optic terminal de abonat 1 si 2 fibre (FTTH) | buc | 2 |
|  | FO Patch-cord | buc | 4 |
|  | SFP module | buc | 2 |
|  | Conector RJ-45 | buc | 60 |
|  | Utilaj Video |  |  |
|  | 8 MP Smart Hybrid Light with ColorVu Fixed Mini Bullet Network Camera HikAI-ISP for excellant noise reduction effect 24/7 colorful imaging via ColorVu technology Scene-adaptive WDR Focus on Person and Vehicle classification based on deep learning Built-in arrayed dual-microphone for real-time high quality audio security Provide real-time security via built-in two-way audio Anti-corrosion design, providing reliability and longevity compared to standard (NEMA4X) Smart Hybrid Light: Integrates IR and White lights, 3 supplemental lighting modes Water and dust resistant (IP67) Active strobe light and audio alarm to warn intruders off Image Sensor 1/1.8" Progressive Scan CMOS Max. Resolution 3840 × 2160 Min. Illumination Color: 0.0005 Lux @ (F1.0, AGC ON),0 Lux with light Shutter Time 1 s to 1/100,000 s Day & Night IR cut filter Angle Adjustment Pan: 0° to 360°,tilt: 0° to 90°,rotate: 0° to 360° Deep Learning Function Face Capture Yes Perimeter Protection Line crossing, intrusion, region entrance, region exiting Support alarm triggering by specified target types (human and vehicle) Material Front cover: Metal, body: Metal, bracket: Metal Power 12 VDC ± 25%, 1.25 A, max. 15 W,Ø5.5 mm coaxial power plug,reverse polarity protection, PoE: IEEE 802.3at, Class 4, max. 18 W | buc | 19 |
|  | Camera Video pentru Interior cu parametrii minimi 8 MP Network Turret Camera with Smart Hybrid Light and ColorVu Technology 8 Megapixel network turret camera featuring Smart Hybrid Light technology and 24/7 full-color imaging. Designed for high-quality video surveillance in residential, commercial and industrial applications, providing excellent image clarity in low-light and nighttime conditions. Intelligent Functions Human and vehicle classification based on deep learning algorithms Line crossing detection Intrusion detection Region entrance detection Region exit detection Motion detection with intelligent filtering Push alarm notifications to mobile devices Video Maximum resolution: 3840 × 2160 (8 MP) High-performance progressive scan CMOS image sensor Full-color imaging 24 hours a day Excellent low-light sensitivity True WDR 120 dB BLC, HLC and 3D DNR image enhancement technologies Multi-stream support Video compression: H.265+, H.265, H.264+, H.264 and MJPEG Lens Fixed focal lens Wide viewing angle for comprehensive area coverage Audio Built-in microphone High-quality audio recording synchronized with video streams Illumination Smart Hybrid Light technology IR illumination for discreet night monitoring White light illumination for full-color night imaging Automatic switching between illumination modes according to scene conditions Illumination range up to 30 m Storage and Network MicroSD/SDHC/SDXC card support (depending on configuration) Ethernet network interface Remote access via web browser, software platform and mobile application Power 12 VDC power supply PoE (Power over Ethernet) support according to IEEE 802.3af standard Environmental IP67-rated weatherproof housing Suitable for indoor and outdoor installation Operating temperature range from -30°C to +60°C Applications Commercial buildings Hotels and hospitality facilities Industrial sites Educational institutions Residential complexes Perimeter and parking area monitoring Warranty 3-year manufacturer warranty | Buc | 17 |
|  | Comutator de retea de 24 porturi cu parametrii minim 24-Port Smart Monitoring PoE+ Switch for Video Surveillance Networks Managed Smart Monitoring PoE+ switch designed for IP video surveillance and security network applications. Provides simultaneous data transmission and power delivery over Ethernet with extended transmission distance of up to 300 m in long-range mode. Ports and Connectivity 24 × 10/100 Mbps PoE+ RJ45 ports 2 × Gigabit Combo ports Extended PoE transmission distance up to 300 m Simultaneous power and data transmission over a single Ethernet cable PoE Features IEEE 802.3af and IEEE 802.3at compliant Maximum PoE output per port: 30 W Total PoE power budget: 370 W Priority power allocation on ports 1–8 Supports power delivery over Ethernet cable pairs 1/2, 3/6 and 4/5, 7/8 Smart Monitoring Functions Network topology visualization Network health monitoring and status overview Real-time bandwidth and traffic statistics Port utilization and line load monitoring Alarm and event notifications Remote switch reboot capability Remote individual port restart Event and system log management Integrated live camera view support Performance Switching capacity: 8.8 Gbps Optimized for stable video and data transmission Reliable operation in high-density surveillance deployments Protection 6 kV surge protection Enhanced protection against lightning-induced electrical surges Power Supply Input voltage: AC 220 V Environmental Fanless design for silent operation and increased reliability Operating temperature: -10°C to +55°C Physical Characteristics Dimensions: 440 × 221 × 44 mm Rack-mountable metal enclosure Applications IP video surveillance systems Security and monitoring networks Commercial and industrial facilities Critical infrastructure installations Building Management Systems (BMS) Access control and integrated security platforms Warranty 3-year manufacturer warranty | Buc | 1 |
|  | Comutator de retea cu 8 porturi cu parametrii minim; SWITCH POE • Switch PoE 8 canale • Putere totala 110W • Distanta de transmisie max. 300 metri • Suporta management de putere PoE • 8 Porturi PoE 10/100 Mbps • 1 Port Ethernet 1000 Mbps • 1 Port SFP 1000 Mbps | buc | 3 |
|  | NVR de 64 canale si 4 spatii pentru HDD pana la 16TB cu parametrii minimi H.265+/H.265/H.264+/H.264 video formats Intelligent analytics based on Guanlan Large-Scale AI Models Up to 64-ch AcuSeek and AcuSearch function Up to 8-ch facial recognition for face picture Up to 64-ch IP camera inputs Up to 32-ch 1080p decoding capability Up to 8-ch IP speakers can be connected AI by NVR Facial recognition AI by NVR and Camera Facial recognition, perimeter protection, video structuralization, AcuSearch, motion detection 2.0, ANPR, people counting, AcuSeek Video Output Mode HDMI1/VGA1 simultaneous output, HDMI2/VGA2 simultaneous output CVBS Output NA Audio Output 2-ch, RCA (Linear, 1 KΩ) Two-Way Audio 1-ch, RCA (2.0 Vp-p, 1 KΩ) HDMI 1 Output 4K（3840×2160）/30Hz，2K（2560×1440）/60Hz，1080P（1920×1080）/60Hz，720P（1280×720）/60Hz Stream Type Video, Video & Audio | buc | 1 |
|  | Switch (comutator de rețea) Layer 3 managed switch Cantitatea porturilor minimala – 12 Minim 2 porturi Gigabit Ethernet sau 2 porturi SPF cu module RJ45 Gigabit Ethernet instalate Minim 10 porturi Gigabit SFP și/sau 10Gigabit SFP Suportă implementarea în rețele IPv6 și IPv4 Inspecție ARP, IP Source Guard, protecție DoS, securitate porturi și DHCP Snooping Controler încorporat pentru gestionarea comutatorului; QoS încorporat permite prioritizarea traficului de rețea. | buc | 1 |
|  | Unitate de stocare Unitate HDD Western Digital WD 16 TB Tip unitate de stocare Unitate HDD Sistem de stocare a datelor 3.5" Capacitate memorie 16 TB Dimensiune memorie tampon (cache) 512 MB Interfață SSD SATA-3 (6 Gb/s) | Buc | 4 |
|  | Calculator Monitorizare Video (set) Monitor: diagonala 32 inch, rezolutia minima 2K, HDMI, DisplayPort PC, cerinte minimale: CPU 8 nuclee 3.7 GHz fiecare, placa de baza cu interfate HDMI si DisplayPort, RAM 16 GB, SSD 128 GB, HDD 1000 GB, PSU – 550W Tastatura USB, Mouse USB UPS 800VA/480W, Line Interactive, AVR, LED, 2xShuko Sockets | set | 1 |
|  | Televizor pentru post de monitorizare 55" LED SMART TV TCL | Buc | 1 |
|  | Surse de Alimentare |  |  |
|  | Sursa de Alimentare de Rezerva Tip UPS: linie interactivă, undă sinusoidală pură Intrare: 220V AC ± 25%, 50Hz / 60Hz ± 10% Ieșire: 220V AC ± 10%, 50Hz / 60Hz ± 0,5% (pe baterie) Prize: intrare IEC C14, ieșiri: Schuko tip EU x 1 buc, IEC C13 x 6 buc Condiții de funcționare: 0°C~40°C, la umiditate 10~90%RH fără condensare Zgomot: < 45 dB (la 1 metru distanta) Putere: 3000 VA (2400 W) Dimensiuni: 440 x 410 x 132 mm | buc | 1 |
|  | Dulapuri si Boxe |  |  |
|  | Dulap Metalic 800x600x250 | buc | 2 |
|  | Dulap de Telecomunicatii Dulap de perete 9U pentru echipamente de rețea cu ușă frontală blocabilă din sticlă călită, panouri laterale detașabile și deschideri de ventilație pentru răcirea echipamentelor. Capacitate maximă de încărcare de 60 kg. Ușă frontală din sticlă blocabilă. Nivel de protecție IP20. Lățime - 600 mm, adâncime - 450 mm, înălțime - 501 mm. Material - oțel laminat la rece. | buc | 1 |
|  | Elemente de Fixare compatibile cu sistemul | set | 10 |
|  | Soft de Monitorizare Video+Paza+ Module | set | 1 |
|  | Servicii de trasare cabluri, instalare si configurare echipamente. | Serv | 1 |

Sistemul Limitare a accesului integrat cu Sistem Video

| Nr.crt. | Denumirea Utilajului si a Materialelor | Unitatea de masura |  | Volumul |  |
| --- | --- | --- | --- | --- | --- |
|  | Utilaj Central |  |  |  |  |
|  | Centrala de acces Biometrica la intrarile pe teritoriu Multifactor Biometric Access Control and Time Attendance Terminal with Face Recognition Advanced biometric access control and time attendance terminal based on deep learning facial recognition technology. Designed for secure identification, workforce management and integration with access control, video surveillance and IP intercom systems. Authentication Methods Facial recognition MIFARE card authentication QR code authentication Multi-factor authentication support Facial Recognition Technology Deep learning-based face recognition engine Dual-lens wide-angle camera system 2 MP image resolution Advanced anti-spoofing technology preventing access using photographs or other non-live facial images Face recognition in low-light and complete darkness with adjustable LED illumination Recognition distance: 0.3 m to 3 m Recognition time: less than 0.2 seconds Health and Safety Functions Face mask detection Configurable mask compliance policy with optional access denial or access permission Real-time notifications for non-compliance Display 7-inch HD LCD capacitive touchscreen Resolution: 1024 × 600 pixels Customizable promotional and informational display screens Audio and Communication Built-in microphone and speaker Two-way audio communication Integration with IP intercom systems Subscriber calling directly from the terminal Database Capacity Up to 6,000 facial templates Up to 6,000 MIFARE cards Event storage capacity up to 50,000 records Network and Interfaces 10/100/1000 Mbps Ethernet interface RS-485 communication port Wiegand interface 2 × USB ports 2 alarm inputs 1 alarm output 2-pin power interface Access Control Features Door lock control Exit button connection Door status sensor connection Remote door opening Real-time monitoring and management Event logging and audit trail Standalone operation capability without external controller Video Integration Recording to Network Video Recorder (NVR) Real-time video viewing Integration with video surveillance platforms Operating System Embedded Linux operating system Power Supply 12 VDC, 2 A Environmental IP65-rated dust and water protection Operating temperature: -30°C to +60°C Physical Characteristics Dimensions: 239 × 117 × 33 mm Applications Office buildings Industrial facilities Educational institutions Hotels and hospitality facilities Critical infrastructure Government and commercial facilities Time attendance and workforce management systems Warranty 3-year manufacturer warranty | buc. |  | 3 |
|  | Monitor pentru Deschidere de la distanta cu parametrii minim 10.1" IP Indoor Touchscreen Control Monitor for Video Intercom and Security Management Advanced indoor IP control monitor featuring a 10.1-inch color touchscreen display and Android-based operating system. Designed for integrated management of video intercom, video surveillance, intrusion alarm and access control systems through a single user interface. Display 10.1-inch color IPS capacitive touchscreen Resolution: 1024 × 600 pixels User-friendly graphical interface LED status illumination integrated into the lower edge of the monitor Operating System and Applications Android-based operating system Pre-installed security management application Centralized access to connected security devices Support for installation of third-party Android applications Integrated management of video surveillance, intercom and alarm systems Video Functions Live video viewing from IP cameras Playback of recorded video archives Reception of video alarm verification clips from intrusion alarm systems Real-time monitoring of connected security devices Video Intercom Functions Video call reception from outdoor stations Two-way audio communication Remote door release functionality Mobile application integration for receiving and answering calls remotely Door unlocking from mobile devices and monitor interface Dedicated physical door release button Audio Built-in microphone and speaker Full duplex two-way communication Noise suppression technology Echo cancellation technology Memory Internal storage: 8 GB RAM: 1 GB Interfaces 10/100 Mbps Ethernet interface Wi-Fi 802.11 b/g/n RS-485 communication interface 8 alarm inputs 2 alarm outputs Tamper protection input Power Supply 12 VDC power input Maximum power consumption: 8 W IEEE 802.3af PoE support Environmental Operating temperature range: -10°C to +50°C Physical Characteristics Dimensions: 254 × 166 × 27 mm Applications Residential buildings Office and commercial facilities Hotels and hospitality environments Integrated security management systems Access control and video intercom installations Building Management Systems (BMS) Package Contents Indoor monitor unit Power supply not included Warranty 3-year manufacturer warranty | buc |  | 2 |
|  | Detectoare |  |  |  |  |
|  | Detector Magnetic Pentru usi Metalice | set |  | 3 |
|  | Surse de alimentare de rezerva și cutii |  |  |  |  |
|  | Sursa de Alimentare 12V/5A cu acumulator | Set | 3 |  |  |
|  | Baterii |  |  |  |  |
|  | Cabluri |  |  |  |  |
|  | Structuri de Cablare, Materiale de fixare si lucrari Considerate in Sistemul De Supraveghere Video |  |  |  |  |

Cerinţe privind caracteristicile tehnice

Pentru fiecare echipament în parte se vor livra accesoriile şi cablurile de conexiune necesare

funcţionării şi interconectării acestora, indiferent dacă acestea au fost sau nu expres solicitate.

Sistemul va asigura stocarea informațiilor conform normativelor în vigoare, dar nu mai puțin de

30 zile.

Accesul la camerele de supraveghere din aplicația mobilă vor deține 3 persoane responsabile.

Este strict necesar de a instala sistemul de supraveghere video cu posibilitatea evitării conflictului  cu rețeaua locală. Sistemul video va fi configurat astfel ca sa permita securizarea perimetrului si teritoriului si va oferi posibilitatea transmiterii acceslui catre agentie de paza care va prelua sistemul ca un sistem complet de paza video.

Sistemul va asigura supravegherea video pe timp de zi şi de noapte a intrărilor/ ieşirilor din/în

clădirea instituţiei astfel încât să se realizeze detecţia şi identificarea oricărei încercări de

pătrundere neautorizată în instituţie, precum şi monitorizarea acceselor autorizate în încăperile

instituţiei. Sistemul dotat cu centrala biometrica va permite blocarea si securizarea intrarilor pe teritoriul institutiei si accesul pe teritoriul doar a persoanelor autorizate pe baza de date biometrice sau coduri tip QR.