# Unofficial translation

# SPECIFICATIONS / TECHNICAL SPECIFICATIONS

Object: Development of an integrated urban transport system in the municipality of Ungheni (Buses)

Contracting authority: Centre Regional Development Agency

| Item no. | CPV Code | Name of the requested goods | Unit of measurement | Quantity | The required full technical specification, standard of reference |
| --- | --- | --- | --- | --- | --- |
| Lot no. 1 Buses for urban public transport General conditions: All transport units will be new, with a warranty period of at least 2 years from the date of receipt of the units. The supplier is obliged to ensure the training of the beneficiary's staff. The supplier is obliged to provide spare parts and aggregates for the repair of transport units, for a fee, for at least 10 years from the expiry of the warranty. The transport units must be properly equipped, so that the service operator is subsequently able to ensure the safety of passengers. The transport units must comply with national and EU requirements (as applicable) in order for the service operator to be able to obtain all the necessary authorisations for the operation of the public passenger transport service. The buses will be intended for operation in temperate-continental climate areas and will ensure reliable operation in the following ambient conditions: • Ambient temperature - 30 °C ... + 45 °C; • Maximum relative humidity 100 % RH at + 25 °C; • The atmospheric pressure between 866 ... 1066 kPa; • Altitude from sea level (0 m) to a maximum of 400 m; • Outdoor agents: dust, rain, fog, mud, snow, frost, ice, salt water, petroleum products, anti-slip materials and solutions. |  |  |  |  |  |
| 1. | 34121000-1 | Small capacity buses for urban public transport | Pcs | 15 | Body type – self-supporting / non-load-bearing, city bus Year of manufacture – from 2021 onwards Dimensions Total length: between 5000 – 7200 mm; Width: max. 2700 mm (without exterior mirrors); Height: max. 3100 mm |
|  |  |  |  |  | Interior dimensions Interior height: minimum 1850 mm; Width inside: minimum 1850 mm Height of the access platform in the ward: maximum 390 mm Minimum width of the space Open door access: (two-leaf door) minimum 1000 mm. Minimum width of the space Open door access: (single-leaf door) minimum 550 mm. Maximum permissible total weight: maximum 8 500 kg Ecological standard – minimum EURO 5 |

|  |  |  |  |  | Body type: Body with anti-corrosion processing Floor level: Floor lowered at entrance, min 60% floor to be lowered for standing passengers Servo or hydraulic steering system Engine type: diesel, four-stroke, turbocharged Motor power: min 115 kW, max 140 kW Gearbox: Automatic/ mechanical Cubic capacity: min 2000 cm³, max 3800 cm³ Safety systems: minimum ABS; EBD; ESP Minimum power 150 Hp Central locking Speed limiter Double passenger access door with driver-controlled automatic opening Electric rear-view mirrors with blind spot elimination Fuel tank: min. 80 liters Ramp for people with disability, preferably electrically operated Adjustable driver's seat Ventilation and heating system: Air conditioning system with minimum power of 11 KW. Air System Warranty Conditionally must not be less than the general warranty for the bus. The side windows of the saloon must ensure its natural ventilation through sliding windows at their top. Depending on the type of size (range) of the bus, it will have at least one ventilation hatch, all with electric opening. Number of seats: minimum 20 people, of which minimum 10 people per seat Stop button at the request of passengers (optional) Durable upholstery and upholstery , easy to maintain and clean Minimum 1 exterior indicator with electronic display for passenger information and route indication, placed at the front and optionally on the side and rear Information system: Electronic information system (front and optional side, rear, interior). LED Technology Passenger counting system (optional) Indoor and outdoor video surveillance system E-ticketing system (optional) All inscriptions on the inside and outside of the buses will be made in Romanian. The exterior painting and all the inscriptions according to the legislation in force (tire pressure, safety exits, places for people with reduced mobility, wheelchairs, etc.) will be carried out by the winning bidder. The specific design elements regarding the exterior painting of the body will be established in agreement with the beneficiary and will include the identity elements, symbols and/or logo of the City Hall and the inscription "Ungheni City Hall, financed from the National Fund for Regional and Local Development, through the Center RDA". |
| --- | --- | --- | --- | --- | --- |

| 1. | 34121000- | Medium-capacity buses for urban public transport | Pcs | 3 | Body type – self-supporting / non-load-bearing, city bus |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Year of manufacture – from 2021 onwards |
|  |  |  |  |  | Dimensions |
|  |  |  |  |  | Total length: between |
|  |  |  |  |  | 8000 – 12,500 mm; |
|  |  |  |  |  | Width: max. 2550 mm (without |
|  |  |  |  |  | exterior mirrors); |
|  |  |  |  |  | Height: max. 3300 mm |
|  |  |  |  |  | Interior dimensions Height inside: minimum 2100 mm; Width inside: minimum 1900 |
|  |  |  |  |  | Mm; |
|  |  |  |  |  | Height of the access platform in |
|  |  |  |  |  | living room: maximum 370 mm; |
|  |  |  |  |  | Minimum width of the space |
|  |  |  |  |  | Open door access: (two-leaf door) minimum 1100 mm. |
|  |  |  |  |  | Minimum width of the space Open door access: (single-leaf door) minimum 550 mm. |
|  |  |  |  |  | Maximum permissible total weight: |
|  |  |  |  |  | between 11000 - 19000 kg |
|  |  |  |  |  | Ecological standard - minimum |
|  |  |  |  |  | EURO 5 |

Page 1 of 3

|  |  |  |  |  | Body type: Body with anti-corrosion processing Floor level: Floor lowered at the entrance, min 60% of the floor to be lowered for standing passengers Servo or hydraulic steering system Engine type: diesel, four-stroke turbocharger Engine power: min 135 kW Gearbox: Automatic / mechanical Cubic capacity: maximum 7700 cm³ Safety systems: minimum ABS; EBD; ESP Power: minimum 180 hp Central locking Speed limiter (optional) Double passenger access door with driver-controlled automatic opening Electric rear-view mirrors with blind spot elimination Fuel tank with min. 140 liters Ramp for people with disabilities, preferably electrically operated Adjustable driver's seat Ventilation and heating system: Air conditioning system with minimum power of 20 KW. Air System Warranty Conditionally must not be less than the general warranty for the bus. The side or upper windows of the saloon must ensure its natural ventilation through sliding windows at their top. Depending on the typesize (range) of the bus, it will have at least one ventilation hatch, all with electric opening. Number of seats , minimum transport capacity: 50 people, of which at least 18 people on seats Stop button at the request of passengers Durable upholstery and upholstery , easy to maintain and clean Minimum 1 external indicator with electronic display for informing travellers, and route indication, placed in front and optionally on the right side Information system: Electronic information system (front, and optional side, rear, interior) LED Technology Passenger counting system (optional) Indoor and outdoor video surveillance system E-ticketing system (optional) The dashboard will be equipped with an on-board computer with a multifunctional digital display The front indicator (and optionally the side and rear indicator) must show the line number, the starting point and the final destination, optionally displaying the intermediate route. All inscriptions on the inside and outside of the buses will be made in Romanian. The exterior painting and all the inscriptions according to the legislation in force (tire pressure, safety exits, places for people with reduced mobility, wheelchairs, etc.) will be carried out by the winning bidder. The specific design elements regarding the exterior painting of the body will be established in agreement with the beneficiary and will include the identity elements, symbols and/or logo of the City Hall and the inscription "Ungheni City Hall, financed from the National Fund for Regional and Local Development, through the Center RDA". |
| --- | --- | --- | --- | --- | --- |

*Note: Payments will be made by transfer based on e-invoices, within 45 days after receipt of the handover-receipt documents, and invoices, signed and accepted by the Co-investor. It is forbidden to make advance payments (advance) by the contracting authority.